3GPP TSG-RAN WG4 Meeting #104-bis-e                                                                                   R4-2215415

Electronic Meeting, October 10 – October 19, 2022

Source: 	CATT

Title: 	General consideration on SAN RF requirements for above 10GHz bands

Agenda item:	6.22.3

Document for:	Approval

- Introduction

In this meeting, RAN4 work on NR NTN (Non-Terrestrial Networks) enhancements starts. This contribution provides our general consideration on SAN RF requirements for above 10GHz bands.

- Discussion

Frequency range and SAN type

Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

In WID[1], satellite harmonized Ka band was considered as reference, the typical frequency in Ka band is 20 GHz for DL and 30GHz for UL[2]. The frequency ranges in TN is defined in Table 5.1-1 of TS 38.014 copied as above. We can observe that the 20 GHz for DL in Ka band is outside all frequency ranges, and 30GHz for UL in Ka band is inside FR2-1 frequency range. To keep DL (20GHz) and UL frequency (30GHz) of Ka band in same frequency range, Extending FR2-1 downwards is needed. So FR2-1 (24250MHz-52600MHz) from TN is not applicable for Ka band of NTN.

Observation 1: FR2-1 (24250MHz-52600MHz) from TN is not applicable for Ka band of NTN, extending FR2-1 downwards is needed.

Currently, only BS type 2-O is defined for FR2-1 in TN. Since 20 GHz for DL in Ka band of NTN is close to 24250MHz of FR2-1. From perspective of radio frequency characteristics, SAN type should be similar to BS type. So to define SAN 2-O for Ka band NTN is reasonable.

Observation 2: To define SAN type 2-O for Ka band NTN.

Radiated requirement for Ka band NTN

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |
| --- | --- |
|  | SAN type 1-O |
| OTA transmit ON/OFF power | N/A |
| OTA time alignment error | N/A |
| OTA transmitter intermodulation | N/A |
| OTA in-band blocking | N/A |
| OTA receiver spurious emission | N/A |
| OTA receiver intermodulation | N/A |

Referring to requirement set applicability for SAN type 1-O in Table 4.6-1 of TS 38.108 copied as above, the OTA transmit ON/OFF power, OTA time alignment error, OTA transmitter intermodulation, OTA in-band blocking, OTA receiver spurious emission, and OTA receiver intermodulation are not applicable for SAN type 1-O because the following reason:

-	FDD, no TDD

-	Single layer (stream) MIMO transmission

-	No surrounding interfering SAN next to SAN

-	No In-band blocker

When it comes to Ka band of NTN, the above same reasons also exist. So the OTA transmit ON/OFF power, OTA time alignment error, OTA transmitter intermodulation, OTA in-band blocking, OTA receiver spurious emission, and OTA receiver intermodulation are not applicable for SAN type 2-O in Ka band of NTN.

Observation 3: The OTA transmit ON/OFF power, OTA time alignment error, OTA transmitter intermodulation, OTA in-band blocking, OTA receiver spurious emission, and OTA receiver intermodulation are not applicable for SAN type 2-O in Ka band of NTN.

The initial applicability considerations for requirement for SAN type 2-O in Ka band of NTN except above not applicable requirement are shown in Table 2-1.

Table 2-1: Summaries of initial considerations for Radiated requirement for SAN type 2-O in Ka band of NTN

| Requirement | Initial applicability consideration |
| --- | --- |
| Radiated transmit power | EIRP accuracy for SAN type 2-O is different from that for SAN type 1-O. the ±3.4dB EIRP accuracy for BS type 2-O from TN can be as reference. |
| OTA Satellite Access Network output power | TRP accuracy for SAN type 2-O is different from that for SAN type 1-O. the ±3dB TRP accuracy for BS type 2-O from TN can be as reference. |
| OTA output power dynamics | This depends on SCS and CBW supported in Ka band of NTN, the requirement for BS type 2-O in FR2-1 from TN can be as a reference. |
| OTA frequency error | To reuse the same requirement for SAN type 1-O defined in TS 38.108. |
| OTA modulation quality | To reuse the same requirement for SAN type 1-O defined in TS 38.108, 64QAM is optional. Define EVM window length for supported SCS for SAN type 2-O. |
| OTA occupied bandwidth | To reuse the same requirement for SAN type 1-O defined in TS38.108 which is following ITU-R Recommendation SM.328. |
| OTA ACLR | This depends on the outcome of coexistence study. |
| OTA out-of-band emission | Define ΔfOBUE for SAN type 2-O, and decide whether OBUE requirement for SAN type 1-O can be reused for SAN type 2-O. |
| OTA transmitter spurious emission | Spurious frequency range for SAN type 2-O can be defined as 30 MHz –2nd harmonic of the upper frequency edge of the DL operating band. The basic limit for SAN type 1-O in TS 38.108 can be as reference. |
| OTA sensitivity | This requirement is not applicable for SAN type 2-O, because the OTA reference sensitivity level considering beamforming gain is sufficient. |
| OTA reference sensitivity level | To reuse same approach for BS type 2-O from TN. EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz SAN channel bandwidth. Decide noise figure for SAN type 2-O. |
| OTA dynamic range | Since OTA dynamic range is not applicable for BS type 2-O in TN. So we think this requirement is not applicable for SAN type 2-O. |
| OTA ACS | This depends on the outcome of coexistence study. |
| OTA out-of-band blocking | Define ΔfOOB for SAN type 2-O, and decide whether 0.0029 V/m Interfering signal RMS field-strength for SAN type 1-O applies to SAN type 2-O. |
| OTA in-channel selectivity | To reuse same approach from TN for BS type 2-O. |

- Conclusion

This contribution provides our general consideration on SAN RF requirement for above 10GHz bands. The following observations are concluded as follows:

Observation 1: FR2-1 (24250MHz-52600MHz) from TN is not applicable for Ka band of NTN, extending FR2-1 downwards is needed.

Observation 2: To define SAN type 2-O for Ka band NTN.

Observation 3: The OTA transmit ON/OFF power, OTA time alignment error, OTA transmitter intermodulation, OTA in-band blocking, OTA receiver spurious emission, and OTA receiver intermodulation are not applicable for SAN type 2-O in Ka band of NTN.

- References

- RP-221819, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales, RAN#96

- 3GPP TR 38.821