3GPP TSG RAN WG4 Meeting #104-bis-e                   R4- 2215417

Electronic Meeting, October 10th - October 19th, 2022

Agenda item:			4.3.6.1

Source:	CATT

Title:	Further discussion on general RRM performance requirements for NR extension to 71 GHz

Document for:	Discussion

- Introduction

In the last RAN4 meeting, WF [1] on NR extension to 71 GHz RRM performance requirements was approved. In this contribution, the remaining issue on general RRM performance requirements for NR extension to 71 GHz is further discussed.

- Discussion

## CCA aspects

In the RAN4#104-e meeting, for CCA model in test cases, the issue of modelling an unavailable SSB/SMTC group for NR extension to 71 GHz has a conclusion, but the exact shifting pattern has not yet reached a consensus, the agreement made during the last meeting is duplicated as below [1]:

Issue 1-5-3: Modelling unavailable SSB/SMTC Agreement: For CCA model in test cases, an unavailable SSB/SMTC group can be modelled as that there is exactly one SSB not transmitted by TE in N consecutive SSB/SMTC occasions - Shift SSB index in each N consecutive SSB/SMTC occasions rather than keeping one fixed SSB index - FFS: Exact shifting pattern

According to the agreement, we have determined that an unavailable SSB/SMTC group modelled as that there is exactly one SSB not transmitted by TE in N confidential SSB/SMTC occasions, and that shift SSB index in each N confidential SSB/SMTC occasions is more reasonable than keep one fixed SSB index. However, how to shift the SSB index is still uncertain.

In order to test the behavior of UE more thoroughly, we prefer to use the following fixed mode to shift the SSB index, as discussed in the last meeting, for example: the unavailable SSB in the first 12 SSBs could be the first SSB, the unavailable SSB in the second 12 SSBs could be the second SSB, and so on [2]. Compared with the real random mode, this fixed sequential mode can avoid frequently repeating SSB indexes more accurately and directly.

Proposal 1: In order to test the behavior of UE more thoroughly, it is suggested to use a fixed sequential mode to shift the SSB index.

- For example, the unavailable SSB in the first 12 SSBs could be the first SSB, the unavailable SSB in the second 12 SSBs could be the second SSB, and so on.

- Summary

In this paper, we provide our views on CCA model in test cases for the extension to 71 GHz. From this discussion we have derived the following proposal:

Proposal 1: In order to test the behavior of UE more thoroughly, it is suggested to use a fixed sequential mode to shift the SSB index.

- For example, the unavailable SSB in the first 12 SSBs could be the first SSB, the unavailable SSB in the second 12 SSBs could be the second SSB, and so on.

- References

- R4-2214478 WF on NR extension to 71 GHz RRM performance requirements, Qualcomm

- R4-2214271, Email discussion summary for [104-e][219] NR_ext_to_71GHz_RRM_2, Qualcomm, RAN4 #104-e, Aug 15 – 26, 2022