3GPP TSG-RAN4 Meeting #104-bis-e	R4-2215422

# Electronic meeting, October 10th - October 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for RedCap UEs for intra-frequency measurement in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Perf |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test cases need to be defined to verify the measurement procedure for NR Redcap UEs in FR1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | According to the work split, SA event triggered reporting tests with per-UE gaps under non-DRX and SA event triggered reporting tests without gap under non-DRX with SSB index reading for intra-frequency measurement in FR1 are to be introduced. A.16.6.1 Intra-frequency Measurements A.16.6.1.5 SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx RedCap UE A.16.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx RedCap UE A.16.6.1.9 SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx RedCap UE A.16.6.1.10 SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx RedCap UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement requirements of NR Redcap UEs in FR1 cannot be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.16.6.1.5, A.16.6.1.6, A.16.6.1.9, A.16.6.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE

A.16.6.1.5.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

A.16.6.1.5.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.16.6.1.5.2-1 and A.16.6.1.5.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.16.6.1.5.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| 5 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 6 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 7 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8 | 15 kHz SSB SCS, 20 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.16.6.1.5.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3, 4 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3, 4, 5, 6, 7, 8 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3, 4, 5, 6, 7, 8 | 40 |  |
| Measurement gap length | ms | 1, 2, 3, 4, 5, 6, 7, 8 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3, 4, 5, 6, 7, 8 | 39 |  |
| CD-SSB configuration |  | 1, 2, 4 | SSB.4 RedCap FR1 |  |
|  |  | 3 | SSB.5 RedCap FR1 |  |
| NCD-SSB configuration |  | 5, 6, 8 | SSB.6 RedCap FR1 Note 4 |  |
|  |  | 7 | SSB.7 RedCap FR1 Note 4 |  |
| SMTC configuration for CD-SSB |  | 1, 4 | SMTC.1 RedCap |  |
|  |  | 2 | SMTC.1 RedCap |  |
|  |  | 3 | SMTC.1 RedCap |  |
| SMTC configuration for NCD-SSB |  | 5,6,7,8 | SMTC.2 RedCap |  |
| CSI-RS parameters |  | 1, 4, 5, 8 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2, 6 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3, 7 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3, 4, 5, 6, 7, 8 | -4.5 |  |
| CP length |  | 1, 2, 3, 4, 5, 6, 7, 8 | Normal |  |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |
| Time To Trigger | s | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6, 7, 8 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3, 4, 5, 6, 7, 8 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 4, 5, 8 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 6 | 3 s | Synchronous cells |
|  |  | 3, 7 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3, 4, 5, 6, 7, 8 | 5 |  |
| T2 | s | 1, 2, 3, 4, 5, 6, 7, 8 | 5 |  |

Table A.16.6.1.5.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 4, 5, 8 | TN/A |  | TN/A |  |
|  |  | 2, 6 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3, 7 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4, 5, 8 | SR.1.1 FDD |  | N/A |  |
|  |  | 2, 6 | SR.1.1 TDD |  |  |  |
|  |  | 3, 7 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1, 4, 5, 8 | CR.1.1 FDD |  | N/A |  |
|  |  | 2, 6 | CR.1.1 TDD |  | N/A |  |
|  |  | 3, 7 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 4, 5, 8 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2, 6 | CCR.1.2 TDD |  | N/A |  |
|  |  | 3, 7 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6, 7, 8 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 4, 5, 8 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2, 6 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3, 7 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.1.1 RedCap |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.1.1 RedCap |  | ULBWP.1.1 |  |
| Dedicated DL BWP configuration |  | 5, 6, 7, 8 | DLBWP.1.3 RedCap Note 5 |  | DLBWP.1.3 RedCap Note 5 |  |
| Dedicated UL BWP configuration |  | 5, 6, 7, 8 | ULBWP.1.3 RedCap Note 6 |  | ULBWP.1.3 RedCap Note 6 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6, 7, 8 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1, 4, 5, 8 | -98 |  |  |  |
|  |  | 2, 6 | -98 |  |  |  |
|  |  | 3, 7 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1, 4, 5, 8 | -98 |  |  |  |
|  |  | 2, 6 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1, 4, 5, 8 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2, 6 |  |  |  |  |
|  |  | 3, 7 |  |  |  |  |
|  | dB | 1, 4, 5, 8 | 4 | 4 | -Infinity | 4 |
|  |  | 2, 6 |  |  |  |  |
|  |  | 3, 7 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 4 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
|  |  | 5, 6, 7, 8 | TDB | TDB | TDB | TDB |
| Io | dBm/9.36 MHz | 1, 4, 5, 8 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2, 6 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3, 7 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6, 7, 8 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: NCD-SSB is configured within dedicated RedCap DL BWP. Note 5: The starting PRB index for dedicated DL BWP1 corresponding to CD-SSB PRB index; the starting PRB index for dediacted DL BWP2 corresponding to NCD-SSB PRB index; Note 6: The starting PRB index for dedicated UL BWP1 is the same as the starting PRB index for dedicated DL BWP1; the starting PRB index for dedicated UL BWP2 is the same as the starting PRB index for dedicated DL BWP2. |  |  |  |  |  |  |

A.16.6.1.5.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800/1000 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE

A.16.6.1.6.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

A.16.6.1.6.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.16.6.1.6.2-1 and A.16.6.1.6.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.16.6.1.6.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| 5 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 6 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 7 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8 | 15 kHz SSB SCS, 20 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.16.6.1.6.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6, 7, 8 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3, 4, 5, 6, 7, 8 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3, 4, 5, 6, 7, 8 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3, 4, 5, 6, 7, 8 | 40 |  |
| Measurement gap length | ms | 1, 2, 3, 4, 5, 6, 7, 8 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3, 4, 5, 6, 7, 8 | 39 |  |
| CD-SSB configuration |  | 1, 2, 4 | SSB.4 RedCap FR1 |  |
|  |  | 3 | SSB.5 RedCap FR1 |  |
| NCD-SSB configuration |  | 5, 6, 8 | SSB.6 RedCap FR1 Note 4 |  |
|  |  | 7 | SSB.7 RedCap FR1 Note 4 |  |
| SMTC configuration for CD-SSB |  | 1, 4 | SMTC.1 RedCap |  |
|  |  | 2 | SMTC.1 RedCap |  |
|  |  | 3 | SMTC.1 RedCap |  |
| SMTC configuration for NCD-SSB |  | 5,6,7,8 | SMTC.2 RedCap |  |
| CSI-RS parameters |  | 1, 4, 5, 8 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2, 6 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3, 7 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3, 4, 5, 6, 7, 8 | -4.5 |  |
| CP length |  | 1, 2, 3, 4, 5, 6, 7, 8 | Normal |  |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |
| Time To Trigger | s | 1, 2, 3, 4, 5, 6, 7, 8 | 0 |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6, 7, 8 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3, 4, 5, 6, 7, 8 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 4, 5, 8 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 6 | 3 s | Synchronous cells |
|  |  | 3, 7 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3, 4, 5, 6, 7, 8 | 5 |  |
| T2 | s | 1, 2, 3, 4, 5, 6, 7, 8 | 5 |  |

Table A.16.6.1.6.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 4, 5, 8 | TN/A |  | TN/A |  |
|  |  | 2, 6 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3, 7 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4, 5, 8 | SR.1.1 FDD |  | N/A |  |
|  |  | 2, 6 | SR.1.1 TDD |  |  |  |
|  |  | 3, 7 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1, 4, 5, 8 | CR.1.1 FDD |  | N/A |  |
|  |  | 2, 6 | CR.1.1 TDD |  | N/A |  |
|  |  | 3, 7 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 4, 5, 8 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2, 6 | CCR.1.2 TDD |  | N/A |  |
|  |  | 3, 7 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6, 7, 8 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 4, 5, 8 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2, 6 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.1.1 RedCap |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3, 4, 5, 6, 7, 8 | DLBWP.1.1 RedCap |  | ULBWP.1.1 |  |
| Dedicated DL BWP configuration |  | 5, 6, 7, 8 | DLBWP.1.3 RedCap Note 5 |  | DLBWP.1.3 RedCap Note 5 |  |
| Dedicated UL BWP configuration |  | 5, 6, 7, 8 | ULBWP.1.3 RedCap Note 6 |  | ULBWP.1.3 RedCap Note 6 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6, 7, 8 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1, 4, 5, 8 | -98 |  |  |  |
|  |  | 2, 6 | -98 |  |  |  |
|  |  | 3, 7 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1, 4, 5, 8 | -98 |  |  |  |
|  |  | 2, 6 |  |  |  |  |
|  |  | 3, 7 |  |  |  |  |
|  | dB | 1, 4, 5, 8 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2, 6 |  |  |  |  |
|  |  | 3, 7 |  |  |  |  |
|  | dB | 1, 4, 5, 8 | 4 | 4 | -Infinity | 4 |
|  |  | 2, 6 |  |  |  |  |
|  |  | 3, 7 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 4 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
|  |  | 5, 6, 7, 8 | TDB | TDB | TDB | TDB |
| Io | dBm/9.36 MHz | 1, 4, 5, 8 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2, 6 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3, 7 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6, 7, 8 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: NCD-SSB is configured within dedicated RedCap DL BWP. Note 5: The starting PRB index for dedicated DL BWP1 corresponding to CD-SSB PRB index; the starting PRB index for dediacted DL BWP2 corresponding to NCD-SSB PRB index; Note 6: The starting PRB index for dedicated UL BWP1 is the same as the starting PRB index for dedicated DL BWP1; the starting PRB index for dedicated UL BWP2 is the same as the starting PRB index for dedicated DL BWP2. |  |  |  |  |  |  |

A.16.6.1.6.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800/1000 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 1>

<Start of Change 2>

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE

A.16.6.1.9.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

A.16.6.1.9.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in Table A.16.6.1.9.2-1 and A.16.6.1.9.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.16.6.1.9.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |

Table A.16.6.1.9.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |
| CD-SSB configuration |  | 1 | SSB.4 RedCap FR1 |  |
| NCD-SSB configuration |  | 2 | SSB.6 RedCap FR1 Note 1 |  |
| SMTC configuration for CD-SSB |  | 1 | SMTC.1 RedCap |  |
| SMTC configuration for NCD-SSB |  | 2 | SMTC.2 RedCap |  |
| A3-Offset | dB | 1, 2 | -4.5 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 2 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |
| Note 1: NCD-SSB is configured within dedicated RedCap DL BWP. |  |  |  |  |

Table A.16.6.1.9.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0,1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| Dedicated DL BWP configuration |  | 2 | DLBWP.1.3 RedCap Note 4 |  | DLBWP.1.3 RedCap Note 4 |  |
| Dedicated UL BWP configuration |  | 2 | ULBWP.1.3 RedCap Note 5 |  | ULBWP.1.3 RedCap Note 5 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98 |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  | dB | 1, 2 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1, 2 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | TBD | TBD | TBD | TBD |
| Io | dBm/9.36 MHz | 1, 2 | -64.60 | -62.25 | --64.60 | -62.25 |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The starting PRB index for dedicated DL BWP1 corresponding to CD-SSB PRB index; the starting PRB index for dediacted DL BWP2 corresponding to NCD-SSB PRB index; Note 5: The starting PRB index for dedicated UL BWP1 is the same as the starting PRB index for dedicated DL BWP1; the starting PRB index for dedicated UL BWP2 is the same as the starting PRB index for dedicated DL BWP2. |  |  |  |  |  |  |

A.16.6.1.9.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1040/1480 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE

A.16.6.1.10.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

A.16.6.1.10.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in Table A.16.6.1.10.2-1 and A.16.6.1.10.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.16.6.1.10.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |

Table A.16.6.1.10.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  |
| CD-SSB configuration |  | 1 | SSB.4 RedCap FR1 |  |
| NCD-SSB configuration |  | 2 | SSB.6 RedCap FR1 Note 1 |  |
| SMTC configuration for CD-SSB |  | 1 | SMTC.1 RedCap |  |
| SMTC configuration for NCD-SSB |  | 2 | SMTC.2 RedCap |  |
| A3-Offset | dB | 1, 2 | -4.5 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 2 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |
| Note 1: NCD-SSB is configured within dedicated RedCap DL BWP. |  |  |  |  |

Table A.16.6.1.10.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0,1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| Dedicated DL BWP configuration |  | 2 | DLBWP.1.3 RedCap Note 4 |  | DLBWP.1.3 RedCap Note 4 |  |
| Dedicated UL BWP configuration |  | 2 | ULBWP.1.3 RedCap Note 5 |  | ULBWP.1.3 RedCap Note 5 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 2 | -98 |  |  |  |
| Note 2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |
|  | dB | 1, 2 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1, 2 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | TBD | TBD | TBD | TBD |
| Io | dBm/9.36 MHz | 1, 2 | -64.60 | -62.25 | --64.60 | -62.25 |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The starting PRB index for dedicated DL BWP1 corresponding to CD-SSB PRB index; the starting PRB index for dediacted DL BWP2 corresponding to NCD-SSB PRB index; Note 5: The starting PRB index for dedicated UL BWP1 is the same as the starting PRB index for dedicated DL BWP1; the starting PRB index for dedicated UL BWP2 is the same as the starting PRB index for dedicated DL BWP2. |  |  |  |  |  |  |

A.16.6.1.10.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920/1240 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 2>