3GPP TSG-RAN4 Meeting #104bis-e	                                                            R4-2215428

Electronic Meeting, October 10 – October 19, 2022

Title:	Discussion on RRM requirements for Inter-RAT measurement without gap

Source:	CATT

Agenda Item:	6.10.3.2

Document for:	Discussion

1 Introduction

In the previous RAN4 meeting, RRM requirements for Inter-RAT measurement without gap were discussed and some WF were agreed in [1]. This contribution further discusses the topic and presents our views.

2 Discussion

First of all, the issue 3-6 (for MR-DC cases) and 3-7 (mixed-numerology cases) as listed in [1], we think the MR-DC can be deprioritized but we are open to further discuss the mix-numerology cases which are meaningful to consider. In the following, we discuss Inter-RAT NR measurements in section 2.1, and Inter-RAT LTE measurements in section 2.2.

2.1 Inter-RAT NR measurements

In issue 3-8 in [1], it is FFS whether to extend the capability of NeedForNCSG to inter-RAT NR measurement.

FFS whether to extend the capability of NeedForNCSG to inter-RAT NR measurement.

In our view there is no need for this and it seems no NeedForNCSG capability in LTE. Instead, the existing capability interRAT-NeedForGapsNR can be reused. As per TS36.331, this capability indicates need for measurement gaps when operating on the LTE band given in those BC lists and measuring on the NR band given by the entry in the InterRAT-BandListNR. So UE reporting ‘FALSE’ for interRAT-NeedForGapsNR can be defined as supporting the inter-RAT NR measurement without gaps.

InterRAT-BandInfoNR-r16 ::= SEQUENCE { interRAT-NeedForGapsNR-r16 BOOLEAN } interRAT-NeedForGapsNR Indicates need for measurement gaps when operating on the E-UTRA band given by the entry in supportedBandListEUTRA or on the E-UTRA band combination given by the entry in supportedBandCombination-r10 or supportedBandCombinationAdd-r11 or supportedBandCombinationReduced-r13 and measuring on the NR band given by the entry in the InterRAT-BandListNR.

Proposal 1: No need to extend the capability of NeedForNCSG to inter-RAT NR measurement. UE reporting ‘FALSE’ for interRAT-NeedForGapsNR can be defined as supporting the inter-RAT NR measurement without gaps.

Then in [1] two options are listed for issue 3-9, i.e., on top of which UE capability to define the inter-RAT NR measurement requirements

Option 1: NeedforNCSGgap-infoNR

Option 2: interRAT-NeedForGapsNR-r16, e.g., when UE report ‘FALSE’ through interRAT-NeedForGapsNR-r16 can be defined as inter-RAT NR measurement without gaps

We think it is useful to clarify on this part. In the previous discussions, there seems to be different understanding on these options. For example, in option 1, NeedForGapNCSG-InfoNR (not NeedforNCSGgap-infoNR) is specified in TS38.331 and it is an NR signalling, which is not applicable to LTE SA. Also the previous issue 3-8 in [1] already covers the capability aspects. Therefore, the only way to reasonably understand this issue 3-9 seems to be about which requirement framework is used as a baseline or starting point for RAN4 to develop the Inter-RAT NR measurements. Otherwise, this issue is the same as issue 3-8.

Proposal 2: It is confirmed that the previous issue 3-9 in the agreed WF is about which requirement framework is used as a baseline or starting point for RAN4 to develop the Inter-RAT NR measurements.

With such understanding the next question is what’s the baseline requirements for Option 2. In our view, it refers to the NR inter-frequency measurement without gaps, which is currently being discussed under another objective in this WID.

Proposal 3: For inter-RAT NR measurements, new measurement requirements can be defined based on NR inter-frequency measurement without gap (when ‘no-gap’ reported via NeedForGapsInfoNR).

2.2 Inter-RAT LTE measurements

First of all in [1] the issue 3-11 is about the target scenario of this work. The following are listed for further discussions.

FFS: For inter-RAT LTE measurement, at least consider the scenario when LTE CRS to be measured is contained in UE’s active BWP

FFS whether to consider the case when LTE CRS to be measured is not completely contained in UE’s active BWP, but there is spare RF chain

In our understanding, both the above cases may be meaningful when it comes to practical use cases. The next question is what would be the requirements or specification work for these different cases.

For the case when LTE CRS to be measured is not completely contained in UE’s active BWP, it seems that the Rel-17 requirements for NCSG are able to cover it. More specifically, the following signaling for NCSG can indicate the need for gap and NCSG for inter-RAT LTE measurement.

NeedForGapNCSG-InfoEUTRA-r17 ::= SEQUENCE { needForNCSG-EUTRA-r17 SEQUENCE (SIZE (1..maxBandsEUTRA)) OF NeedForNCSG-EUTRA-r17 } NeedForNCSG-EUTRA-r17 ::= SEQUENCE { bandEUTRA-r17 FreqBandIndicatorEUTRA, gapIndication-r17 ENUMERATED {gap, ncsg, nogap-noncsg} } gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform measurements on the concerned E-UTRA target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration message or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that NCSG is needed, value nogap-noncsg indicates neither a measurement gap nor a NCSG is needed.

The case when UE report ‘nogap-noncsg’ through NeedForGapNCSG-InfoEUTRA is defined as inter-RAT LTE measurement without gaps in existing specification. And RAN4 already agreed the following in R17 gap enhancement in the previous meeting

Define requirements for NR – LTE inter-RAT measurement without gap (UE reports ‘nogap-noncsg’ for inter-RAT measurement).

So we assume this will be done as part of Rel-17 and it can be inherited for Rel-18.

Proposal 4: There is no need to extend the capability of NeedForGaps to inter-RAT LTE measurement.

Proposal 5: The case when UE reporting ‘nogap-noncsg’ for NeedForGapNCSG-InfoEUTRA is defined as supporting the inter-RAT LTE measurement without gaps for the corresponding LTE target band. Rel-17 measurement requirements will be specified for this case, so no need to define new requirements in Rel-18.

Then the case when LTE CRS to be measured is contained in UE’s active BWP actually resembles the inter-frequency SSB measurement without gap, where it requires the target SSB is completely contained in the active BWP of the UE. Naturally, we prefer to take the measurement requirements for LTE inter-frequency measurement as a starting point but more detailed discussion is needed since only gap-based inter-frequency measurement is defined in LTE.

Proposal 6: For the case when LTE CRS to be measured is contained in UE’s active BWP, new measurement requirements for inter-RAT LTE measurement without gaps need to be defined and the measurement requirements for LTE inter-frequency measurement can be taken as a starting point.

3 Summary

This contribution discusses RRM requirements for Inter-RAT measurement without gap. The following proposals are made.

Inter-RAT NR measurements

Proposal 1: No need to extend the capability of NeedForNCSG to inter-RAT NR measurement. UE reporting ‘FALSE’ for interRAT-NeedForGapsNR can be defined as supporting the inter-RAT NR measurement without gaps.

Proposal 2: It is confirmed that the previous issue 3-9 in the agreed WF is about which requirement framework is used as a baseline or starting point for RAN4 to develop the Inter-RAT NR measurements.

Proposal 3: For inter-RAT NR measurements, new measurement requirements can be defined based on NR inter-frequency measurement without gap (when ‘no-gap’ reported via NeedForGapsInfoNR).

Inter-RAT LTE measurements

Proposal 4: There is no need to extend the capability of NeedForGaps to inter-RAT LTE measurement.

Proposal 5: The case when UE reporting ‘nogap-noncsg’ for NeedForGapNCSG-InfoEUTRA is defined as supporting the inter-RAT LTE measurement without gaps for the corresponding LTE target band. Rel-17 measurement requirements will be specified for this case, so no need to define new requirements in Rel-18.

Proposal 6: For the case when LTE CRS to be measured is contained in UE’s active BWP, new measurement requirements for inter-RAT LTE measurement without gaps need to be defined and the measurement requirements for LTE inter-frequency measurement can be taken as a starting point.

4 References

- R4-2214346 WF on further enhancements on measurement gaps and measurements without gaps

- R4-2215427, Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR, CATT