3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215446

Electronic Meeting, Oct. 10 – 19, 2022

Agenda Item: 6.20.2

Source: MediaTek Inc.

Title: Discussion on improvement on FR2 SCell/SCG setup/resume

Document for: Discussion

- Introduction

RAN4#104-e had extensive discussion on candidate enhanced measurement to reduce FR2 SCell/SCG setup/resume delay. In this contribution, we will further provide our views based on the discussion in RAN4#104-e.

- Discussion

Based on the discussion in RAN4#104-e, there are two potential directions for further study. One is measurement enhancement during RRC connection setup/resume (measurement in the orange part in Fig.1). Another is further enhancement on EMR (measurement in the green part in Fig.1).

Fig. 1

For the latter, it refers to the enhanced measurement that is performed in idle/inactive mode and before UE initiating access. According to the WID [1], it is clear that the improved measurement to study starts when UE requests RRC connection setup/resume. Therefore, the latter direction is not in the scope.

- To study the following, with completion targeted by RAN#98 meeting [RAN4]: - The impact of FR2 RRM mobility measurement acquisition and reporting on FR2 SCell/SCG setup/resume delay for a UE connecting from idle/inactive mode. - The level of feasible improvement in FR2 SCell/SCG setup delay from defining new UE measurement procedures and RRM core requirements, and whether additional information from the network would help the UE to perform those measurements effectively. The following sequence of events should be assumed. - The UE initiates and performs improved measurements when it requests RRC connection setup/resume. - After acquiring those improved measurements, the UE subsequently reports those measurements to the network to support SCell/SCG setup.

Observation 1: Enhanced early measurement in idle/inactive mode is not in the scope.

For the former, the key issue is how long the time can be used for enhanced measurement. So in last meeting, RAN4 had a lot of discussion on the start point of the measurement and some company proposed to clarify when UE initiates RRC connection setup/resume. The WF is as follows [2]. Next, we would like to provide our views on the two issues.

<Way forward >: Issue 2-1-1: Carification on time point of “when UE has initiated access” - Option 1 (Ericsson, Nokia): RAN4 to clarify that the “when UE has initiated access” is the point in time when RRC procedures in clauses 5.3.2.3, 5.3.3.2 or 5.3.13.2 (38.331) are initiated. - Paging - Initiation (UE receives SIB1 and apply default MAC cell group configuration) - RRC connection resume - Option 2 (CMCC, Apple, QC, xiaomi, HW, vivo, MTK): Discuss the starting point for improved measurement directly. <Way forward >: Issue 2-1-2: Potential direction for further study: measurement enhancement when UE is about to enter connected mode - Option 1(CMCC): UE perform enhanced measurement closely before RRC connection setup/resume. - For MT originating call, the measurement is performed during the period between paging reception and UE send RRCResumeRequest/ RRCSetupRequest. - For MO originating call, the measurement is performed after upper layers request establishment of an RRC connection and before UE send RRCResumeRequest/ RRCSetupRequest. - Option 2 (Apple, Ericsson, Nokia): UE perform enhanced measurement during RRC connection setup/resume. - Option 2a: perform FR2 cell measurements during connection setup (i.e. paging, RRC connection establishment, RRC connection resume) - Option 2b (MTK, vivo, xiaomi, HW): after receiving paging for MT originating call - Option 2c (MTK, vivo, xiaomi, HW): after first RACH preamble transmission, i.e. Msg1

We copy the related spec in 38.331 as below for convenience. In our understanding, upon receiving paging, UE behaviour would be as following:

- Idle mode: after decoding paging and identifying UE-ID in PagingRecord, then forward paging information to the upper layers.

- Inactive mode:

  - RAN paging: after decoding paging and identifying UE-ID in PagingRecord, then initiate RRC connection resumption procedure.

  - CN paging: after decoding paging and identifying UE-ID in PagingRecord, then forward paging information to the upper layers and kind of go to idle mode.

According to cl. 5.3.3.2, when UE is in idle mode, UE initiates RRC connection setup procedure upon upper layers request both for MO (mobile originate) call and MT (mobile terminate) call. According to cl. 5.3.13.2, when UE is in inactive mode, UE initiates RRC connection resumption procedure upon upper layers request (MO call) or RAN paging (MT call). No matter the initiation is triggered by upper layers request or RAN paging, UE would first encode RRCSetupRequest or RRCResumeRequest. The time needed in this step is up to UE implementation. And then UE would transmit Msg1 on available RACH occasion. The time needed in this step is up to NW configuration. In summary, the time mainly contains two parts from UE initiating RRC connection setup/resumption procedure to Msg1. One part is up to UE implementation, the other is up to NW configuration. Considering that it is better to define a new behavior that is testable, we suggest using Msg1 as the start point for later discussions of potential enhanced measurement.

#### 5.3.2.3 Reception of the Paging message by the UE or PagingRecord by the L2 U2N Remote UE Upon receiving the Paging message by the UE or receiving PagingRecord from its connected L2 U2N Relay UE by a L2 U2N Remote UE, the UE shall: 1> if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message, or 1> if in RRC_IDLE, for each of the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE: 2> if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers: 3> if upper layers indicate the support of paging cause: 4> forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers; 3> else: 4> forward the ue-Identity and accessType (if present) to the upper layers; 1> if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or 1> if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE: 2> if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI: 3> if the UE is configured by upper layers with Access Identity 1: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess; 3> else if the UE is configured by upper layers with Access Identity 2: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess; 3> else if the UE is configured by upper layers with one or more Access Identities equal to 11-15: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess; 3> else: 4> initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access; NOTE: A MUSIM UE may not initiate the RRC connection resumption procedure, e.g. when it decides not to respond to the Paging message due to UE implementation constraints as specified in TS 24.501 [23]. 2> else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers: 3> if upper layers indicate the support of paging cause: 4> forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers; 3> else: 4> forward the ue-Identity and accessType (if present) to the upper layers; 3> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'; #### 5.3.3.2 Initiation The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE and it has acquired essential system information, or for sidelink communication as specified in clause 5.3.3.1a. #### 5.3.13.2 Initiation The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/V2X sidelink communication as specified in clause 5.3.13.1a) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

Proposal 1: During feasibility evaluation, assume that UE starts to perform enhanced measurement during RRC connection setup/resume procedure after first RACH preamble transmission, i.e. Msg1.

According to the data 3GPP submitted to IMT2020 [3], the typical RRC connection setup/resume delay is less than 20ms. This 20ms is from preamble to RRC Resume Complete message. But the measurement period is as long as 480ms for intra-frequency without gap in connected mode when Mmeas_period_w/o_gaps =24 (power class 3, handheld UE), Kp =1, Klayer1_measurement=1, CSSFintra=1, SMTC = 20ms and no DRX. Here we choose measurement period of intra-frequency without gap in connected mode to compare with, due to it is the shortest one for FR2. It can be observed that it is not possible to finish the measurement even on a single frequency layer during RRC connection setup/resume delay.

For information: From 38.133 Table 9.2.5.2-2: Measurement period for intra-frequency measurements without gaps (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

In last meeting, some methods are proposed to reduce measurement delay, such as reducing number of frequency layers, reducing beam sweeping factors and/or reducing the samples. We will share our views on these methods.

- Reduce number of frequency layers to measure

As shown above, it is not possible to finish the measurement even on a single frequency layer during RRC connection setup/resume delay. It is meaningless to discuss whether and how to reduce number of frequency layers to measure any more.

- Reduce beam sweeping factors and/or the samples

It is to reduce the value of Mmeas_period_w/o_gaps in our understanding. Some company proposed to use the value of scaling factor 2 or 6 pending on the different deployment as in FR2 HST. As the target deployment in this WI is different from FR2 HST, we don’t think it is applicable here.

Some company proposed to introduce UE capability indicating the support of smaller Rx beam sweeping factor as positioning. Supporting smaller Rx beam sweeping factor may require not only more panels but also more simultaneous active RF chains. It will lead to much higher cost to require such higher UE capability just for the measurement during RRC setup/resume. Without multiple panels, it means UE as to use less Rx beams for measurement, which leads to insufficient spherical coverage. And thus the reported measurement accuracy are compromised.

Regarding the method of using previous beam information, it cannot guarantee that beam direction remains the same as previous measurement, so accuracy would not be guaranteed.

Regarding the method of reducing the samples, accuracy cannot be guaranteed either.

We also want to point out that RRC connection setup/resume procedure is more important and urgent than measurement. Enhanced measurement should bring zero impact on RRC connection setup/resume procedure. Due to RF retuning needed, it is not feasible to measure inter-band frequency layers during RRC connection setup/resume procedure with same active RF chain of serving cell or other idle RF chains.

Proposal 2: Enhanced measurement should bring zero impact on RRC connection setup/resume procedure.

Observation 2: Due to RF retuning needed, it is not feasible to measure inter-band frequency layers during RRC connection setup/resume procedure with same active RF chain of serving cell or other idle RF chains.

Anyway, considering that RRC setup/resume delay is too short and the proposed methods fail to reduce the measurement delay with guaranteed accuracy, we think improved measurement during RRC setup/resume procedure is not feasible.

Proposal 3: Improved measurement during RRC setup/resume procedure is not feasible.

- Summary

In this paper, we have some discussion on improvement of FR2 SCell/SCG setup/resume. We have the following proposals and observations:

Observation 1: Enhanced early measurement in idle/inactive mode is not in the scope.

Proposal 1: During feasibility evaluation, assume that UE starts to perform enhanced measurement during RRC connection setup/resume procedure after first RACH preamble transmission, i.e. Msg1.

Proposal 2: Enhanced measurement should bring zero impact on RRC connection setup/resume procedure.

Observation 2: Due to RF retuning needed, it is not feasible to measure inter-band frequency layers during RRC connection setup/resume procedure with same active RF chain of serving cell or other idle RF chains.

Proposal 3: Improved measurement during RRC setup/resume procedure is not feasible.

4 Reference

RP-221799, “Revised WID on Further NR Mobility Enhancements”, MediaTek Inc., RANP#96 meeting, June 2022.

R4-2214348, “WF on Further NR Mobility Enhancements”, MediaTek Inc., RAN4#104 e-meeting, Aug. 2022.

37.910, “Study on self evaluation towards IMT-2020 submission”, 3GPP