3GPP TSG-RAN4 Meeting #104bis-e 	R4-2215448

E-meeting, 10th Oct. – 19th Oct., 2022

Title:	Discussion on the remaining issues for NTN RRM

Source:	Xiaomi, CAICT

Agenda Item:	4.2.5

Document for:	Discussion

- Introduction

In RAN#104-e meeting, the remaining issues for NTN RRM has been discussed, and the WF on NR NTN RRM requirements was approved in [1]. However, the issue of the requirement for fully overlapping concurrent gaps is still open. In this contribution, we discuss this remaining issue and provide our proposals.

- Discussion

Fully Overlapping Concurrent MGs

Agreement: - Option 1: Do not define requirements for fully overlapping concurrent MGs - Option 2: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2 - Option 2A: - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms. - A MG with the lowest ID, i.e. 0, gets priority over the other, and the dropping rule starts from SFN=0, i.e. MG-ID#0 is selected and MG-ID#1 is dropped at the first collision instance after SFN=0, and it alternates afterwards. - [RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only.] - Option 2B: - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms. - RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only. - Option 2C: - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

In last meeting, majority companies supported to use gap sharing rules for fully overlapped case. And FFS the applicability for gap sharing rule for fully overlapped case. In my understanding, there is no need to define the UE behavior when gap sharing rules are applied. In legacy intra/inter-frequency measurement requirement, the sharing rules are applied when defining the CSSF scaling factor, and there is no specific UE behavior on which carrier should be measured or dropped. It is totally up to UE implementation as long as the sharing mechanism works. Thus, we think no need to define the dropping rule when gap sharing rules are applied.

Regarding whether to introduce a new UE capability to support fully overlapping concurrent MGs in NTN, we also think there is no need to introduce such UE capability. In my understanding, whether the concurrent MGs are fully overlapped or non-fully overlapped is not related to the UE capability but related to NW configuration. In addition, in TN system, there is no specific UE capability to support fully overlapping concurrent MGs.

Proposal 1: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2.

- It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

- Conclusion

In this contribution, we discuss the remaining issue for NR NTN RRM and proposal are provided as follows:

Proposal 1: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2.

•	It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

- Reference

[1]	R4-2214471	WF on NR NTN RRM requirements, Qualcomm