3GPP TSG-RAN4 Meeting #104-bis-e 	R4-2215457

E-meeting, 10th Oct. – 19th Oct., 2022

Title:	RRM requirements for the combination of concurrent gaps, pre-configured MG and NCSG

Source:	Xiaomi

Agenda Item:	6.10.2

Document for:	Discussion

- Introduction

In RAN4#104-e meeting, Ran4 had some initial discussion on RRM requirements for UEs configured with a combination of pre-configured MGs, and/or multiple concurrent MGs and/or NCSG, and the related WF was approved in [1]. In this contribution, we would like to discuss the related requirements and provide our proposals.

- Discussion

Scenarios

- Issue 2-1: Whether to define requirements for one Pre-configured MG and one NCSG < Way forward >: - RAN4 to clarify whether one Pre-configured MG and one NCSG is included in Case 1 and/or Case 2 - FFS to explicitly set the priority of different combinations - Issue 2-2: Whether to define requirements for one Pre-configured NCSG < Agreement >: - RAN4 focuses on Case 1 and Case 2 first. RAN4 can continue discuss whether and when to discuss requirements for Pre-configured NCSG - Issue 2-3: NR SA and MR-DC < Way forward >: - FFS whether to deprioritize MR-DC in Rel-18

In this WI, RAN4 will define the joint requirements for multiple concurrent gaps, pre-configured MG and NCSG, and, case 1 and case 2 are prioritized. According to RAN#97 plenary discussion, it was clarified that the requirements for gap combination of pre-MG + NCSG will be discussed after RAN#99 meeting.

Proposal 1: RAN4 starts to discuss the joint requirements for UE configured with pre-configured MG and NCSG after RAN#99 meeting.

Regarding whether to deprioritize MR-DC in Rel-18, some companies supported to deprioritize MR-DC scenarios as there may need some configuration information exchange between MN and SN. However, according to TS37.340 in [2], if per-UE gap is used, the MN will decide the gap configuration,. If per-FR gap is used, MN will decide both FR1 and FR2 gap configuration in NR-DC and NE-DC scenarios, and in EN-DC case, the MN will decide FR1 related gap configuration and SN will decide FR2 related gap configuration. In addition, the measurement gap configuration assistance information can be exchanged between the MN and SN. Thus, there is no extra work to support MR-DC scenario in this WI.

TS37.340 section 7.2: If per-UE gap is used, the MN decides the gap pattern and the related gap sharing configuration. If per-FR gap is used, in EN-DC and NGEN-DC, the MN decides the FR1 gap pattern and the related gap sharing configuration for FR1, while the SN decides the FR2 gap pattern and the related gap sharing configuration for FR2; in NE-DC and NR-DC, the MN decides both the FR1 and FR2 gap patterns and the related gap sharing configurations. In EN-DC and NGEN-DC, the measurement gap configuration from the MN to the UE indicates if the configuration from the MN is a per-UE gap or an FR1 gap configuration. The MN also indicates the configured per-UE or FR1 measurement gap pattern and the gap purpose (per-UE or per-FR1) to the SN. Measurement gap configuration assistance information can be exchanged between the MN and the SN. For the case of per-UE gap, the SN indicates to the MN the list of SN configured frequencies in FR1 and FR2 measured by the UE. For the per-FR gap case, the SN indicates to the MN the list of SN configured frequencies in FR1 measured by the UE and the MN indicates to the SN the list of MN configured frequencies in FR2 measured by the UE. In NE-DC, the MN indicates the configured per-UE or FR1 measurement gap pattern to the SN. The SN can provide a gap request to the MN, without indicating any list of frequencies. In NR-DC, the MN indicates the configured per-UE, FR1 or FR2 measurement gap pattern and the gap purpose to the SN. The SN can indicate to the MN the list of SN configured frequencies in FR1 and FR2 measured by the UE.

Observation 1: No extra work is needed to introduce the joint requirements for the combination of concurrent gaps, pre-MG and NCSG in MR-DC scenarios.

Proposal 2: RAN4 does not deprioritize to define the joint requirements for the combination of concurrent gaps, pre-MG and NCSG in MR-DC scenarios in Rel-18.

- Issue 2-7: Legacy measurement gaps < Way forward >: - Proponents are encouraged to clarify the proposal “consider legacy MG, Pre-MG, concurrent MG, and NCSG when we discuss joint MG requirements.” in the next meeting. - Issue 2-8: Positioning measurement gaps < Agreement >: - Rel-17 MGE Pre-MG for RRC-based POS is in the scope of this WI < Way forward >: - FFS Rel-17 MAC-CE based ePOS gap is in or out of scope of this WI

In RAN#97 plenary meeting, the definition of legacy MG was discussed in [3], and majority companies supported that the legacy MG can be defined as the gap(s) configured via GapConfig without suffix. In addition, RAN2 discussed and supported the concurrent gap configuration including legacy gap via legacy field (without gap ID and gap priority) and other gap via new field (with gap ID and gap priority), and there is no requirement in case of gap collision. From RAN4 perspective, the UE measurement behaviour is confused and joint requirement is complicated when the legacy gap is collided with the gap configured with gap priority. Thus, we propose to follow RAN2 agreement that no joint requirements are defined for the gap combination of legacy MG and other gap(s) configured via GapConfig with suffix.

- Agreement in RAN2#119e [4]: For concurrent gap configuration, RAN2 understands that: The NW could configure one gap via legacy field (without gap ID and gap priority) and the other gap via new field (with gap ID and gap priority). RAN2 observes that in this configuration, there is currently no requirement in case of gap collision.

Proposal 3: Legacy MG is defined as the gap(s) configured via GapConfig without suffix.

Proposal 4: RAN4 not to define the joint requirements for the gap combination of legacy MG and other gap(s) configured via GapConfig with suffix.

Regarding Rel-17 MAC-CE based ePOS gap, since the pre-configured MG for positioning measurements beyond Rel-16 is not considered in Rel-17 pre-MG agreed in [5], thus, we think ePOS gap should not be considered in the joint combination of pre-MG, concurrent gap and NCSG, and it is outside the scope of WI.

Proposal 5: Rel-17 MAC-CE based ePOS gap is out of scope of this WI.

Max number of supported gaps

- Issue 2-11: Max # of gaps for Case 1 (Pre-configured MG and multiple concurrent MGs) < Agreement >: - For the max number of gaps for Case 1 (Pre-configured MG and multiple concurrent MGs), the Rel-17 conclusions will be taken as the baseline. < Way forward >: - FFS whether to increase the max number - FFS whether to introduce new UE capability for # of supported Pre-MG - Issue 2-12: Max # of gaps for Case 2 (NCSG and multiple concurrent MGs) < Agreement >: - For the max number of gaps for Case 2 (NCSG and multiple concurrent MGs), the Rel-17 conclusions will be taken as the baseline. < Way forward >: - FFS whether to increase the max number - FFS whether to introduce new UE capability for # of supported NCSG - Issue 2-13: Detail combinations < Way forward >: - Companies are encouraged to bring high-level principle about the possible gap combinations before going into details

The aim of the this WI is to introduce the joint requirements for the combination of concurrent gaps, pre-MG and NCSG, thus, the max number of gaps for case 1 and case 2 should be the same as what is supported in Rel-17. Otherwise, RAN4 need to define additional requirements for the case that the max number of gaps is larger than what is supported in Rel-17. Thus, we don’t see the motivation and benefit to support more MGs than what is supported in Rel-17, instead, it will bring additional complexity on NW scheduling and UE implementation.

Proposal 6: The max number of gaps for case 1 and case 2 is the same as what is supported in Rel-17.

Collision definition and handling

- Issue 2-14: Required changes for Pre-MG on collision < Agreement >: - For Case 1 (Pre-configured MG and multiple concurrent MGs), the baseline requirement considers collisions on Pre-MG is only considered when Pre-MG is activated. < Way forward >: - FFS whether Pre-MG priority can be further decided by the associated Mos being measured or other signalling parameters - Issue 2-15: Required changes for NCSG on collision < Agreement >: - When determining if a collision occurs between a NCSG instance and another gap instance, the baseline requirement considers the total NCSG duration, including both the VILs and the ML. < Way forward >: - FFS other potential enhancements regarding spare RF chains, interruption due to collision on RRT, revised proximity condition, … etc. - Issue 2-16: general gap collision handling < Agreement >: - On gap collision handling, take priority rule and overlapping rules from Rel 17 concurrent gaps as the baseline < Way forward >: - FFS whether to consider gap sharing rule

When UE is configured with concurrent gap and NCSG, the proximity condition and gap collision may need to be revisited, since it is assumed that UE perform the measurement with NCSG in idle Rx chain. If the MGL of legacy gap is overlapped with the ML of NCSG, even though UE can simultaneously perform the measurement with legacy gap and NCSG, the reception in ML would be interrupted due to RF retuning time or the reception in MGL would be interrupted by VIL, as shown in figure 2 and 3. If the impact on measurement performance due to RTT is negligible, UE can perform the measurements on the collided gaps simultaneously and no need to consider the dropping rule.

And the proximity condition for gap collision may not be needed, since UE may use the different Rx chain to perform the measurement with legacy gap and NCSG respectively.

Figure 2: ML of NCSG is interrupted by RF retuning

Figure 3: Collision happens when VIL1 or VIL2 is overlapped with MGL

Proposal 7: When the MGL of concurrent gap or the activated pre-configured MG is overlapped with the ML of NCSG, or when VIL1/VIL2 of NCSG is overlapped with the MGL of concurrent gap or the activated pre-configured MG, if the impact on measurement performance due to RTT is negligible, UE can perform the measurements on the collided gaps simultaneously and no need to consider the dropping rule.

Proposal 8: No need to consider proximity condition for gap collision between concurrent gap and NCSG.

Measurement delay requirements

- Issue 2-17: Required changes for Pre-MG on related delays < Way forward >: - FFS whether the measurement requirements with concurrent MGs defined in Rel-17 can be reused except that only activated gaps are considered when defining CSSF - FFS whether the pre-configured gap activation delay requirements need to be extended, when the activation procedures of multiple pre-configured gaps overlap.

In Rel-17 multiple concurrent gap, it was agreed that the CSSF should be calculated separately for each concurrent gap and the associated measurement types. And we think the same principle can be applied to the joint configuration for concurrent gaps, pre-configured MG and NCSG, and for pre-MG related requirement, only the activated gaps are considered when defining CSSF.

Proposal 9: The measurement requirements with concurrent MGs defined in Rel-17 can be reused provided that only activated gaps are considered when defining CSSF.

When the pre-configured MG activation procedure is overlapped with the concurrent gap, UE may not be able to perform the measurement on the concurrent gap occasion.

Proposal 10: When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion, UE shall drop the collided concurrent gap occasion.

- Conclusion

In this contribution, we discuss the related requirements for the joint configuration of concurrent gap, pre-configured MG and NCSG and provide our proposals as follows.

Proposal 1: RAN4 starts to discuss the joint requirements for UE configured with pre-configured MG and NCSG after RAN#99 meeting.

Observation 1: No extra work is needed to introduce the joint requirements for the combination of concurrent gaps, pre-MG and NCSG in MR-DC scenarios.

Proposal 2: RAN4 does not deprioritize to define the joint requirements for the combination of concurrent gaps, pre-MG and NCSG in MR-DC scenarios in Rel-18.

Proposal 3: Legacy MG is defined as the gap(s) configured via GapConfig without suffix.

Proposal 4: RAN4 not to define the joint requirements for the gap combination of legacy MG and other gap(s) configured via GapConfig with suffix.

Proposal 5: Rel-17 MAC-CE based ePOS gap is out of scope of this WI.

Proposal 6: The max number of gaps for case 1 and case 2 is the same as what is supported in Rel-17.

Proposal 7: When the MGL of concurrent gap or the activated pre-configured MG is overlapped with the ML of NCSG, or when VIL1/VIL2 of NCSG is overlapped with the MGL of concurrent gap or the activated pre-configured MG, if the impact on measurement performance due to RTT is negligible, UE can perform the measurements on the collided gaps simultaneously and no need to consider the dropping rule.

Proposal 8: No need to consider proximity condition for gap collision between concurrent gap and NCSG.

Proposal 9: The measurement requirements with concurrent MGs defined in Rel-17 can be reused provided that only activated gaps are considered when defining CSSF.

Proposal 10: When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion, UE shall drop the collided concurrent gap occasion.

- Reference

[1]	R4-2214346	WF on further enhancements on measurement gaps and measurements without gaps,	MediaTek Inc

[2]	TS37.340 V17.1.0

[3]	RP-222585	Moderator's summary for discussion [97e-25-R18-MeasGapEnh],	MediaTek Inc

[4]	Report of 3GPP TSG RAN WG2 meeting #119-e, Online

[5]	R4-2120301	WF on R17 NR MG enhancements – Pre-configured MG		Intel Corporation