3GPP TSG-RAN4 Meeting #104-bis-e 	R4-2215458

E-meeting, 10th Oct. – 19th Oct., 2022

Title:	Discussion on improvement of FR2 SCell/SCG setup

Source:	Xiaomi

Agenda Item:	6.20.2

Document for:	Discussion

- Introduction

In RAN4#104-e meeting, RAN4 has some initial discussion on the feasibility of the improvement in FR2 SCell/SCG setup delay, and the related WF was approved in [1]. In this contribution, we would like to discuss the related issues and provide our proposals.

- Discussion

Clarification and potential direction

<Way forward >: Issue 2-1-1: Carification on time point of “when UE has initiated access” - Option 1 (Ericsson, Nokia): RAN4 to clarify that the “when UE has initiated access” is the point in time when RRC procedures in clauses 5.3.2.3, 5.3.3.2 or 5.3.13.2 (38.331) are initiated. - Paging - Initiation (UE receives SIB1 and apply default MAC cell group configuration) - RRC connection resume - Option 2 (CMCC, Apple, QC, xiaomi, HW, vivo, MTK): Discuss the starting point for improved measurement directly. <Way forward >: Issue 2-1-2: Potential direction for further study: measurement enhancement when UE is about to enter connected mode - Option 1(CMCC): UE perform enhanced measurement closely before RRC connection setup/resume. - For MT originating call, the measurement is performed during the period between paging reception and UE send RRCResumeRequest/ RRCSetupRequest. - For MO originating call, the measurement is performed after upper layers request establishment of an RRC connection and before UE send RRCResumeRequest/ RRCSetupRequest. - Option 2 (Apple, Ericsson, Nokia): UE perform enhanced measurement during RRC connection setup/resume. - Option 2a: perform FR2 cell measurements during connection setup (i.e. paging, RRC connection establishment, RRC connection resume) - Option 2b (MTK, vivo, xiaomi, HW): after receiving paging for MT originating call - Option 2c (MTK, vivo, xiaomi, HW): after first RACH preamble transmission, i.e. Msg1 <Way forward >: Issue 2-1-3: Potential direction for further study: enhancement on R16 EMR, i.e. measurement enhancement in idle/inactive mode - Option 1 (Apple, xiaomi, MTK, HW, vivo): further enhancement on Rel-16 EMR (measurement during green part) is out of scope - Option 2 (Ericsson, Nokia, LGE): further enhancement on Rel-16 EMR (measurement during green part) is in the scope Note: Option 1 is not precluding using the measurement results obtained during EMR for measurement during RRC connection procedure. “Enhancement on R16 EMR (measurement during green part)” refers to that the enhanced measurement is still performed in idle/inactive mode and before UE initiating access. <Way forward/Agreement>: Issue 2-1-4: Applicable scenarios Agreement: - The enhanced measurement (if feasible) at least applies to scenarios that the EMR measurement results are unavailable or invalid. Way forward - Revised Option 2 (QC, vivo, LGE, Nokia): Further study whether enhanced measurement is applicable to scenarios that EMR measurement results are available and valid - e.g. On the cells that have been detected/measured in early measurement

In last meeting, the clarification on the time point of “when UE has initiated access” was discussed, and in our understanding, it is about the starting point when UE initiates the improved measurement during RRC connection setup/resume procedure. And according to the email discussion summary in [2], for MT-originating calls, the earliest time for the potential improved measurement is the time after receiving paging, and for MO-originating calls, the earliest time for the potential improved measurement is the time after Msg 1 transmission.

Proposal 1: The time point of “when UE has initiated access” is the starting point when UE initiates the improved measurement during RRC connection setup/resume procedure.

Proposal 2: The starting point for the improved measurement during RRC connection setup/resume is the time:

- after receiving paging for MT originating call

- after Msg1 transmission for MO originating call

Another aspect is on whether the enhancement on Rel-16 EMR requirement is in or out of scope of WI. In our understanding, this objective is to study the feasibility on the improvement in FR2 SCell/SCG setup delay by defining new UE measurement procedures when the EMR measurement results are not available or invalid. The shorter EMR delay may not be able to resolve this issue, as UE may request RRC connection setup/resume after the EMR measurement result is outdated. And if the EMR measurement results are available or valid, the UE does not need to trigger the improved measurement during the RRC connection setup/resume procedure.

Proposal 3: Further enhancement on Rel-16 EMR requirements is out of scope.

Proposal 4: If the EMR measurement results are available or valid, the UE does not need to trigger the improved measurement during the RRC connection setup/resume procedure.

Assumptions for feasibility study

<Way forward >: Issue 2-2-1: Assumption for feasibility study: RF chain status when performing enhanced measurement - Option 1 (Apple, HW, QC): One active RF chain - Option 2 (CMCC, xiaomi, MTK, Ericsson, vivo, Nokia): Two active RF chains - Option 3 (Nokia): more than two active RF chains <Agreement >: Issue 2-2-2: Assumption for feasibility study: number of frequency layers - Reduce the number of EMR carriers to be measured for improved measurement. - FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable. <Way forward >: Issue 2-2-3: Assumption for feasibility study: Reduced number of samples - Option 1: Yes - Option 2 (Apple, MTK, vivo, HW ): No - Option 3 (CMCC, xiaomi, Ericsson, LGE, Nokia): FFS how to reduce number of samples without measurement accuracy degradation. <Way forward >: Issue 2-2-4: Assumption for feasibility study: Reduce the scaling factor of Rx beam sweeping - Option 1(Ericsson, Nokia): Reduce the scaling factor of Rx beam sweeping - Option 1a (CMCC): use Rx beam sweeping factor of R17 HST FR2 or R17 positioning - For FR2 HST, the value of scaling factor is 2 or 6 pending on the different deployment. - In Rel-17 positioning WI, the candidate Rx beam sweep numbers for reduced Rx beam sweeping factor (<8) UE capability are {1, 2, 4, 6}. - Option 1b (xiaomi, Ericsson, vivo, QC): FFS use prior information on the UE Rx beam to reduce the scaling factor of Rx beam sweeping - Option 2 (Apple, MTK, HW): Not to reduce the scaling factor of Rx beam sweeping - Option 3 (Apple, Ericsson): Further discussion <Way forward/Agreement>: Issue 2-2-5: Assumption for feasibility study: Configuration assumption Agreement: DRX is not in use during RRC connection setup/resume procedure for enhanced measurement. Way forward: - Option 1 (vivo, QC): use SMTC when specifying the requirements - Option 2 (Ericsson ): use SSB period when specifying the requirements - Option 3 (Apple, xiaomi, Ericsson, MTK, HW, Nokia): Further discussion

For active RF chain assumption, as UE may need to sweep its Rx beam to perform the potential improved measurement during RRC connection setup/resume procedure, the UE may not be able to receive Msg 2/4 during RACH procedure. Thus, it is reasonable to assume 2 active Rx chain for inter-band carrier used for inter-band CA/DC set-up purpose. For intra-band CA/DC case, one active Rx chain is assumed, and according to the side condition for FR2 SCell activation requirements, the same Rx beam is assumed for receiving the serving cell and the target carrier, thus, there is no impact on RACH procedure due to Rx beam sweeping.

Observation 1: For intra-band CA/DC case, the same Rx beam is assumed for receiving the SCC and the target intra-band CC, and there is no impact on RACH procedure due to Rx beam sweeping.

Proposal 5: For FR2 inter-band CA/DC case, 2 active Rx chains is assumed when performing the improved measurement during RRC connection set-up/resume.

Proposal 6: For FR2 intra-band CA/DC case, one active Rx chain is assumed when performing the improved measurement during RRC connection set-up/resume.

In existing TS38.133, the EMR measurement delay is defined as Kcarrier * Tdetect,NR_Inter + k*TSSB_index,NR if UE is required to report the index of the SSB being measured. And it is observed that the measurement delay in FR2 is dominant by the scaling factor due to Rx beam sweeping and the number of EMR carriers to be measured. Thus, reducing the number of EMR carrier to be measured is one of the feasibility to reduce the delay of improved measurement. Considering the available measurement time during RRC connection setup/resume, UE is required to measure only one of EMR carrier in improved measurement procedure, and there are the following alternatives to select the carrier to be measured:

- Alternative 1: Select the carrier which has the best single quality among the EMR measurement results.

- Alternative 2: Select the carrier which has the highest priority among the configured EMR carriers.

Proposal 7: UE is required to measure one of EMR carrier in improved measurement procedure, and there are the following alternatives is considered to select the carrier to be measured:

- Alternative 1: Select the carrier which has the best single quality among the EMR measurement results.

- Alternative 2: Select the carrier which has the highest priority among the configured EMR carriers.

Another aspect is to reduce the scaling factor due to Rx beam sweeping, since the selected carrier to be measured may have been measured during EMR measurement procedure, UE may have the prior information of Rx beam for the selected carrier. When the UE initiate the improved measurement during the RRC connection procedure, the UE can use sub-set of Rx beam to measure the selected carrier, e.g. from 8 to 4. And the side condition can be considered to apply the feasibility to reduce the scaling factor of Rx beam sweeping, e.g.

- The carrier to be measured has been measured during EMR measurement procedure

- The measured RSRP is higher than a threshold

Proposal 8: It is feasible to reduce the scaling factor of Rx beam sweeping in improved measurement provided the following conditions are fulfilled:

- The carrier to be measured has been measured during EMR measurement procedure

- The measured RSRP is higher than a threshold

Regarding whether it is feasibility to reduce the number of samples in improved measurement, in our understanding, if the measurement accuracy is not degraded, RAN4 can study how to reduce the number of the measurement samples, e.g. from 5 sample to 3 samples.

Proposal 9: If the measurement accuracy is not degraded, it is feasibility to reduce the number of samples for the improved measurement.

In last meeting, it was agreed that DRX is not used in improved measurement during RRC connection setup/resume to further reduce the measurement delay, and FFS on SMTC. In our understanding, the SMTC period can be configured as the same as SSB period, in addition, UE is not aware of SSB configuration before enter RRC_Connected mode.

Proposal 10: RAN4 to use SMTC configuration when specifying the requirements for improved measurement during RRC connection setup/resume.

- Conclusion

In this contribution, we discuss the potential feasibility on the improvement in FR2 SCell/SCG setup delay and provide our proposals as follows.

Proposal 1: The time point of “when UE has initiated access” is the starting point when UE initiates the improved measurement during RRC connection setup/resume procedure.

Proposal 2: The starting point for the improved measurement during RRC connection setup/resume is the time:

- after receiving paging for MT originating call

- after Msg1 transmission for MO originating call

Proposal 3: Further enhancement on Rel-16 EMR requirements is out of scope.

Proposal 4: If the EMR measurement results are available or valid, the UE does not need to trigger the improved measurement during the RRC connection setup/resume procedure.

Observation 1: For intra-band CA/DC case, the same Rx beam is assumed for receiving the SCC and the target intra-band CC, and there is no impact on RACH procedure due to Rx beam sweeping.

Proposal 5: For FR2 inter-band CA/DC case, 2 active Rx chains is assumed when performing the improved measurement during RRC connection set-up/resume.

Proposal 6: For FR2 intra-band CA/DC case, one active Rx chain is assumed when performing the improved measurement during RRC connection set-up/resume.

Proposal 7: UE is required to measure one of EMR carrier in improved measurement procedure, and there are the following alternatives is considered to select the carrier to be measured:

- Alternative 1: Select the carrier which has the best single quality among the EMR measurement results.

- Alternative 2: Select the carrier which has the highest priority among the configured EMR carriers.

Proposal 8: It is feasible to reduce the scaling factor of Rx beam sweeping in improved measurement provided the following conditions are fulfilled:

- The carrier to be measured has been measured during EMR measurement procedure

- The measured RSRP is higher than a threshold

Proposal 9: If the measurement accuracy is not degraded, it is feasibility to reduce the number of samples for the improved measurement.

Proposal 10: RAN4 to use SMTC configuration when specifying the requirements for improved measurement during RRC connection setup/resume.

- Reference

[1]	R4-2214348	WF on Further NR Mobility Enhancements,	MediaTek

[2]	R4-2214286	Email discussion summary for [104-e][237] NR_Mob_enh2,	MediaTek