3GPP TSG-RAN WG4 Meeting #104bis-e                              R4-2215460

Electronic Meeting, 10th Oct. – 19th Oct., 2022

Source:	Xiaomi

Title:	Discussion on RRM requirements for FR2 HST

Agenda Item:	6.12.5

Document for:	Discussion

# 1 Introduction

New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2) was agreed in previous RAN meeting [1]. In this paper, we provide our views on RRM requirement for high speed train scenario and give our proposals.

- Specify the RF requirements for intra-band carrier aggregation (CA) scenario, and investigate and specify the RRM requirements for intra-band carrier aggregation (CA) scenario [RAN4]

- Discussion

In Rel-18, intra-band carrier aggregation (CA) scenario is to be supported for FR2 HST. From our consideration, based on the Rel-17 FR2 HST RRM requirement, the potential impact would be expected for the following requirements:

- PSS/SSS detection and measurement for activated/deactivated SCell

- Beam failure detection and candidate beam detection for active SCell

- L1-RSRP measurement for active SCell

In Rel-17, the requirements of PSS/SSS detection and intra-/inter-frequency measurement are enhanced by a configurable scaling factor under the case of non-DRX and DRX cycle ≤ 80ms, and the FR2 HST enhancement is applied to SMTC periodicity ≤ 40ms. From our perspective, similar enhancement on PCell measurement in R17 FR2 HST could be reused for active/deactived SCell measurement in Rel-18 FR2 HST enhancement.

Table 9.2.5.1-11: Time period for PSS/SSS detection when [highSpeedMeasFlagFR2-r17] is configured, (Frequency range FR2) when SMTC period <= 40ms

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80ms | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80ms< DRX cycle≤ 320ms | ceil(1.5 x Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps Note 3 x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if [highSpeedMeasFlagFR2-r17 = set1] or M1 = 18 if [highSpeedMeasFlagFR2-r17 = set2] NOTE 3: Mpss/sss_sync_w/o_gaps =24. |  |

Proposal 1: For PSS/SSS detection and measurement for activated/deactivated SCell, the enhancement defined in R17 FR2 HST can be reused in Rel-18 FR2 HST.

In Rel-17 FR2 HST, the BFD requirement was enhanced by a configurable scaling factor under the case of non-DRX and DRX cycle ≤ 80ms, while CBD requirement has not been enhanced. Since the beam failure recovery on SCell is an optional feature, RAN4 need to decide whether the train roof-mounted high-power devices (FR2 HST UE) support the beam failure recovery on SCell. If yes, RAN4 need to define the enhanced requirement for BFD.

Table 8.5.2.2-3: Evaluation period TEvaluate_BFD_SSB for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(5 P N Note2) TSSB) |
| DRX cycle ≤ 80ms | Max(50, Ceil(7.5 P N Note2) Max(TDRX,TSSB)) |
| Note 1: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. Note 2: scaling factor N=2 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N=6 when highSpeedMeasFlagFR2-r17 is configured to set2.ctor N=6 when [highSpeedMeasFlagFR2-r17] is configured to [set2]. |  |

Proposal 2: RAN4 to decide whether the train roof-mounted high-power devices (FR2 HST UE) support the beam failure recovery on SCell.

The L1-RSRP measurement was enhanced by a configurable scaling factor under the case of non-DRX and DRX cycle ≤ 80ms in Rel-17 for FR2 HST. And the existing enhanced requirement is only applicable to PCell, as single carrier is supported in Rel-17 FR2 HST. However, the L1-RSRP measurement requirements should be applicable to a serving cell, including PCell, PSCell and SCell. Thus, the enhanced requirement of L1-RSRP measurement defined for Rel-17 FR2 HST is applicable to intra-band CA case.

Table 9.5.4.1-3: Measurement period TL1-RSRP_Measurement_Period_SSB configured with [highSpeedMeasFlagFR2-r17] for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N1Note2)*TSSB) |
| DRX cycle ≤ 80ms | max(TReport, ceil(M*P*N1Note2*M2)*max(TDRX,TSSB)) |
| 80ms< DRX ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: N1 = 2 when [highSpeedMeasFlagFR2-r17] = [set1]; N1 = 6 when [highSpeedMeasFlagFR2-r17] = [set2]. Note 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Proposal 3: The L1-RSRP measurement requirement defined for Rel-17 FR2 HST is applicable to intra-band SCell in Rel-18 FR2 HST.

- Conclusion

In this contribution, we discuss the RRM requirements for intra-band CA in FR2 HST and provide our proposals as follows:

Proposal 1: For PSS/SSS detection and measurement for activated/deactivated SCell, the enhancement defined in R17 FR2 HST can be reused in Rel-18 FR2 HST.

Proposal 2: RAN4 to decide whether the train roof-mounted high-power devices (FR2 HST UE) support the beam failure recovery on SCell.

Proposal 3: The L1-RSRP measurement requirement defined for Rel-17 FR2 HST is applicable to intra-band SCell in Rel-18 FR2 HST.

- Reference

[1]	RP-222272,	Revised WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2), Samsung