3GPP TSG-RAN WG4 Meeting # 104bis-e   									R4-2215485

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.17.2.2.2

Source: 	CMCC

Title: 	study the feasibility of and impact on RF requirements on UE aspects

Document for:	Discussion

# 1. Introduction

RAN4 send LS to RAN1 in last meeting with initial thought [1]. It seems we still need to refine the reply. In this contribution, we focus on the discussion of how to refine the LS to RAN1.

# 2. Discussion

In last meeting, LS with RAN4 initial thoughts has been send out to RAN1. For UE side, agreements are listed as below:

In context of UE-UE co-channel inter-subband CLI modelling, RAN4 agree on below candidate requirements specified in TS38.101-1 and TS38.101-2 for FR1 and RF2 respectively.

- TX model can refer to existing UE requirement in TS38.101-1 and TS38.101-2

•	In-band emission as starting point, which defines a per-RB emission across the channel

•	RAN4 is still studying whether ACLR may also apply in certain restricted configurations

- RX model can refer to existing UE requirement in TS38.101-1 and TS38.101-2

  - Maximum input power as threshold based on above specification

  - In-channel selectivity requirements for the UE are not defined, and RAN4 is still investigating the feasibility of providing an indicative co-channel Rx modelling in the presence of interference.

For Rx part, if we assume there is no selectivity requirement, this means all the Tx power will fall into victim UE without any selectivity. At the victim part, total interference includes two parts, one from Tx unwanted leakage that fall into Rx sub-band, and the other part from Tx wanted single that all fall into Rx sub-band if we assume no ICS requirements. since Tx wanted signal power is much larger than unwanted part, wanted signal at Tx aggressor UE will determine final interference. To be honest, we even don’t need to consider IBE requirements.

Observation 1: if no ICS requirement is sent to RAN1, interference is very severe and all Tx power from aggressor will fall into Rx side without selectivity and RAN1 even don’t need to model IBE interference which is negligible.

For example, if we assume UE transmit with max 23dBm/20MHz. max input power for legacy UE is -25dBm. so at least 48 dB isolation is required between Tx aggressor and Rx victim UE. In some cases when UE are near to each other, 48dB isolation can’t be met. If we assume UE could accept REFSENSE+1dB, I=N-6dB, I=-111dBm/MHz with 9dB NF assumption. At least 121dB isolation is required between Tx aggressor and Rx victim UE. This means it’s very hard to deploy aggressor UE and victim UE in the same cell.

Observation 2: if there is no ICS requirements, it seems it’s scarcely possible to configure aggressor UE and victim UE in the same cell if victim UE only allow 1dB REFSENSE degradation like gNB.

It’s better to study how to avoid such interference at UE side. Following lists two options to avoid UE-UE inter-sub band interference.

- Option 1: RAN4 collect the minimum requirements from UE vendors that could reflect typical legacy UE’s Rx selectivity for inter-sub band case and reply it to RAN1 for their simulation and scheme design.

- Option 2: RAN4 send the LS to RAN1 and ask RAN1 design schemes to avoid UE-UE inter-sub band interference with 0dB Rx inter-sub band selectivity requirements

Observation 3: above two options are listed to avoid UE-UE inter-sub band interference based on the assumption that legacy UE could also work in SBFD network.

The commercial UE must have certain capability to suppress Rx interference, it’s better to find out the common performance at RAN4 rather than just send this interference to RAN1 without reflecting actual interference issue. If final interference still exists even with typical UE performance, RAN1 schemes are suggested rather than let RAN1 resolve such sever interference with 0dB ICS.

Proposal 1: it’s suggested to provide an typical co-channel Rx modelling to RAN1 other than assuming 0dBi ICS.

# 3. Conclusions

In this contribution, LS to RAN1 is discussed with following observations and proposals:

Observation 1: if no ICS requirement is sent to RAN1, interference is very severe and all Tx power from aggressor will fall into Rx side without selectivity and RAN1 even don’t need to model IBE interference which is negligible.

Observation 2: if there is no ICS requirement, it seems it’s scarcely possible to configure aggressor UE and victim UE in the same cell if victim UE only allow 1dB REFSENSE degradation like gNB.

Observation 3: above two options are listed to avoid UE-UE inter-sub band interference based on the assumption that legacy UE could also work in SBFD network.

Proposal 1: it’s suggested to provide an typical co-channel Rx modelling to RAN1 other than assuming 0dBi ICS.

# 4. Reference

[1] R1-2205543, LS on interference modelling for duplex evolution, Samsung.