3GPP TSG-RAN WG4 Meeting # 104bis-e                                                   R4-2215492

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.6.4.1

Source: 	CMCC

Title: 			      NCD-SSB configurations and test cases

Document for:	Discussion

- Introduction

In last meeting, NCD-SSB configurations were discussed [1]. In this contribution, we discuss the NCD-SSB configurations and test cases for RedCap, and provide our views.

- Discussion

Issue 1: Whether to define test case for NCD-SSB?

UE-specific RRC configured DL BWP with NCD-SSB is mandatory supported by RedCap UE. And NCD-SSB is one of the most important functions introduced for RedCap. Hence, we support to define test case for NCD-SSB for RedCap.

Proposal 1: Define RedCap test cases for NCD-SSB.

Issue 2: Which test case(s) should be used for NCD-SSB test in Handover if issue 1 is yes?

For the handover test cases, in order to reduce the overall test overload, different NCD-SSB and CD-SSB can be considered for different HO test cases. For example:

| Test Index | Test |
| --- | --- |
| 1 | Intra-frequency handover from FR1 CD-SSB to FR1 CD-SSB; known target cell for UE (1Rx, 2Rx) |
| 2 | Intra-frequency handover from FR1 NCD-SSB to FR1 NCD-SSB; unknown target cell for UE (1Rx, 2Rx) |
| 3 | Inter-frequency handover from FR1 CD-SSB to FR1 NCD-SSB; unknown target cell for UE (1Rx, 2Rx) |

Proposal 2: Define handover test cases for NCD-SSB as follows:

| Test Index | Test |
| --- | --- |
| 1 | Intra-frequency handover from FR1 CD-SSB to FR1 CD-SSB; known target cell for UE (1Rx, 2Rx) |
| 2 | Intra-frequency handover from FR1 NCD-SSB to FR1 NCD-SSB; unknown target cell for UE (1Rx, 2Rx) |
| 3 | Inter-frequency handover from FR1 CD-SSB to FR1 NCD-SSB; unknown target cell for UE (1Rx, 2Rx) |

Issue 3: Which test case(s) should be used for NCD-SSB test in intra-frequency measurement if issue 1 is yes?

For intra-frequency measurement, test cases can be split for NCD-SSB and CD-SSB with DRX and non-DRX. For DRX test cases, CD-SSB is configured. For non-DRX test cases, NCD-SSB is configured.

Proposal 3: Define intra-frequency measurement test cases for NCD-SSB as follows:

| Test Index | Test |
| --- | --- |
| 1 | SA event triggered reporting tests without gap under non-DRX (1Rx, 2Rx) |
| 2 | SA event triggered reporting tests with per-UE gaps under non-DRX (1Rx, 2Rx) |
| 3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading (1Rx, 2Rx) |
| 4 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (1Rx, 2Rx) |

Issue 4: Other test cases for NCD-SSB

In addition to handover and measurement, active BWP switch test cases should also be introduced for NCD-SSB. BWP switching between BWP-1 with CD-SSB and BWP-2 with NCD-SSB needs to be considered.

Proposal 4: Define active BWP switch test cases between BWP-1 with CD-SSB and BWP-2 with NCD-SSB as follows:

| Test Index | Test |
| --- | --- |
| 1 | DCI-based and Timer-based Active BWP Switch: NR FR1 DL active BWP switch of Cell with non-DRX in SA (1Rx, 2Rx) |
| 2 | DCI-based and Timer-based Active BWP Switch: NR FR1 DL active BWP switch with non-DRX in SA (1Rx, 2Rx) |

Issue 5: What’s the NCD-SSB configuration can be introduced if issue 1 is yes?

Candidate options:

- Option 1: NCD-SSB periodicity 80ms, NCD-SSB offset 5ms

- Option 2: NCD-SSB periodicity 40ms, NCD-SSB offset 20ms

- Option 3: Others

Considering the overload of transmitting NCD-SSB, the periodicity of NCD-SSB should be larger than CD-SSB. We prefer to use option 1 for NCD-SSB configuration.

Proposal 5: NCD-SSB periodicity 80ms, NCD-SSB offset 5ms.

- Conclusions

In this contribution, we discuss the NCD-SSB configurations and test cases for RedCap. Our proposals are:

Proposal 1: Define RedCap test cases for NCD-SSB.

Proposal 2: Define handover test cases for NCD-SSB as follows:

| Test Index | Test |
| --- | --- |
| 1 | Intra-frequency handover from FR1 CD-SSB to FR1 CD-SSB; known target cell for UE (1Rx, 2Rx) |
| 2 | Intra-frequency handover from FR1 NCD-SSB to FR1 NCD-SSB; unknown target cell for UE (1Rx, 2Rx) |
| 3 | Inter-frequency handover from FR1 CD-SSB to FR1 NCD-SSB; unknown target cell for UE (1Rx, 2Rx) |

Proposal 3: Define intra-frequency measurement test cases for NCD-SSB as follows:

| Test Index | Test |
| --- | --- |
| 1 | SA event triggered reporting tests without gap under non-DRX (1Rx, 2Rx) |
| 2 | SA event triggered reporting tests with per-UE gaps under non-DRX (1Rx, 2Rx) |
| 3 | SA event triggered reporting tests without gap under non-DRX with SSB index reading (1Rx, 2Rx) |
| 4 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (1Rx, 2Rx) |

Proposal 4: Define active BWP switch test cases between BWP-1 with CD-SSB and BWP-2 with NCD-SSB as follows:

| Test Index | Test |
| --- | --- |
| 1 | DCI-based and Timer-based Active BWP Switch: NR FR1 DL active BWP switch of Cell with non-DRX in SA (1Rx, 2Rx) |
| 2 | DCI-based and Timer-based Active BWP Switch: NR FR1 DL active BWP switch with non-DRX in SA (1Rx, 2Rx) |

Proposal 5: NCD-SSB periodicity 80ms, NCD-SSB offset 5ms.

- Reference

[1] R4-2214485, WF on RedCap RRM requirements, Ericsson.

[2] R4-2213411, Test case list for RedCap RRM performance part, Ericsson.

[2] R4-2210595, Test case list for RedCap RRM performance part, Ericsson.

[3] R4-2211050, Workplan to RedCap RRM performance requirements, Ericsson.