3GPP TSG-RAN WG4 Meeting # 104-bis-e      	R4-2215505

Electronic Meeting, Oct 10 – Oct 19, 2022

Agenda Item:	6.13.5

Source: 	CMCC

Title: 			 Discussion on RRM requirements for ATG

Document for:	Discussion

- Introduction

In RAN4#104-e meeting, the discussion for ATG RRM had been initiated, some RRM requirements have been down-scoped for further discussion considering ATG feature. The WF has been approved in [1]. In this contribution, we continue the discussion about RRM impaction on ATG.

- General

ISD

After the discussion at the last meeting, we found that the study on RRM core requirements highly depends on ISD and cell range assumption, and we have agreed to wait for the input from RF session. However, to avoid repeated discussions without any progress, we propose to first consider the basic ISD assumption 100-200km for RRM requirements study. If RF session come to other conclusions beyond the ISD 100-200km assumption, we can come back to have more study on ISD related requirements.

Proposal 1: First study ATG RRM requirements with ISD assumption of 100-200km. If RF session come to other conclusions beyond the assumption range, then come back to have more study on ISD related requirements.

Duplex mode, SCS and Frame structure configuration

For R18 ATG, both FDD and TDD duplex mode should be considered. For FDD, 15kHz SCS is the most typical case, for TDD, it is more likely to operate in 30kHz SCS. Therefore, we propose to consider FDD 15kHz SCS and TDD 30kHz SCS as the basic assumption for ATG RRM study.

Proposal 2: Consider FDD 15kHz SCS and TDD 30kHz SCS as the basic assumption for ATG RRM study.

As for the frame structure of TDD 30kHz SCS, two impacts should be considered.

First, the large ISD will lead to longer propagation delay, such delay should be considered in guard period design, to prevent the cross-link interference between ATG gNBs, ATG gNB and ATG UE.

Second, in ATG application scenario, typically, there are more DL data transmission than UL data transmission. Therefore, the resource allocation between DL and UL should be well designed to match the ATG application demand.

With above two considerations, for TDD 30kHz SCS, we propose a 20ms period frame structure for ATG, which is illustrated in Figure 1. 4 flexible subframes can hold the large GP caused by ATG propagation delay. 30 downlink subframes can provide enough DL transmission resources for ATG application.

| n-1 |  |  |  |  |  |  |  |  |  | n |  |  |  |  |  |  |  |  |  | n+1 |  |  |  |  |  |  |  |  |  | n+2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| D | D | S | S | S | S | U | U | U | U | U | U | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D | D |

Figure 1. TDD frame structure for ATG in 30kHz SCS

Proposal 3: 20ms period TDD frame structure could be configured for ATG TDD 30kHz SCS, which the TDD pattern is 30D4S6U.

UE assistance information

- RAN4 further study ATG UE assistance information

  - such as altitude, location, propagation delay difference, flight path, speed, velocity etc., or change in any of these parameters.

First of all, it should be noted that ATG WI only involves RAN4 group. Hence, new UAI reporting method is not supported, only current reporting can be introduced to ATG.

Proposal 4: ATG WI should avoid to introduce new UAI reporting method.

The UAI examples proposed by companies can basically divided into two categories：

- Cat1 (location related information): including altitude, speed, location, velocity, propagation delay difference.

- Cat2 (flight path related information): including flight path.

Firstly, we give our analysis on whether the example UAI could be obtained by existing approach. For Cat1, network can acquire such information through immediate MDT by configuring obtainCommonLocation-r16 and propDelayDiffReportConfig signalling. For Cat2, flight path and flight path changing reporting have not been introduced in NR. In our view, it is more feasible and reasonable for OAM to acquire flight path information by implementation than for ATG UE to report the flight path information to gNB.

Observation 1: The ATG UAI reporting including altitude, location, propagation delay difference, flight path, speed, velocity reporting could be supported by existing approach in current NR spec.

Observation 2: The ATG UAI reporting including flight path reporting couldn’t be supported by existing approach in current NR spec.

Proposal 5: Don’t introduce flight path as ATG UAI.

Secondly, we further analyze whether the example UAI in Cat1 is necessary for ATG. According to the discussion in R17 NTN WI, the location related information is tended to support or optimize timing adjustment procedure, mobility procedure and measurement procedure. However, even in NTN, the specific location information reporting like location, altitude, speed, velocity is not supported. In ATG, these information keeps time-varying due to high-speed feature of ATG UE. Therefore, we don’t see the feasibility and necessity of introducing ATG example UAI reporting including altitude, location, speed, velocity reporting.

Regarding propagation delay difference reporting, in NTN, due to potential large propagation delay difference between NGSOs, propagation delay difference has been introduced to be reported in order to support network to configure the SMTC and/or MG for inter satellite measurement. However, whether such UAI is needed to be introduced in ATG depends on the max ISD. If the max ISD is smaller than 300km (the propagation delay difference smaller than 1ms), we think network can configure the suitable SMTC and MG without UE assistance information of propagation delay difference. With the basic assumption of 100-200km ISD, we think there is no necessity of introducing ATG UAI of propagation delay difference.

Proposal 6: No need to introduce altitude, location, speed, velocity as ATG UAI.

Proposal 7: There is no necessity of introducing ATG UAI of propagation delay difference with the ISD assumption of 100-200km. The conclusion can be revisited after RF session draw the final conclusion of ISD.

- Mobility

  - Mobility in RRC_IDLE/INACTIVE

Cell re-selection measurement requirements

- Option 1: Reusing legacy R15 requirements of intra-frequency and inter-frequency measurement as the starting point, further check whether to use HST requirements for ATG until a typically ATG network deployment scenario such as ISD is concluded in RF group.

  - Option 1-1: The A2G UE is allowed to not measure on the neighbour cells based on the coverage information of the serving cell e.g. if serving cell RSRP is above threshold.

    - For cell reselection and handover, the A2G UE should resume the neighbour cell measurement in normal manner without any relaxation if there is any unpredictable change in flight path or sudden drop in aircraft height due to any critical or emergency situation.

    - For cell reselection and handover, UE can determine the sudden change in the flight path autonomously (e.g. internally from flight data) or based on assistance information from the ground base station. Details are FFS

For the RRC_IDLE/INACTIVE state cell re-selection requirement, we should guarantee that the time span between S criteria is met and serving cell stop serving is larger than the Tdetect.

Considering two deployment methods, we give a relationship between cell reselection requirement and the minimum ISD in Table 1 and Table 2, provided that the time span criteria can be fulfilled under the assumption of this minimum ISD. Here we consider S criteria will be met when the distance between the ATG UE and the cell center is larger than half of cell radius, and the UE speed is 1200km/h.

Two deployment methods are illustrated in Figure 2.

| Deployment Method 1 | Deployment Method 2 |
| --- | --- |

Figure 2. Two potential deployment methods in ATG network

Table 1. The minimum ISD when using legacy R15 intra-frequency and inter-frequency measurement requirements

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tdetect,NR_Inter [s] (number of DRX cycles) | Minimum cell radius (km) | Minimum ISD (km) Deployment 1 | Minimum ISD (km) Deployment 2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 |  |  |  |  |  |
| 0.32 | 1 | 11.52 x N1 x M2 (36 x N1 x M2) | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 11.52 | 34.56 | 23.04 |
| 0.64 |  | 17.92 x N1 (28 x N1) | 17.92x N1 (28 x N1) | 11.94 | 35.84 | 23.88 |
| 1.28 |  | 32 x N1 (25 x N1) | 32 x N1 (25 x N1) | 21.33 | 64 | 42.66 |
| 2.56 |  | 58.88 x N1 (23 x N1) | 58.88 x N1 (23 x N1) | 39.25 | 117.76 | 78.5 |

Table 2. The minimum ISD when using HST intra-frequency and inter-frequency measurement requirements

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tdetect,NR_Inter_HST [s] (number of DRX cycles) | Minimum cell radius (km) | Minimum ISD (km) Deployment 1 | Minimum ISD (km) Deployment 2 |
| --- | --- | --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2) | [3.2 x M2 (10 x M2)] Note 1 | 3.2 | 9.6 (M2=1.5) | 6.4 |
| 0.64 | 5.12 (8) | [6.4 (10)] | 4.26 | 12.8 | 8.52 |
| 1.28 | 8.96 (7) | [10.24 (8)] | 6.83 | 20.48 | 13.66 |
| 2.56 | 58.88 (23) | 58.88 (23) | 39.25 | 117.76 | 78.5 |

Based on the calculation, for Deployment method 1 and DRX cycle length 2.56s, when ISD is smaller than 117.76km, both legacy R15 requirements and HST requirements can’t support UE to complete cell-reselection. Therefore, considering the basic ISD assumption 100-200km, an applicability rule for DRX cycle will be needed. Similar conclusion can be adopted as R17 NTN, that whether and what DRX cycle length to configure is up to NW, but UE is not required to fulfil the requirements for 2.56s DRX cycle length for ATG deployment method 1.

Observation 3: For ATG deployment method 1, when ISD is smaller than 117.76km, both legacy R15 requirements and HST requirements can’t support UE to complete cell-reselection.

Proposal 8: If ATG deployment method 1 is adopted, whether and what DRX cycle length to configure is up to NW. As long as the ISD is smaller than 117.76km, UE is not required to fulfill the cell re-selection requirements for 2.56s DRX cycle length.

Generally, if the typical ISD assumption is larger than 78.5km, we think the legacy R15 cell-reselection requirement can be reused for ATG regardless of deployment method. While if the typical ISD assumption is smaller than 64km, then the HST cell-reselection requirements should be reused for ATG. Currently, we consider the basic ISD assumption of 100-200km, therefore, we propose to reuse legacy R15 cell-reselection requirement. The conclusion can be revisited after receiving final conclusion about ISD assumption and deployment approach from RF session.

Observation 4: If the typical ISD assumption is larger than 78.5km, we think the legacy R15 cell-reselection requirement can be reused for ATG. While if the typical ISD assumption is smaller than 64km, then the HST cell-reselection requirements should be reused for ATG.

Proposal 9: With the basic ISD assumption of 100-200km, the legacy R15 cell-reselection requirement could be reused for ATG. The conclusion can be revisited after receiving final conclusion about ISD assumption and deployment approach from RF session.

In addition, an extra procedure in Option 1-1 was brought up in last meeting. For the main bullet of Option 1-1, “The A2G UE is allowed to not measure on the neighbour cells based on the coverage information of the serving cell e.g. if serving cell RSRP is above threshold.” It equals to that the ATG UE is allowed to not perform intra-frequency measurement, higher, equal and lower priority inter-frequency measurement when serving cell RSRP is above the threshold. The main difference from current design is that the higher priority inter-frequency measurement can be omitted. However, the higher priority inter-frequency configuration could also be utilized in ATG network, it is important to guarantee that UE could perform corresponding measurement for re-selecting to higher priority carrier. Besides, we think at least in this release, ATG UE don’t have power saving demand of measurement relaxation. Therefore, we don’t see enough gain of this neighbour cell measurement skipping design.

Proposal 10: Reuse the legacy cell-reselection rule and procedure.

Minimization of Drive Tests

It is very beneficial to introduce MDT feature in ATG, it could help network to optimize the deployment, which is more complex compared with ground-based network. In last meeting, RAN4 didn’t identify the MDT RRM impact on ATG. However, after re-checking, we found that the MDT measurement requirements are related to cell-reselection measurement requirements. Therefore, we think the conclusion can be delayed until RAN4 decide how to define ATG cell-reselection measurement requirements.

Proposal 11: The conclusion on MDT measurement requirements can be delayed until RAN4 decide how to define ATG cell-reselection measurement requirements.

Small Data Transmissions (SDT)

- Option 1: SDT requirements are defined for A2G. Details are FSS (Ericsson)

- Option 2: RAN4 is not going to define ATG specific requirements (HW, CMCC)

Since SDT is depends on UE capability and network configuration, we are fine with introducing SDT to ATG network as long as UE and network could capable with it. The main work is to identify that whether ATG specific requirements are needed or not for SDT requirements.

For CG-SDT requirements, we think the TA validation requirements, the scheduling restriction and applicability conditions can be reused.

For RA-SDT requirements, the corresponding RA requirements will apply. Therefore, we can just focus on RA requirements, no need to have more discussion here.

Proposal 12: For SDT RRM requirements, the legacy requirements can be reused, no need to define ATG specific requirements.

  - Mobility in RRC_CONNECTED

Issue 2-2-1-1: NR Handover

- Option 1: Both intra-frequency HO and inter-frequency HO need to be defined.

  - Option 1-1: Reusing legacy intra-frequency HO and inter-frequency HO requirements as the starting point, FFS other details and potential revisions for ATG.

  - Option 1-2: The A2G UE is allowed to not measure on the neighbour cells based on the coverage information of the serving cell e.g. if serving cell RSRP is above threshold.

    - For cell reselection and handover, the A2G UE should resume the neighbour cell measurement in normal manner without any relaxation if there is any unpredictable change in flight path or sudden drop in aircraft height due to any critical or emergency situation.

    - For cell reselection and handover, UE can determine the sudden change in the flight path autonomously (e.g. internally from flight data) or based on assistance information from the ground base station. Details are FFS

In NR FR1 to FR1 handover requirement, Dhandover = RRC procedure delay + Tsearch + TIU + Tprocessing + T∆ + Tmargin ms. Based on our understanding, all the components can reuse the legacy definition and requirement. As for Option 1-2, we gave our analysis above under cell re-selection requirement, we don’t see enough gain of this neighbour cell measurement skipping rule. Therefore, for NR ATG handover, reuse legacy NR HO requirements.

Proposal 13: For NR ATG handover, reuse legacy NR HO requirements.

Issue 2-2-1-4: NR Conditional Handover

- FFS which kind of CHO will be introduced

- FFS whether ATG specific CHO requirements are needed.

We support to introduce legacy CHO in ATG network. First, under ATG large propagation delay scenario, CHO could improve network performance by decreasing handover failure. Second, CHO configuration could be optimized in ATG scenario. For example, network could configure suitable candidate target cell(s) according to some assistance information, such as flight path information from OAM and so on, the signalling burden at network side and measurement workload at UE side can be both reduced. As for the CHO delay requirement in RAN4 RRM scope, we think the legacy R16 CHO requirement can be reused for ATG.

Proposal 14: Introduce legacy CHO for ATG, the legacy R16 CHO delay requirements can be reused.

In addition, we think the location-based CHO should be introduced in ATG network. On one hand, as depicted in Figure 3, the PL variation around cell edge is not obvious, when the horizontal distance between UE and cell center is larger than 90km, the PL variation will smaller than 1dB every 10km, with the assumption of ATG UE height 10km. On the other hand, ATG UE could support location-based CHO since RAN4 have already agreed that ATG UE is capable of GNSS in last meeting.

Figure 3. The passloss in ATG scenario with 10km UE height

We think the logic and signalling from R17 NTN could be reused, network should configure condEventD1 and an additional legacy RRM conditional measurement event to UE. As for the Location-based CHO delay requirement, the NTN CHO requirement (Section 6.1C.2.2 in 38.133) could not be reused to ATG, since the measurement in ATG don’t need to considering the NTN specific scaling factor.

Proposal 15: Introduce location-based CHO for ATG, the logic and signalling from R17 NTN could be reused, while the location-based CHO delay requirements should be revisited in ATG scenario.

Issue 2-2-2-1: SA: RRC Re-establishment

- Reuse the principle from the legacy RRC Re-establishment delay requirements as baseline for ATG

  - Further discuss whether the ATG specific RRC Re-establishment requirement is needed.

Issue 2-2-2-3: SA: RRC Connection Release with Redirection

- Reuse the principle from the legacy RRC Connection Release with Redirection requirements as baseline for ATG

  - Further discuss whether the ATG specific RRC Connection Release with Redirection requirement is needed

In our view, if UE could mitigate different doppler shift from intra/inter-frequency cells, then the legacy RRC Re-establishment delay could be reused, or else, the Tidentify_intra_NR and Tidentify_inter_NR,i in RRC Re-establishment delay requirement and Tidentify-NR in RRC Connection Release with Redirection delay requirement should be scaled according to the UE capability about Doppler handling. However, based on our knowledge, the ATG CPE is more powerful than smart phone, the max Doppler about ±1ppm can be handled by ATG CPE, especially when BS could sends its PVT information to UE. Therefore, we support to reuse the legacy RRC Re-establishment delay requirement and legacy RRC Connection Release with Redirection delay requirement.

Proposal 16: For NR ATG RRC Re-establishment and RRC Connection Release with Redirection, reuse the legacy NR delay requirements.

Issue 2-2-2-2: Random access

- Reuse the principle from the legacy random access requirements as baseline for ATG

  - Further discuss the ATG specific impaction and details

  - Further discuss whether to define requirements for 2-step RA for A2G, and whether ATG specific impaction should be involved.

Based on our analysis on Timing adjustment procedure, the UE specific TA pre-compensation is not necessary for ATG. Therefore, no additional RA procedure will be added in ATG network. In other words, for both 4-step and 2-step RA, the legacy R15/R16 requirements could be reused for ATG.

Proposal 17: For both 4-step and 2-step RA, reuse the legacy requirements for ATG.

- Timing and frequency adjustment

  - General issues

Issue 3-1-2: The mechanism of Koffset and Kmac

- Option 1: The mechanism of Koffset and Kmac for NTN system should be used for ATG network. (CATT, Ericsson (if NTN as base), LGE)

- Option 2: no need to introduce the mechanism of Koffset and Kmac for ATG system (HW, CMCC, ZTE)

- Option 3: FFS (Apple)

In NTN, Kmac is the time between reference point and BS, it is on the feeder link. However, there is no feeder link in ATG, so Kmac shall not be considered in ATG.

As for Koffset, in 15kHz SCS scenario, if the cell radius larger than 150km, where the RTT will larger than 1ms=1slot, Koffset should be configured to compensate the RTT between UE and BS. In 30kHz SCS scenario, if the cell radius larger than 75km, where the RTT will larger than 0.5ms=1slot, Koffset should be configured to compensate the RTT between UE and BS.

As we proposed in Proposal 1, ISD 100km-200km is taken as the basic assumption, the cell radius may exceed 75km, therefore, Koffset mechanism shall be introduced in ATG network. The Koffset value is configured by network through Kcell,offset and KUE,offset.

Proposal 18: Introduce the mechanism of Koffset in ATG system. The conclusion can be revisited after RF session draw the final conclusion about ISD and so on.

  - Timing and frequency pre-compensation by UE

Issue 3-2-1: Whether to introduce UE based Timing pre-compensation

- Option 1: The ATG UE should do the compensation of transmit frequency based on relative moving velocity and distance between UE and gNB. (CATT, ZTE, Ericsson, LGE)

- Option 2: Further study whether to introduce the UE based UL timing pre-compensation based on necessity and performance gain. (CMCC, Apple, HW, Ericsson, ZTE)

  - Option 2-1: Use the current timing adjustment procedure as the baseline. (CMCC, HW)

Refer to the current procedure of transmission timing adjustment, which is sub-clause 4.2 in TS 38.213. In case of Random access, the TAC can cover 2ms RTT when SCS is 15kHz and 1ms RTT when SCS is 30kHz. Considering of ISD 100-200km, the largest cell radius is about 100km, the corresponding maximum RTT is about 0.67ms. Therefore, the max RTT can be easily covered with current timing advance command.

In RRC-Connected state, one TAC MAC CE could compensate the timing drift of 0.016ms. With the assumption of max UE speed 1200km/h, if network can update the TA within 7.26s for 15kHz SCS and 3.63s for 30kHz SCS, the timing drift caused by UE movement can be addressed by one TAC.

With above analysis, we think the current procedure of transmission timing adjustment could handle timing compensation in ATG network, therefore, there is no need to introduce the UE based UL timing pre-compensation.

Proposal 19: There is no need to introduce the UE based UL timing pre-compensation under the ISD assumption of 100-200km. The conclusion could be revisited after receiving the input about ISD and cell radius from RF session.

Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation

- Option 1: The ATG UE should do the compensation of timing based on relative moving velocity and distance between UE and gNB. (CATT, Ericsson, LGE, CMCC)

- Option 2: Further study whether to introduce the UE based UL frequency pre-compensation based on necessity and performance gain. (CMCC, HW, Ericsson, Apple, ZTE)

  - Option 2-1: Use the current frequency pre-compensation procedure as the baseline. (HW)

It is beneficial to introduce UE based frequency pre-compensation method in ATG. The pre-compensation can be done provided that the UE velocity and UE location can be acquired based on GNSS, and network send the gNB location to UE by PVT information. However, whether UE perform frequency pre-compensation or not seems have no RRM impact on RRM requirements. We are open to have more discussion.

  - Timing requirements

Issue 3-3-1-1: Initial transmit timing requirements Te

- Option 1: Need defined RRM requirements for ATG UE. (CATT, Apple, CMCC, HW, Ericsson)

  - FFS if UE specific TA shall be considered in the Te requirement design, like in NTN (Apple, CMCC, HW)

  - Introduce UE specific TA in the Te requirement design. (CATT)

Issue 3-3-3: Timing advance

- FFS on the necessity of considering the open loop TA (UE specific TA if needed) and close loop (TAC based adjustment) for the TA adjustment requirement, like in NTN. (CMCC, Apple, Ericsson, LGE, ZTE)

Based on our proposal 13 that there is no need to introduce the UE based UL timing pre-compensation, the initial transmit timing requirement Te and timing advance adjustment requirement can reuse the legacy R15 TN requirement. However, the koffset mechanism may need to be introduced in timing advance adjustment delay requirement. To be mentioned, the issue should be reviewed after receiving the conclusion about ISD and cell radius from RF session.

Proposal 20: Reuse the legacy R15 TN requirement for initial transmit timing requirement Te with the assumption of ISD 100-200km. The issue should be reviewed after receiving the conclusion about ISD and cell radius from RF session.

Proposal 21: The koffset mechanism may need to be introduced in timing advance adjustment delay requirement.

Issue 3-3-1-2: Gradual timing adjustment

- Tp and Tq shall be updated for ATG UE

  - FFS if TN or NTN rel-17 is used as baseline

  - FFS how to define the exact value

First of all, without introducing UE based timing pre-compensation, the TN gradual timing adjustment procedure can be used as the baseline.

Regarding the exact Tp and Tq value, Tq should include time drift due to frequency error per 200ms, time drift due to UE movement per 200ms, and 1.5Ts DigRF error. We provide the calculation in Table 3 with the assumption of max UE speed 1200km/h.

Table 3. Time drift calculation

| Factors | Time drift |
| --- | --- |
| Frequency error | [0.1]PPM |
| Time drift due to frequency error | [20]ns |
| speed of UE movement | 1200km/h aircraft |
| Time drift due to UE movement per [200]ms | 222.2ns |
| Sum of time drift above | 7.45Ts |
| DigRF error | 1.5Ts |

Considering of the sample period of UL transmission is 4Ts in 15kHz and 30kHz SCS, the Tq used by UE should be (4*7.45/4+1.5)Ts, which is 9.5Ts.

Proposal 22: For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS.

Issue 3-3-4: Cell phase synchronization accuracy

- Cell phase synchronization accuracy will be defined for ATG, the legacy TN requirement can be the baseline

  - FFS whether to tighten the requirements or not

The cell phase synchronization accuracy requirement has always been reused since 2G network, we don’t see the necessity of tighten the requirement in ATG scenario, we prefer to reuse 3µs requirement for ATG.

Proposal 23: Reuse Cell phase synchronization accuracy 3µs requirement for ATG.

Issue 3-3-5: deriveSSB-IndexFromCell tolerance

- Option 1: Need defined RRM requirements for ATG UE. (Apple, CMCC, ZTE)

  - Option 1-1: The time misalignment tolerance for ‘deriveSSB-IndexFromCell= true’ shall be revisited due to the extreme large radius of ATG cell. (Apple, LGE)

  - Option 1-2: The legacy TN requirement can be reused (CMCC)

  - Option 1-3: The propagation delay different may impact the tolerance. FFS details. (HW, Ericsson)

- Option 2: Not applicable for R18 ATG (CATT)

Issue 3-3-6: deriveSSB-IndexFromCell-inter tolerance

- Option 1: This section was introduced in MG enhancement WI which is not needed for ATG UE (Apple, CATT)

- Option 2: No ATG impact (HW, CMCC)

- Option 3: Need for ATG, NCSG (deriveSSB-IndexFromCell-inter) has benefit for A2G system since the throughput is an important KPI for ATG (Ericsson, CMCC)

Based on our understanding, the tolerance requirement can be used for every scenario, if UE could satisfy the tolerance requirement, then UE could derive neighbour cell SSB-Index according to serving cell SSB-Index. However, in ATG scenario, because of the large propagation delay between cells, the tolerance requirement couldn’t be satisfied in most cases. Therefore, on one hand, do not update tolerance requirements for ATG, on the other hand, whether to configure ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ can up to network, considering the real deployment scenario, such as ISD.

Proposal 24: Do not update ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ tolerance requirements. It is up to network to decide whether to configure ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ or not, based the real deployment scenario, such as ISD.

- Signalling characteristics

Issue 4-1-1: Radio Link Monitoring

- Reuse the principle from the legacy RLM requirements as baseline for ATG

  - Further discuss whether ATG specific impaction should be involved.

Issue 4-1-2: Link Recovery Procedure

- Reuse the principle from the legacy Link Recovery Procedure requirements as baseline for ATG

  - Further discuss whether ATG specific impaction should be involved.

With the high speed of terminal, the validity time of RLF and BFD indication are limited, frequent RLF and BFD indication have no performance gain to network. Therefore, we propose to reuse the legacy RLM Out-Of-Sync evaluation period and legacy BFD evaluation period.

As for RLM In-sync evaluation period and Candidate beam detection evaluation period, we are open to have further discuss about whether the requirement should be tightened or not.

Proposal 25: Reuse the legacy RLM OOS evaluation period and legacy BFD evaluation period.

Proposal 26: Further discuss whether the requirement for RLM In-sync evaluation period and Candidate beam detection evaluation period should be tightened or not in ATG network.

Issue 4-1-3: Active BWP switching delay

- Reuse the principle from the legacy Active BWP switching delay requirements as baseline for ATG

  - Further discuss whether ATG specific impaction should be involved.

Issue 4-1-4: Active TCI state switching delay

- Reuse the principle from the legacy Active TCI state switching delay requirements as baseline for ATG

  - Further discuss whether ATG specific impaction should be involved.

Issue 4-1-8: Active downlink TCI state switching delay for unified TCI

- Further discuss whether ATG specific impaction is need be involved.

Issue 4-1-9: Active uplink TCI state switching delay for unified TCI

- Further discuss whether ATG specific impaction is need be involved.

Issue 4-1-7: Pathloss reference signal switching delay

- Reuse the principle from the legacy Pathloss reference signal switching delay requirements as baseline for ATG

  - Further discuss whether ATG specific impaction should be involved.

After checking, we haven’t identified the ATG impaction on above requirements, we propose to reuse the corresponding legacy requirement for ATG.

Proposal 27: Reuse the legacy ‘Active BWP switching delay’, ‘Active TCI state switching delay’, ’Active downlink TCI state switching delay for unified TCI’, ’Active uplink TCI state switching delay for unified TCI’ and ‘Pathloss reference signal switching delay’ requirement for ATG.

Issue 4-1-5: Uplink spatial relation switch delay

- Reuse the principle from the legacy Active spatial relation switch delay requirements as baseline for ATG

  - Further discuss whether ATG specific impaction should be involved.

After double check, this requirement is only for FR2, therefore, it will not be applicable for R18 ATG.

Proposal 28: Uplink spatial relation switch delay requirement will not be applicable for R18 ATG.

Issue 4-1-11: Pre-configured measurement gap activation/deactivation delay

- Option 1: Need defined RRM requirements for ATG UE (CATT, Ericsson)

- Option 2: GAP enhancement contents are not considered for ATG UE in Rel-18. (Apple, CMCC, HW, LGE, ZTE)

In ATG network, it is more common that CPE use full bandwidth to meet the high demand of data transmission, the gain of this feature will be limited. Therefore, we think the pre-configured measurement gap activation/deactivation could not be introduced in R18 ATG.

Proposal 29: No need to introduce the pre-configured measurement gap for R18 ATG.

- Measurement

Issue 5-1-1: General measurement requirement

- FFS the MG design

  - Option 1: Only FR1 MG is considered in ATG network

- FFS the UE measurement capability/scaling

For measurement gap, since there is only FR1 scenario in R18 ATG network, it is reasonable to only consider FR1 MG. However, it doesn’t need to be explicitly written in spec, since the MG configuration is up to network.

For measurement capability of Monitoring of multiple layers using gap, ATG UE could reuse the legacy requirement in 9.1.3.1a, which is Nfreq, SA = Nfreq, SA, NR + Nfreq, SA, E-UTRA+ Nfreq, SA, UTRA. Although Nfreq, SA, E-UTRA and Nfreq, SA, UTRA will always be 0 for ATG since network will not configure such carriers for UE.

Proposal 30: Reuse the legacy MG requriements and legacy measurement capability of Monitoring of multiple layers using gap for R18 ATG.

For measurement capability of maximum allowed layers for multiple monitoring, at the current stage, we prefer to reuse legacy intra and inter frequency measurement capability, which are

-	Depending on UE capability, 7 NR SSB inter-frequency carriers configured by PCell, and

-	Depending on UE capability, 8 NR inter-frequency carriers including SSB and CSI-RS in total configured by PCell.

However, since ATG UE doesn’t need to monitor E-UTRA and UTRA carriers, the total effective carrier frequency layers that UE shall be capable of monitoring should be updated. We propose 8 as the candidate value at the current stage.

Proposal 31: For measurement capability of maximum allowed layers for multiple monitoring, reuse the current legacy intra and inter frequency measurement capability as the baseline. the total number of effective carrier frequency layers that UE shall be capable of monitoring update to 8.

For the CSSF, the assumption of searchers’ number should be aligned first. In our view, even though CA/DC is not included in R18 ATG, ATG UE could still support 2 searchers which is the legacy R15 assumption, since ATG support more processing power.

Proposal 32: Take 2 searchers as the assumption when defining the CSSF requirement.

Issue 5-1-2: NR intra-frequency measurements

- Reuse the principle from the legacy R15 NR intra-frequency measurements as baseline for ATG

  - Further discuss whether ATG specific impaction should be involved, such as CSSF and so on.

Issue 5-1-3: NR inter-frequency measurements

- Reuse the principle from the legacy R15 NR inter-frequency measurements as baseline for ATG

  - Further discuss whether ATG specific impaction should be involved.

Issue 5-1-6: CSI-RS based L3 measurements

- Option 1: Need defined RRM requirements for ATG UE (CATT, CMCC)

- Option 2: FFS Whether to define requirements for CSI-RS based measurement and positioning measurement for ATG. (HW, Ericsson, Apple, LGE, ZTE)

For above requirements, we think the legacy measurement requirements can be reused for ATG. On one hand, legacy scenario contains ATG scenario that only have NR SA deployment in FR1. On the other hand, due to large cell range, the measurement requirement doesn’t need to be tightened, UE have enough time to finish the measurement.

Proposal 33: The legacy NR intra-frequency measurements requirement, NR inter-frequency measurements requirement and CSI-RS based L3 measurements requirement can be reused for R18 ATG.

Issue 5-1-5: Cross Link Interference measurements

- Option 1: Need defined RRM requirements for ATG UE (CATT)

- Option 2: This requirement is not necessary for ATG UE (Apple, CMCC, Ericsson, LGE, ZTE)

- Option 3: FFS the RRM impact of CLI (HW, CMCC)

With the assumption of well-designed frame structure as proposed in proposal 1, there will be no cross-link interference among ATG UEs. However, there may exist cross-link interference between ATG UE and TN UE.  Taken our proposal 1 as the assumption of ATG frame structure, and DDDSUDDSUU as the assumption of TN frame structure.

| n-1 |  |  |  |  |  |  |  |  |  |  | n |  |  |  |  |  |  |  |  |  |  | n+1 |  |  |  |  |  |  |  |  |  |  | n+2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |
| D | D | S | S | S | S | U | U | U | U |  | U | U | D | D | D | D | D | D | D | D |  | D | D | D | D | D | D | D | D | D | D |  | D | D | D | D | D | D | D | D | D | D |  |
| n-1 |  |  |  |  |  |  |  |  |  | n |  |  |  |  |  |  |  |  |  |  | n+1 |  |  |  |  |  |  |  |  |  |  | n+2 |  |  |  |  |  |  |  |  |  |  |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |  |
| D | D | D | S | U | D | D | S | U | U |  | D | D | D | S | U | D | D | S | U | U |  | D | D | D | S | U | D | D | S | U | U |  | D | D | D | S | U | D | D | S | U | U |  |

In frame n, sub-frame 4, 8 and 9, frame n+2, sub-frame 4, 8 and 9, the UL transmission from TN UE may interfere the DL transmission for ATG UE. Based on our calculation, with the simulation assumption of CPE height 10km, operating frequency 4.9GHz, the REFSENSE of ATG CPE will be degraded by 3dB. Therefore, we think the CLI may need to be considered in ATG.

Issue 5-1-7: L1-RSRP measurements for a cell with different PCI from serving cell

- Option 1: Need defined RRM requirements for ATG UE (CATT, CMCC)

- Option 2: FFS (HW, Ericsson, Apple, LGE, ZTE)

Since this feature is mainly used in multi-TRP, which is not typical scenario in ATG, there is no need to introduce this feature to ATG.

Proposal 34: Don’t consider L1-RSRP measurement for a cell with different PCI from serving cell in R18 ATG.

Issue 5-1-8: NR measurements with autonomous gaps

- Option 1: Need defined RRM requirements for ATG UE (CATT, CMCC)

  - Option 1-1: Reusing legacy requirements. (CMCC, HW, Ericsson)

- Option 2: FFS (Apple, ZTE)

We support to introduce this feature in ATG. It could help to save the cost of adding and optimizing neighbour cell relationship through ANR, which may be higher in ATG network. The legacy delay requirement can be reused for ATG.

Proposal 35: Reuse the legacy delay requirement for NR measurements with autonomous gaps for R18 ATG.

- Specification Documentation

Issue 6-1-1: How to involve ATG RRM core requirements in TS38.133

- Option 1: The RRM requirements for ATG UE can be defined in new sections of section number with suffix D in specification. (CATT, Apple)

- Option 2: Add ATG related requirements in the current corresponding section, similar as HST. (CMCC, ZTE)

We think how to involve ATG RRM core requirements depends on the number of requirements which are impacted by ATG.

Based on our proposal 1 to proposal 33, the ATG impacted requirements are listed as follows: In our view, RRM impacts due to ATG scenario are not that much. we think that adding the ATG related requirements in current section as HST is a better way.

| ATG impacted requirements | Location-based CHO Gradual timing adjustment CSSF |
| --- | --- |
| Potential ATG impacted requirements (Depends on the input from RF session) | Cell re-selection measurement Initial transmit timing requirements Te Timing advance adjustment requirement |
| Potential ATG impacted requirements (need further discussion) | RLM In-sync and Candidate beam detection evaluation period CLI measurement |

Proposal 36: Add ATG related requirements in the current corresponding section, similar as HST

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: First study ATG RRM requirements with ISD assumption of 100-200km. If RF session come to other conclusions beyond the assumption range, then come back to have more study on ISD related requirements.

Proposal 2: Consider FDD 15kHz SCS and TDD 30kHz SCS as the basic assumption for ATG RRM study.

Proposal 3: 20ms period TDD frame structure could be configured for ATG TDD 30kHz SCS, which the TDD pattern is 30D4S6U.

Proposal 4: ATG WI should avoid to introduce new UAI reporting method.

Observation 1: The ATG UAI reporting including altitude, location, propagation delay difference, flight path, speed, velocity reporting could be supported by existing approach in current NR spec.

Observation 2: The ATG UAI reporting including flight path reporting couldn’t be supported by existing approach in current NR spec.

Proposal 5: Don’t introduce flight path as ATG UAI.

Proposal 6: No need to introduce altitude, location, speed, velocity as ATG UAI.

Proposal 7: There is no necessity of introducing ATG UAI of propagation delay difference with the ISD assumption of 100-200km. The conclusion can be revisited after RF session draw the final conclusion of ISD.

Observation 3: For ATG deployment method 1, when ISD is smaller than 117.76km, both legacy R15 requirements and HST requirements can’t support UE to complete cell-reselection.

Proposal 8: If ATG deployment method 1 is adopted, whether and what DRX cycle length to configure is up to NW. As long as the ISD is smaller than 117.76km, UE is not required to fulfill the cell re-selection requirements for 2.56s DRX cycle length.

Observation 4: If the typical ISD assumption is larger than 78.5km, we think the legacy R15 cell-reselection requirement can be reused for ATG. While if the typical ISD assumption is smaller than 64km, then the HST cell-reselection requirements should be reused for ATG.

Proposal 9: With the basic ISD assumption of 100-200km, the legacy R15 cell-reselection requirement could be reused for ATG. The conclusion can be revisited after receiving final conclusion about ISD assumption and deployment approach from RF session.

Proposal 10: Reuse the legacy cell-reselection rule and procedure.

Proposal 11: The conclusion on MDT measurement requirements can be delayed until RAN4 decide how to define ATG cell-reselection measurement requirements.

Proposal 12: For SDT RRM requirements, the legacy requirements can be reused, no need to define ATG specific requirements.

Proposal 13: For NR ATG handover, reuse legacy NR HO requirements.

Proposal 14: Introduce legacy CHO for ATG, the legacy R16 CHO delay requirements can be reused.

Proposal 15: Introduce location-based CHO for ATG, the logic and signalling from R17 NTN could be reused, while the location-based CHO delay requirements should be revisited in ATG scenario.

Proposal 16: For NR ATG RRC Re-establishment and RRC Connection Release with Redirection, reuse the legacy NR delay requirements.

Proposal 17: For both 4-step and 2-step RA, reuse the legacy requirements for ATG.

Proposal 18: Introduce the mechanism of Koffset in ATG system. The conclusion can be revisited after RF session draw the final conclusion about ISD and so on.

Proposal 19: There is no need to introduce the UE based UL timing pre-compensation under the ISD assumption of 100-200km. The conclusion could be revisited after receiving the input about ISD and cell radius from RF session.

Proposal 20: Reuse the legacy R15 TN requirement for initial transmit timing requirement Te with the assumption of ISD 100-200km. The issue should be reviewed after receiving the conclusion about ISD and cell radius from RF session.

Proposal 21: The koffset mechanism may need to be introduced in timing advance adjustment delay requirement.

Proposal 22: For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS.

Proposal 23: Reuse Cell phase synchronization accuracy 3µs requirement for ATG.

Proposal 24: Do not update ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ tolerance requirements. It is up to network to decide whether to configure ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ or not, based the real deployment scenario, such as ISD.

Proposal 25: Reuse the legacy RLM OOS evaluation period and legacy BFD evaluation period.

Proposal 26: Further discuss whether the requirement for RLM In-sync evaluation period and Candidate beam detection evaluation period should be tightened or not in ATG network.

Proposal 27: Reuse the legacy ‘Active BWP switching delay’, ‘Active TCI state switching delay’, ’Active downlink TCI state switching delay for unified TCI’, ’Active uplink TCI state switching delay for unified TCI’ and ‘Pathloss reference signal switching delay’ requirement for ATG.

Proposal 28: Uplink spatial relation switch delay requirement will not be applicable for R18 ATG.

Proposal 29: No need to introduce the pre-configured measurement gap for R18 ATG.

Proposal 30: Reuse the legacy MG requriements and legacy measurement capability of Monitoring of multiple layers using gap for R18 ATG.

Proposal 31: For measurement capability of maximum allowed layers for multiple monitoring, reuse the current legacy intra and inter frequency measurement capability as the baseline. the total number of effective carrier frequency layers that UE shall be capable of monitoring update to 8.

Proposal 32: Take 2 searchers as the assumption when defining the CSSF requirement.

Proposal 33: The legacy NR intra-frequency measurements requirement, NR inter-frequency measurements requirement and CSI-RS based L3 measurements requirement can be reused for R18 ATG.

Proposal 34: Don’t consider L1-RSRP measurement for a cell with different PCI from serving cell in R18 ATG.

Proposal 35: Reuse the legacy delay requirement for NR measurements with autonomous gaps for R18 ATG.

Proposal 36: Add ATG related requirements in the current corresponding section, similar as HST

- Reference

[1] R4-2214347, WF on NR ATG RRM core requirements, CMCC.

[2] RP-221369, Revised WID on Air-to-ground network for NR, CMCC.