# 3GPP TSG-RAN WG4 Meeting #104-bis-e                                                       R4-2215510

# Electronic Meeting, October 10 – October 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 5.26.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for updating simultaneous Rx/Tx requirements for CA_n39-n41 and CA_n40-n41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_Simult_RxTx_R18 |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ΔTIB,c and ΔRIB,c of the following CA bands apply to simultaneous Rx/Tx requirements. - CA_n39-n41 - CA_n40-n41 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete duplicate term for CA_n34-n79. Remove non-simultaneous Rx/Tx requirements for The ΔTIB,c and ΔRIB,c of the following CA bands - CA_n39-n41 - CA_n40-n41 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The notes of simultaneous Rx/Tx capability for the following CA bands configurations remain missing and the specification remains unclear. - CA_n39-n41 - CA_n40-n41 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4, 7.3A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

### 6.2A.4	Configured output power for CA

#### 6.2A.4.2	ΔTIB,c for CA

<Unchanged texts are omitted>

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3 | n1 | 0.3 |
|  | n3 | 0.3 |
| CA_n1-n5 | n1 | 0.3 |
|  | n5 | 0.3 |
| CA_n1-n7 | n1 | 0.5 |
|  | n7 | 0.6 |
| CA_n1-n8 | n1 | 0.3 |
|  | n8 | 0.3 |
| CA_n1-n18 | n1 | 0.3 |
|  | n18 | 0.3 |
| CA_n1-n20 | n1 | 0.3 |
|  | n20 | 0.3 |
| CA_n1-n28 | n1 | 0.3 |
|  | n28 | 0.6 |
| CA_n1-n38 | n1 | 0.5 |
|  | n38 | 0.5 |
| CA_n1-n40 | n1 | 0.5 |
|  | n40 | 0.5 |
| CA_n1-n41 | n1 | 0.5 |
|  | n41 | 0.5 |
| CA_n1-n67 | n1 | 0.3 |
| CA_n1-n74 | n1 | 0.3 |
|  | n74 | 0.3 |
| CA_n1-n77 | n1 | 0.6 |
|  | n77 | 0.8 |
| CA_n1-n78 | n1 | 0.3 |
|  | n78 | 0.8 |
| CA_n2-n5 | n2 | 0.3 |
|  | n5 | 0.3 |
| CA_n2-n7 | n2 | 0.5 |
|  | n7 | 0.5 |
| CA_n2-n12 | n2 | 0.3 |
|  | n12 | 0.3 |
| CA_n2-n14 | n2 | 0.3 |
|  | n14 | 0.3 |
| CA_n2-n29 | n2 | 0.3 |
| CA_n2-n30 | n2 | 0.5 |
|  | n30 | 0.3 |
| CA_n2-n48 | n2 | 0.6 |
|  | n48 | 0.8 |
| CA_n2-n66 | n2 | 0.5 |
|  | n66 | 0.5 |
| CA_n2-n77 | n2 | 0.6 |
|  | n77 | 0.8 |
| CA_n2-n78 | n2 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n5 | n3 | 0.3 |
|  | n5 | 0.3 |
| CA_n3-n7 | n3 | 0.5 |
|  | n7 | 0.5 |
| CA_n3-n8 | n3 | 0.3 |
|  | n8 | 0.3 |
| CA_n3-n18 | n3 | 0.3 |
|  | n18 | 0.3 |
| CA_n3-n20 | n3 | 0.3 |
|  | n20 | 0.3 |
| CA_n3-n28 | n3 | 0.3 |
|  | n28 | 0.3 |
| CA_n3-n34 | n3 | 0.5 |
|  | n34 | 0.5 |
| CA_n3-n38 | n3 | 0.5 |
|  | n38 | 0.5 |
| CA_n3-n40 | n3 | 0.5 |
|  | n40 | 0.5 |
| CA_n3-n41 | n3 | 0.5 |
|  | n41 | 0.34 |
|  |  | 0.85 |
| CA_n3-n74 | n3 | 0.8 |
|  | n74 | 0.9 |
| CA_n3-n77 | n3 | 0.6 |
|  | n77 | 0.8 |
| CA_n3-n78 | n3 | 0.6 |
|  | n78 | 0.8 |
| CA_n3-n79 | n3 | 0.3 |
|  | n79 | 0.8 |
| CA_n5-n7 | n5 | 0.3 |
|  | n7 | 0.3 |
| CA_n5-n12 | n5 | 0.8 |
|  | n12 | 0.4 |
| CA_n5-n14 | n5 | 0.5 |
|  | n14 | 0.5 |
| CA_n5-n25 | n5 | 0.3 |
|  | n25 | 0.3 |
| CA_n5-n28 | n5 | 0.5 |
|  | n28 | 0.5 |
| CA_n5-n29 | n5 | 0.5 |
| CA_n5-n30 | n5 | 0.3 |
|  | n30 | 0.3 |
| CA_n5-n40 | n5 | 0.3 |
|  | n40 | 0.3 |
| CA_n5-n48 | n5 | 0.3 |
|  | n48 | 0.3 |
| CA_n5-n66 | n5 | 0.3 |
|  | n66 | 0.3 |
| CA_n5-n77 | n5 | 0.6 |
|  | n77 | 0.8 |
| CA_n5-n78 | n5 | 0.6 |
|  | n78 | 0.8 |
| CA_n7-n8 | n7 | 0.3 |
|  | n8 | 0.6 |
| CA_n7-n25 | n7 | 0.5 |
|  | n25 | 0.5 |
| CA_n7-n28 | n7 | 0.3 |
|  | n28 | 0.3 |
| CA_n7-n40 | n7 | 0.5 |
|  | n40 | 0.6 |
| CA_n7-n46 | n7 | 0.3 |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |
| CA_n7-n77 | n7 | 0.5 |
|  | n77 | 0.8 |
| CA_n7-n78 | n7 | 0.5 |
|  | n78 | 0.8 |
| CA_n7-n79 | n7 | 0.5 |
|  | n79 | 0.8 |
| CA_n8-n20 | n8 | 0.4 |
|  | n20 | 0.4 |
| CA_n8-n28 | n8 | 0.6 |
|  | n28 | 0.5 |
| CA_n8-n34 | n8 | 0.3 |
|  | n34 | 0.3 |
| CA_n8-n38 | n8 | 0.6 |
|  | n38 | 0.3 |
| CA_n8-n39 | n8 | 0.3 |
|  | n39 | 0.3 |
| CA_n8-n40 | n8 | 0.3 |
|  | n40 | 0.3 |
| CA_n8-n41 | n8 | 0.6 |
|  | n41 | 0.3 |
| CA_n8-n75 | n8 | 0.3 |
| CA_n8-n77 | n8 | 0.6 |
|  | n77 | 0.8 |
| CA_n8-n78 | n8 | 0.6 |
|  | n78 | 0.8 |
| CA_n8-n79 | n8 | 0.3 |
|  | n79 | 0.8 |
| CA_n12-n25 | n12 | 0.3 |
|  | n25 | 0.3 |
| CA_n12-n30 | n12 | 0.3 |
|  | n30 | 0.3 |
| CA_n12-n48 | n12 | 0.3 |
|  | n48 | 0.3 |
| CA_n12-n66 | n12 | 0.8 |
|  | n66 | 0.3 |
| CA_n12-n71 | n12 | 1 |
|  | n71 | 1 |
| CA_n12-n77 | n12 | 0.5 |
|  | n77 | 0.8 |
| CA_n13-n25 | n13 | 0.3 |
|  | n25 | 0.3 |
| CA_n13-n66 | n13 | 0.3 |
|  | n66 | 0.3 |
| CA_n13-n77 | n13 | 0.5 |
|  | n78 | 0.8 |
| CA_n14-n30 | n14 | 0.3 |
|  | n30 | 0.3 |
| CA_n14-n66 | n14 | 0.3 |
|  | n66 | 0.3 |
| CA_n14-n77 | n14 | 0.5 |
|  | n77 | 0.8 |
| CA_n18-n28 | n18 | 0.5 |
|  | n28 | 0.5 |
| CA_n18-n41 | n18 | 0.3 |
|  | n41 | 0.3 |
| CA_n18-n74 | n18 | 0.3 |
|  | n74 | 0.3 |
| CA_n18-n77 | n18 | 0.3 |
|  | n77 | 0.8 |
| CA_n18-n78 | n18 | 0.3 |
|  | n78 | 0.8 |
| CA_n20-n28 | n20 | 0.5 |
|  | n28 | 0.5 |
| CA_n20-n40 | n20 | 0.3 |
|  | n40 | 0.3 |
| CA_n20-n75 | n20 | 0.3 |
| CA_n20-n78 | n20 | 0.6 |
|  | n78 | 0.8 |
| CA_n20-n67 | n20 | 0.5 |
| CA_n24-n41 | n24 | 0.3 |
|  | n41 | 0.46 |
|  |  | 0.97 |
| CA_n24-n48 | n24 | 0.6 |
|  | n48 | 0.8 |
| CA_n24-n77 | n24 | 0.6 |
|  | n77 | 0.8 |
| CA_n25-n29 | n25 | 0.3 |
| CA_n25-n38 | n25 | 0.5 |
|  | n38 | 0.5 |
| CA_n25-n41 | n25 | 0.5 |
|  | n41 | 0.46 |
|  |  | 0.97 |
| CA_n25-n48 | n25 | 0.6 |
|  | n48 | 0.8 |
| CA_n25-n66 | n25 | 0.5 |
|  | n66 | 0.5 |
| CA_n25-n71 | n25 | 0.3 |
|  | n71 | 0.6 |
| CA_n25-n77 | n25 | 0.6 |
|  | n77 | 0.8 |
| CA_n26-n66 | n26 | 0.3 |
|  | n66 | 0.3 |
| CA_n26-n70 | n26 | 0.3 |
|  | n70 | 0.3 |
| CA_n28-n34 | n28 | 0.3 |
|  | n34 | 0.3 |
| CA_n28-n38 | n28 | 0.3 |
|  | n38 | 0.3 |
| CA_n28-n39 | n28 | 0.3 |
|  | n39 | 0.3 |
| CA_n28-n40 | n28 | 0.3 |
|  | n40 | 0.3 |
| CA_n28-n41 | n28 | 0.3 |
|  | n41 | 0.3 |
| CA_n28-n50 | n28 | 0.3 |
|  | n50 | 0.4 |
| CA_n28-n71 | n28 | 1.1 |
|  | n71 | 1.1 |
| CA_n28-n74 | n28 | 0.6 |
|  | n74 | 0.4 |
| CA_n28-n75 | n28 | 0.3 |
| CA_n28-n77 | n28 | 0.5 |
|  | n77 | 0.8 |
| CA_n28-n78 | n28 | 0.5 |
|  | n78 | 0.8 |
| CA_n28-n79 | n28 | 0.5 |
|  | n79 | 0.8 |
| CA_n29-n30 | n30 | 0.3 |
| CA_n29-n66 | n66 | 0.3 |
| CA_n29-n70 | n70 | 0.3 |
| CA_n29-n71 | n71 | 0.5 |
| CA_n29-n77 | n77 | 0.8 |
| CA_n30-n66 | n30 | 0.5 |
|  | n66 | 0.8 |
| CA_n30-n77 | n30 | 0.3 |
|  | n77 | 0.8 |
| CA_n34-n41 | n34 | 0.3 |
|  | n41 | 0.3 |
| CA_n34-n79 | n34 | 0.3 |
|  | n79 | 0.8 |
| CA_n38-n40 | n38 | 0.53 |
|  | n40 | 0.53 |
| CA_n38-n66 | n38 | 0.5 |
|  | n66 | 0.5 |
| CA_n38-n78 | n38 | 0.3 |
|  | n78 | 0.8 |
| CA_n38-n79 | n38 | 0.3 |
|  | n79 | 0.8 |
| CA_n39-n41 | n39 | 02 |
|  | n41 | 02 |
|  | n39 | 0.5 |
|  | n41 | 0.5 |
| CA_n39-n79 | n39 | 0.3 |
|  | n79 | 0.8 |
| CA_n40-n41 | n40 | 0.5 |
|  | n41 | 0.5 |
| CA_n40-n77 | n77 | 0.5 |
| CA_n40-n78 | n78 | 0.5 |
| CA_n40-n79 | n40 | 0.3 |
|  | n79 | 0.8 |
| CA_n41-n48 | n41 | 0.3 |
|  | n48 | 0.8 |
| CA_n41-n50 | n41 | 0.3 |
|  | n50 | 0.4 |
| CA_n41-n66 | n41 | 0.86 |
|  |  | 1.37 |
|  | n66 | 0.5 |
| CA_n41-n70 | n41 | 0.5 |
|  | n70 | 0.5 |
| CA_n41-n71 | n41 | 0.3 |
|  | n71 | 0.6 |
| CA_n41-n74 | n41 | 0.3 |
|  | n74 | 0.3 |
| CA_n41-n771 | n41 | 0.3 |
|  | n77 | 0.8 |
| CA_n41-n781 | n41 | 0.3 |
|  | n78 | 0.8 |
| CA_n41-n79 | n41 | 0.3 |
|  | n79 | 0.8 |
| CA_n46-n48 | n48 | 0.5 |
| CA_n46-n78 | n78 | 0.8 |
| CA_n46-n96 | n96 | 0.5 |
| CA_n48-n53 | n48 | 0.53 |
|  | n53 | 0.33 |
| CA_n48-n66 | n48 | 0.8 |
|  | n66 | 0.6 |
| CA_n48-n70 | n48 | 0.8 |
|  | n70 | 0.6 |
| CA_n48-n71 | n48 | 0.3 |
|  | n71 | 0.3 |
| CA_n48-n96 | n48 | 0.5 |
|  | n96 | 0.5 |
| CA_n50-n78 | n50 | 02 |
|  | n78 | 02 |
|  | n50 | 0.53 |
|  | n78 | 0.53 |
| CA_n66-n70 | n66 | 0.5 |
|  | n70 | 0.5 |
| CA_n66-n71 | n66 | 0.3 |
|  | n71 | 0.3 |
| CA_n66-n77 | n66 | 0.6 |
|  | n77 | 0.8 |
| CA_n66-n78 | n66 | 0.6 |
|  | n78 | 0.8 |
| CA_n70-n71 | n70 | 0.3 |
|  | n71 | 0.6 |
| CA_n70-n78 | n70 | 0.6 |
|  | n78 | 0.8 |
| CA_n71-n77 | n71 | 0.5 |
|  | n77 | 0.8 |
| CA_n71-n78 | n71 | 0.5 |
|  | n78 | 0.8 |
| CA_n74-n77 | n74 | 0.4 |
|  | n77 | 0.8 |
| CA_n74-n78 | n74 | 0.4 |
|  | n78 | 0.8 |
| CA_n75-n78 | n78 | 0.8 |
| CA_n76-n78 | n78 | 0.8 |
| CA_n77-n79 | n77 | 0.5 |
|  | n79 | 0.5 |
| CA_n78-n79 | n78 | 0.5 |
|  |  | 1.58 |
|  | n79 | 0.5 |
|  |  | 1.58 |
| CA_n78-n92 | n78 | 0.8 |
|  | n92 | 0.6 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

### 7.3A.3	ΔRIB,c for CA

<Unchanged texts are omitted>

##### 7.3A.3.2.1	ΔRIB,c for two bands

Table 7.3A.3.2.1-1: ΔRIB,c due to CA (two bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n28 | n28 | 0.2 |
| CA_n1-n67 | n67 | 0.2 |
| CA_n1-n77 | n1 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n78 | n78 | 0.5 |
| CA_n2-n48 | n2 | 0.2 |
|  | n48 | 0.5 |
| CA_n2-n66 | n2 | 0.3 |
|  | n66 | 0.3 |
| CA_n2-n77 | n2 | 0.2 |
|  | n77 | 0.5 |
| CA_n2-n78 | n2 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n41 | n41 | 04 |
|  |  | 0.55 |
| CA_n3-n67 | n3 | 0.3 |
| CA_n3-n74 | n3 | 0.3 |
|  | n74 | 0.5 |
| CA_n3-n77 | n3 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n79 | n79 | 0.5 |
| CA_n5-n12 | n5 | 0.5 |
|  | n12 | 0.3 |
| CA_n5-n77 | n5 | 0.2 |
|  | n77 | 0.5 |
| CA_n5-n78 | n5 | 0.2 |
|  | n78 | 0.5 |
| CA_n7-n8 | n8 | 0.2 |
| CA_n7-n40 | n40 | 0.5 |
| CA_n7-n46 | n7 | 0.3 |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |
| CA_n7-n77 | n77 | 0.5 |
| CA_n7-n78 | n7 | 0.5 |
|  | n78 | 0.5 |
| CA_n7-n79 | n79 | 0.5 |
| CA_n8-n28 | n8 | 0.2 |
|  | n28 | 0.2 |
| CA_n8-n77 | n8 | 0.2 |
|  | n77 | 0.5 |
| CA_n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n8-n79 | n79 | 0.5 |
| CA_n12-n66 | n12 | 0.5 |
| CA_n12-n71 | n12 | 0.8 |
|  | n71 | 0.8 |
| CA_n12-n77 | n12 | 0.2 |
|  | n77 | 0.5 |
| CA_n13-n77 | n13 | 0.2 |
|  | n78 | 0.5 |
| CA_n14-n77 | n14 | 0.2 |
|  | n77 | 0.5 |
| CA_n18-n77 | n77 | 0.5 |
| CA_n18-n78 | n78 | 0.5 |
| CA_n20-n78 | n78 | 0.5 |
| CA_n20-n40 | n40 | 0.5 |
| CA_n24-n48 | n24 | 0.2 |
|  | n48 | 0.5 |
| CA_n24-n77 | n24 | 0.2 |
|  | n77 | 0.5 |
| CA_n25-n48 | n25 | 0.2 |
|  | n48 | 0.5 |
| CA_n25-n66 | n25 | 0.3 |
|  | n66 | 0.3 |
| CA_n25-n71 | n71 | 0.3 |
| CA_n25-n77 | n25 | 0.2 |
|  | n77 | 0.5 |
| CA_n25-n78 | n25 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n71 | n28 | 0.7 |
|  | n71 | 0.7 |
| CA_n28-n74 | n28 | 0.2 |
| CA_n28-n75 | n28 | 0.2 |
| CA_n28-n77 | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n28-n79 | n28 | 0.2 |
|  | n79 | 0.5 |
| CA_n29-n77 | n29 | 0.2 |
|  | n77 | 0.5 |
| CA_n30-n66 | n30 | 0.5 |
|  | n66 | 0.4 |
| CA_n30-n77 | n77 | 0.5 |
| CA_n34-n40 | n34 | 0.3 |
|  | n40 | 0.3 |
| CA_n34-n79 | n79 | 0.5 |
| CA_n38-n66 | n38 | 0.5 |
|  | n66 | 0.5 |
| CA_n38-n78 | n38 | 0.4 |
|  | n78 | 0.5 |
| CA_n38-n79 | n38 | 0.5 |
|  | n79 | 0.5 |
| CA_n39-n40 | n39 | 0.3 |
|  | n40 | 0.3 |
| CA_n39-n41 | n39 | 0.22 |
|  | n41 | 0.22 |
|  | n39 | 0.2 |
|  | n41 | 0.2 |
| CA_n39-n79 | n79 | 0.5 |
| CA_n40-n77 | n40 | 0.4 |
|  | n77 | 0.5 |
| CA_n40-n78 | n40 | 0.4 |
|  | n78 | 0.5 |
| CA_n40-n79 | n79 | 0.5 |
| CA_n41-n48 | n41 | 0.5 |
|  | n48 | 0.5 |
| CA_n41-n66 | n41 | 0.56 |
|  |  | 17 |
|  | n66 | 0.5 |
| CA_n41-n71 | n71 | 0.2 |
| CA_n41-n771 | n77 | 0.5 |
| CA_n41-n781 | n78 | 0.5 |
| CA_n41-n79 | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_ n46-n48 | n48 | 0.5 |
| CA_n46-n78 | n78 | 0.5 |
| CA_n48-n53 | n48 | 0.53 |
| CA_n48-n66 | n48 | 0.5 |
|  | n66 | 0.2 |
| CA_n48-n70 | n48 | 0.5 |
|  | n70 | 0.2 |
| CA_n48-n96 | n48 | 0.5 |
| CA_n50-n78 | n50 | 0.22 |
|  | n78 | 0.22 |
|  | n50 | 0.23 |
|  | n78 | 0.23 |
| CA_n66-n77 | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n66-n78 | n66 | 0.2 |
|  | n78 | 0.5 |
| CA_n70-n78 | n70 | 0.2 |
|  | n78 | 0.5 |
| CA_n71-n77 | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n71-n78 | n71 | 0.2 |
|  | n78 | 0.5 |
| CA_n74-n77 | n77 | 0.5 |
| CA_n74-n78 | n78 | 0.5 |
| CA_n75-n78 | n78 | 0.5 |
| CA_n76-n78 | n78 | 0.5 |
| CA_n78-n92 | n78 | 0.5 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz |  |  |

<< End of change >>