3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215523

Electronic Meeting, 10 October – 19 October 2022

Source:	Skyworks Solutions, Inc.

Title:	MSD for DC_3A-(n)7AA

Agenda item:			5.1.1 UE RF Requirements

Document for:	Approval

# 1. Introduction

As a result of meeting RAN4 #104e, the test point was updated for DC_3A-(n)7AA [1]. The test point specifies the MSD of each cell group for the intra-band contiguous DC_(n)7AA. The MSD levels were tentatively proposed in brackets, based on a quick evaluation of the fourth order intermodulation product (IMD4) power spectral density (PSD). This contribution elicits measurement data that is used to refine these levels and proposes an updated MSD test point to align with the fall-back DC_3_n7 agreements.

# 2. Discussion

The requested uplink/downlink configurations for DC_3-(n)7 are reproduced from [1] in Table 1 below. An IMD4 interference, resulting from dual-uplink transmissions of LTE B3 and NR n7, may affect both cell groups of DL band 7/n7.

- From an uplink perspective: the DC_3A_n7A configuration is identical to the fallback combination DC_3_n7. Thus, it makes sense to adopt an MSD test point that is aligned with the agreed test point for DC_3_n7.

- From a downlink perspective, the IMD4 may overlap both the LTE and the NR cell groups in DL band 7/n7. This type of MSD is new to RAN4, and additional evaluation is necessary.

Table 1: Uplink Configurations for DC_3-(n)7 [1]

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-(n)7AA | DC_3A_n7A | CA_3A-7A | n7A |
| DC_3C-(n)7AA | DC_3A_n7A | CA_3C-7A | n7A |

## 2.1	Background: DC_3A_n7A Legacy MSD Test Point

The MSD test point for the fall-back combination DC_3A_n7A is based on text proposal [2], which proposed a 10.2dB MSD for band n7 based on the average of the LTE uplink CA_3A-7A 13dB band 7 (B7) MSD, and the DC_3A_n7A 7.5dB band n7 MSD calculation of [3] – refer to Table 2 reproduced from [2].

Table 2: Average MSD proposal for Band n7 MSD due to IMD4 2 UL interference for DC_3A_n7A

| DC bands | UL band | IMD |  | B7 MSD for LTE CA_3A-7A | n7 MSD for DC_3A_n7A [3] | Average n7 MSD for DC_3A_n7A |
| --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n7A | 3 | IMD4 | ¦fB7 -3*fB3¦ | 13 | 7.5 | 10.25 |
|  | n7 |  |  |  |  |  |

The agreed upon MSD test point for fall-back DC_3_n7 is reproduced in Table 3. This test point sets the IMD4 center frequency at 2655MHz, which corresponds to a direct hit and complete overlap of the affected DL band n7.

Table 3: Agreed Band n7 MSD due to IMD4 2 UL interference for DC_3A_n7A

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | IMD4 |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD |  |

Observation 1: For fall-back DC_3_n7, the band n7 MSD due to dual uplink IMD4 is the result of a direct hit and complete IMD4 overlap. The n7 10.2dB MSD is the average of LTE UL CA_3A-7A and DC_3A_n7A calculated MSD in [3].

## 2.2	Test Point Selection for DC_3A_(n)7AA

Table 4 addresses the agreed upon MSD test point from [1] for DC_3_(n)7. The IMD4 center frequency is 2629MHz. ie. Thus, it is nearly perfectly centered on the n7 DL Scell. This test point is aligned with the assumptions of the DC_3_n7 fall-back combination.

Table 4: DC_3_(n)7 MSD test point for n7/B7 [1]

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 7 | N/A | 5 | N/A | 2623.5 | [6.5] | IMD4 |
|  | n7 | 2508.5 | 5 | 25 | 2628.5 | [10.2] | IMD4 |

However, 5MHz CBW is proposed for DC_(n)7AA operation for DC_3_(n)7, while 10MHz CBW is specified for the fallback test point. We therefore offer a counter proposal as shown in of Table 5, with changes from [1] highlighted in yellow. For LTE B7 Scell, a 5MHz CBW is proposed to ensure that the worst-case MSD is specified.

Table 5: MSD RF parameter assumptions for DC_3_(n)7 MSD evaluation

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 7 | N/A | 5 | N/A | 2647.5 | [TBD] | IMD4 |
|  | n7 | 2535 | 10 | 50 | 2655 | [10.2] | IMD4 |

## 2.3	IMD4 Measurements and MSD evaluation

The IMD4 interference level is evaluated using Power Amplifier (PA) measurements, with the following assumptions:

- Power class 3 operation

- PA calibration: calibrated to meet -30dBc at 26dBm output power using a 20 MHz, SCS15 kHz, QPSK,    DFT-S-OFDM, waveform with Lcrb=100 RB at lower channel edge

- Post PA losses: 					 4dB

- Local Oscillator (LO) leakage: 	-28dBc

- IQ Image rejection: 				-28dB

- RF transceiver impairments: 		C-IM3: -60dBc, C-IM5: -70dBc

- Forward IMD4 only is evaluated at 2655MHz and 2647.5MHz for dual uplink transmissions of:

  - B3 UL Fc=1730MHz, 5MHz CBW, Lcrb=25RB, QPSK, DFT-s-OFDM, SCS15kHz

  - n7 UL Fc=2535MHz, 10MHz CBW, Lcrb=50RB, QPSK, DFT-s-OFDM, SCS15kHz.

### 2.3.1	IMD4 Power Spectral Density

The measured IMD4 PSD at 2655MHz is plotted in blue in Figure 1. For illustration purposes, the LTE and NR Scells are overlaid in Figure 1 in orange and red respectively. The black overlay illustrates the fall-back n7 10MHz CBW 10.2dB MSD test condition.

Figure 1: Measured IMD4 PSD (blue). For illustration purposes: n7 10MHz DL carrier overlaid in red and black for DC_3_(n)7/DC_3_n7. B7 5MHz DL carrier in orange.

### 5.2.4	REFSENS requirements

Since the agreed upon MSD level is derived from an average of different RF-FE assumptions, we propose to evaluate the MSD by measuring the relative IMD4 power levels integrated in each of the DC_(n)7AA DL Scell/Cell group. The measured IMD4 level integrated in the LTE B7 5MHz Scell is approximately 7.9dB lower (orange dashed area – Figure 1) than the interference power integrated in the NR n7 10MHz Scell (red dashed area – Figure 1).

Observation 2: The IMD4 interference level integrated in the LTE B7 5MHz Scell is approximately 7.9dB lower than the IMD4 interference integrated in the n7 10MHz DL Scell.

Based on observation 2, we propose the following MSD specifications.

Proposal: For DC_3_(n)7, adopt the MSD test point from Table 6.

Table 6: Proposed DC_3_(n)7 MSD test point

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 7 | N/A | 5 | N/A | 2647.5 | 6.9 | IMD4 |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |

# 3. Conclusion

In this contribution, we present power amplifier IMD4 measurements to evaluate the band n7/B7 intra-band contiguous ENDC MSD resulting from dual uplink transmission of band B3 and band n7 IMD4 product for DC_3_(n)7. We propose to adopt the following MSD test point.

Proposal: For DC_3_(n)7, adopt the MSD test point from Table 6.

Table 5: Proposed DC_3_(n)7 MSD test point

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-(n)7AA DC_3C-(n)7AA | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 7 | N/A | 5 | N/A | 2647.5 | 6.9 | IMD4 |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |

# Reference

[1]		R4-2213094, “TP for TR 37.718-21-11: Including band combinations DC_3-(n)7”, 3GPP TSG-RAN WG4 Meeting #104-e, Ericsson, Telstra.

[2]		R4-1712512, “TP for TR 37.863-01-01 UE requirement for DC_3A_n7A”, 3GPP TSG-RAN WG4 Meeting #85, CHTTL.

[3]	R4-1710724, “MSD analysis for NSA DC UE at sub-6GHz”, 3GPP TSG-RAN WG4 Meeting #84bis, LG Electronics.