3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215526

Electronic Meeting, 10 October – 19 October 2022

Source:	DISH Network, Samsung

Title:	TP for TR 38.718-03-01: CA_n29-n70-n71

Agenda item:			5.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-03-01 to include CA_n29A-n70A-n71A.

All the fallbacks have already been specified.

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n29-n70-n71

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n29-n70-n71 | n29 | N/A | – | N/A | 717 MHz | – | 728 MHz | FDD |
|  | n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n29A-n70A-n71A | CA_n70A-n71A | n29 | 5, 10 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |

The following notes are applied to the above tables:

NOTE 1:	This UE channel bandwidth is applicable only to downlink.

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n29-n70-n71,

- the TIB,c values are reused from CA_n70-n71.

- the RIB,cvalues are reused from CA_n5-n14-n77 for n29 and n71, and n70 value from n2 three combinations (0.2dB is used when n2 is the only mid band).

These are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n29-n70-n71 | n29 | 0 |
|  | n70 | 0.3 |
|  | n71 | 0.6 |

Table 5.x.1.3-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n29-n70-n71 | n29 | 0.2 |
|  | n70 | 0.2 |
|  | n71 | 0.2 |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

UE co-existence has already been studied for 2DL/1UL fallback combinations. The impact to the 3rd band Rx is following:

- No IMD generated by dual uplink of Band n70 + Band n71 falls into Band n29 DL.

#### 5.x.2.2	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

---End of changes---

# Reference

[1] 		RP-222092, Revised WID on Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2),  ZTE