3GPP TSG-RAN WG4 Meeting #104-bis-e 	R4-2215534

Electronic Meeting, 10-19 Oct., 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On PDCCH and PBCH Requirements for ext71GHz

Agenda item:	4.3.7.2.2

Document for:	Discussion

Introduction

During the last RAN4#104-e meeting, good progress was made on the topic of defining requirements for PDCCH and PBCH for 71GHz.

In this contribution we will express our views on the open issues and open new discussions, if necessary.

Discussion

PDCCH Demodulation Requirements

Set of PDCCH Test Cases

In RAN4#104e it was agreed to define PDCCH requirements according to below table [1]:

Issue 2-1-2: Set of PDCCH Test Cases - Agreement: Define PDCCH requirements according to the table below. Test number; CBW/SCS; CORESET duration; CORESET RB; Interleaver size; REG bundle size; Aggregation level; Propagation Condition; DCI format; Information Bit; Antenna configuration and correlation Matrix. 1; 100/120; 1; 60; 3; 2; 2; [TDLA30-200]; 1_0; 40; 1x2 Low. 2; 100/120; 1; 60; 2; 6; 4; [TDLA30-200]; 1_1; 56; 1x2 Low. 3; 100/120; 1; 60; 3; 2; 8; [TDLA30-200]; 1_1; 56; 2x2 Low. 4; 100/120; 2; 60; 3; 2; 16; [TDLA30-200]; 1_0; 40; 2x2 Low. 5; 400/480; 1; 60; 2; 6; 4; [TDLA10-200]; 1_1; 56; 1x2 Low. 6; 400/480; 1; 60; 2; 6; 8; [TDLA10-200]; 1_1; 56; 1x2 Low. 7; 400/480; 2; 60; 3; 2; 16; [TDLA10-200]; 1_0; 40; 2x2 Low.

We have provided our simulation results for the agreed table in [2].

The only remaining issue is to agree on the propagation conditions. The propagation conditions provided in the above table are aligned with agreements to use TDLA30-200 for 120kHz SCS and TLDA10-200 for 480kHz SCS test cases.

Current propagation conditions are aligned with other test cases.

Use TDLA30-200 for 120kHz SCS and TLDA10-200 for 480kHz SCS.

  - PBCH Demodulation Requirements

Test setup for PBCH requirements

In RAN4#104e it was agreed to define PBCH requirements according to below table [1].:

Issue 2-2-2: Test setup for PBCH requirements - Define requirements for the tests below. Propagation condition to be further discussed; Test number; SCS/CBW; Antenna configuration; Propagation condition; PBCH SNR (dB) @ 1% Pm-bch; SS/PBCH block index. 1-1; 120kHz/100MHz; 1x2 Low; [TDLA10-200]; New; unknown. 1-2; 480kHz/400MHz; [TDLA10-200]; New; unknown.

Test 1-1 is suggested with TDLA10-200, which is not aligned with 120kHz/100MHz using TDLA30-200 for all other requirement definitions. In our simulation results in [2], we have included a test 1-1a where TDLA30-200 is used.

Test 1-1 in the agreed tests for PBCH Demodulation requirements have TDLA10-200 as proposed propagation delay. This is not aligned with all other tests for 120kHz/100MHz SCS/CBW.

Change propagation condition for PBCH test 1-1 (120kHz/100MHz) to TDLA30-200. Use TDLA10-200 for test 1-2 (480kHz/400MHz).

Conclusion

In this contribution we have provided our views on various open issues with relation to PDCCH and PBCH for the extension to 71GHz.

We have made the following observations and proposals:

PDCCH Demodulation Requirements

- Current propagation conditions are aligned with other test cases.

- Use TDLA30-200 for 120kHz SCS and TLDA10-200 for 480kHz SCS.

PBCH Demodulation Requirements

Test 1-1 in the agreed tests for PBCH Demodulation requirements have TDLA10-200 as proposed propagation delay. This is not aligned with all other tests for 120kHz/100MHz SCS/CBW.

Change propagation condition for PBCH test 1-1 (120kHz/100MHz) to TDLA30-200. Use TDLA10-200 for test 1-2 (480kHz/400MHz).

- References

- R4-2214390 - WF on FR2-2 UE demodulation requirements

- R4-2215535 - PDCCH and PBCH simulation results for ext71GHz