3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215535

Electronic Meeting, 10-19 Oct., 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PDCCH and PBCH simulation results for ext71GHz

Agenda item: 	5.3.7.2.2

Document for: 	Discussion

Introduction

This paper presents PDCCH and PBCH simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

The simulations are done using the proposed channel models defined in [1].

Simulation Results

PDCCH Simulation Results

Table 1 Results for PDCCH

| Test number | CBW/SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Pm dsg (%) | SNRBB (dB) |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | [TDLA30-200] | 1_0 | 40 | 1x2 Low | 1 | 2.23 |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | [TDLA30-200] | 1_1 | 56 | 1x2 Low | 1 | -1.19 |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | [TDLA30-200] | 1_1 | 56 | 2x2 Low | 1 | -1.91 |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | [TDLA30-200] | 1_0 | 40 | 2x2 Low | 1 | -7.23 |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | [TDLA10-200] | 1_1 | 56 | 1x2 Low | 1 | -1.75 |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | [TDLA10-200] | 1_1 | 56 | 1x2 Low | 1 | -4.95 |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 | [TDLA10-200] | 1_0 | 40 | 2x2 Low | 1 | -7.22 |

PBCH Simulation Results

Table 2 Test parameters for PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| Note 1: as specified in clause 4.1 of TS 38.213 |  |  |

Table 3 Minimum performance PBCH in case SS/PBCH block index is not known

| Test number | SCS/CBW | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | [TDLA10-200] | 4.5 | unknown |
| 1-1a | 120kHz/100MHz | 1x2 Low | [TDLA30-200] | -0.73 | unknown |
| 1-2 | 480kHz/400MHz | 1x2 Low | [TDLA10-200] | 2.28 | unknown |

References

- R4-2214390 - WF on FR2-2 UE demodulation requirements