3GPP TSG-RAN4 Meeting #104-bis-e	R4-2215542
Electronic Meeting, 10th October – 19th October 2022

Agenda item:	6.15.1

Source:	Keysight Technologies

Title:	On FR2 Requirements Framework and Correlation between Simulations and Measurements

Document for:	Approval

# Introduction

This contribution addresses the FR2 requirements framework, specifically the correlation between simulations and measurements which Keysight can support.

# Discussion

In the last Status Report [1] of the Rel-17 NR MIMO OTA [2], it was highlighted that the FR2 requirements could not be agreed (on time)

- Agreements on FR2 Performance requirements - Issue 3-1: General views on FR2 MIMO OTA requirements development - No agreements in RAN4 on FR2 MIMO OTA requirements. How to conclude the performance part of FR2 MIMO OTA depending on the RAN-P decision.

which is why the FR2 requirements were removed from the original WI [2]

Specify the FR2 MIMO OTA requirements: - Define the detailed Figure of Merit for FR2 - Based on how to process the data, specify the sensitivity value - FR2 requirements - Band n257, n258, n260 and n261 are the first priority

One of the key concerns highlighted during the discussions in RAN4 was the lack of correlation between simulations and measurements (even though it was previously agreed: “both simulation results and measurement results are permitted to define requirements”) and the gap between simulations from two companies making different antenna assumptions.

In order to define FR2 requirements after all, a new REl-18 WI was approved [3] where the requirements framework is considered a high priority for the upcoming two RAN4 meetings.

- FR2 MIMO OTA test methodology enhancement and leftovers - Study and define the framework for FR2 MIMO OTA requirement development (1st priority) - Hybrid of simulation results and measurement results will be considered as the starting point in FR2 requirement definition - Note: Revisit in RAN#98 to check the feasibility and progress of hybrid simulation and measurement approach. - The correlation between simulation results and measurement results will be considered - Simulation assumptions agreed in R17 MIMO OTA WI can be considered as the baseline - Study how to handle the situation when not enough measurement results can be collected

Here, we share our view on the FR2 requirements framework, especially the aspect on correlating the simulations and measurements.

Since FR2 OTA requirements in the past (UE RF, RRM, Demod) with and without channel modelling were defined in the past solely based on simulations, a simulation approach to define requirements should not be precluded for FR2 MIMO OTA.

Proposal 1: Allow the FR2 MIMO OTA requirements to be defined solely based on simulations.

Given the feedback in the last couple RAN4 meetings suggesting the need for correlating simulations with measurements, it is proposed to formalize the need of such correlation.

Proposal 2: Correlate simulations with measurements as part of the FR2 MIMO OTA requirements framework.

Given the relatively low adoption rate of FR2 MIMO OTA systems at this point and the corresponding lead times, it is proposed not to require an inter-lab alignment and measurement campaign to define requirements similar to FR1/LTE MIMO OTA and FR1 TRP/TRS.

Proposal 3: Avoid an inter-lab alignment and measurement campaign to define FR2 MIMO OTA requirements

As Keysight has a commercial FR2 MIMO OTA solution readily available, we are willing to support a limited number of companies that in turn support the FR2 requirements definition using simulations with FR2 MIMO OTA measurements of selected devices (commercial UEs, prototypes, breadboards, etc.) to advance the efforts to correlate simulations with measurements. Details of this correlation campaign with OEMs/chipset vendors (number of devices, bands, logistics) are TBD.

Observation 1: Keysight can support a limited number of companies actively contributing on FR2 simulations with measurements to advance efforts to correlate simulations with measurements.

Sample FR2 MIMO test data for three commercial DUTs tested based on the test cases and test methodology defined in [4][5] for Band n261 is presented in Figure 1 which shows the sensitivity levels measured at each of the 36 test points. Additionally, the MASC results are tabulated in Table 1 for these devices.

Figure 1: Sample FR2 MIMO OTA test data for three commercial devices (Band n261)

Table 1: MASC Results for three commercial devices (Band n261)

| Device | MASC |
| --- | --- |
| DUT #1 | -106.87 dBm |
| DUT #2 | -108.24 dBm |
| DUT #3 | -102.97 dBm |

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: Keysight can support a limited number of companies actively contributing on FR2 simulations with measurements to advance efforts to correlate simulations with measurements.

Proposal 1: Allow the FR2 MIMO OTA requirements to be defined solely based on simulations.

Proposal 2: Correlate simulations with measurements as part of the FR2 MIMO OTA requirements framework.

Proposal 3: Avoid an inter-lab alignment and measurement campaign to define FR2 MIMO OTA requirements

# References

- RP-222634, Status report for WI: Perf. part: Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs; rapporteur: CAICT, RAN4, 3GPP TSG RAN meeting #97e, September 2022

- RP-222348, Revised WID: Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs, CAICT, OPPO, 3GPP TSG RAN meeting #97e, September 2022

- RP-222668, R18 New WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs, CAICT, OPPO, Keysight Technologies, 3GPP TSG-RAN Meeting #97-e, September 2022

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements, V17.2.0 (2022-09)

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE), V17.8.0 (2022-09)