3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215544

Electronic Meeting, October 10 – October 19, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Sub-slot based PUCCH repetition Simulation Results

Agenda item: 	4.7.3.1

Document for: 	Discussion

Introduction

In RAN4#104e almost all open issues for sub-slot based PUCCH repetition performance requirements were agreed, as captured in the summary [1] and WF [2].

In this contribution we collect our simulation results, which are partially re-used in our companion discussion paper.

Simulation setup

Table 1: Main test parameters forPF0 sub-slot repetition.

| Parameter | Test |
| --- | --- |
| PUCCH Format | 0 |
| Number of information bits | 1 (HARQ ACK payload) |
| Number of PRBs | 1 |
| Number of PUCCH symbols (nrofSymobls) | 2 |
| Number of sub-slot PUCCH repetitions (nrofSlots) | 2 |
| Number of Sub-slot symbols (subslotLengthForPUCCH-r16) | 7 |
| First symbol of sub-slot (startingSymbolIndex) | 5 |
| Intra-subslot frequency hopping (intraSlotFrequencyHopping) | disabled |
| Inter-subslot frequency hopping (interslotFrequencyHopping) | enabled |
| First PRB prior to frequency hopping | 0 |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |

Table 2: Further simulation parameters.

| Parameter | Value |
| --- | --- |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Carrier frequency (GHz) for simulation alignment | 4 |
| Cyclic Prefix | Normal |
| Propagation condition | TDLC-300-100 Low |
| SCS and BW | 30kHz/10MHz |
| Timing offset | 0 |
| Frequency offset | 0 |
| Tx EVM | None |
| Testing metric | SNR @ Prob(DTX to ACK)<1%. FFS: SNR @ Prob(ACK miss)<1% FFS: SNR @ Prob(PUCCH NACK to ACK bits) < 0.1%. |

A simplified graphical representation of this configuration could be understood as follows:

Figure 1: Simplified graphical representation of simulated configuration.

Simulation results

Following the above simulation scenarios.

DTX to ACK

This subsection is included to state that the DTX detection thresholds are correctly set, such that Prob(DTX to ACK)<1% for all AWGN power densities commonly used for 30kHz/10MHz (i.e., -80.6 dBm / 8.64 MHz).
Since this figure would just be a single data point, we have opted to not include such a figure.

ACK MD/NACK to ACK bottleneck

This subsection contains graphs to see the performance behaviour of ACK MD and NACK to ACK, which is needed to evaluate the bottleneck KPI (assuming one KPI dominates the other in the first place).

Figure 1: NACK-ACK probability and miss-Ack probability

ACK MD

This subsection contains our results for SNR @ Prob(ACK miss)<1%.

| Number of TX | Number of RX | Cyclic Prefix | Propagation conditions and correlation matrix | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- |
| antennas | antennas |  | (Annex G) | 10MHz |
| 1 | 2 | Normal | TDLC-300-100 Low | -0.8dB |

Table 2 simulation results of ACK MD < 1% probability  for multi-subslot PUCCH format 0 with 30kHz SCS/10MHz.

NACK to ACK

This subsection contains our results for SNR @ Prob(PUCCH NACK to ACK bits) < 0.1%.

| Number of TX | Number of RX | Cyclic Prefix | Propagation conditions and correlation matrix | Channel bandwidth / SNR (dB) |
| --- | --- | --- | --- | --- |
| antennas | antennas |  | (Annex G) | 10MHz |
| 1 | 2 | Normal | TDLC-300-100 Low | 4.36dB |

Table 2 simulation results of NACK 2 ACK < 0.1% probability  for multi-subslot PUCCH format 0 with 30kHz SCS/10MHz.

Conclusion

In this contribution we have provided our simulation results for Sub-slot based PUCCH repetition.
We have not made any observations or proposals.

References

- R4-2214318 Email Discussion Summary for [104-e][329] NR_IIOT_URLLC_enh_Demod, RAN4#104e, Moderator (Nokia, Nokia Shanghai Bell).

- R4-2214396 WF on enhanced IIoT and URLLC support demodulation and CSI requirements, RAN4#104e, Nokia, Nokia Shanghai Bell.