3GPP TSG-RAN WG4 Meeting #104-bis-e 	       R4-2215546

# Electronic Meeting, 10 - 19 October 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on PDSCH demodulation requirements for NTN

Agenda item: 	4.2.7.3.1

Document for:	Approval

# Introduction

The NTN WI is presented in [1], where the following RAN4 objectives are defined:

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite Note 2: The spectrum usage on the service link for HAPS might be a different spectrum allocation than for Satellite.

In this contribution we present discussion of some open issues for PDSCH demodulation requirements.

# Discussion

Issue 2-3: K-offset value

- Proposals: Only specify the k_offset values for LEO-600. Encourage TE vendors to provide the input on the feasibility of supporting a large k_offset value referring to 30o elevation angle for NTN-TDLA test cases and refers to 90° elevation angle for NTN-TDLC test cases.

  - Option 1: Provide the specific values of k_offset for HARQ disabled and HARQ process of 32

- Agreement:

  - K_offfset = [8 slots] applied for all HARQ configurations

Issue 2-5: Modulation order

- Proposals: to confirm the following tentative agreements:

  - Option 1: Do not specify PDSCH for performance requirements for 64QAM

  - Option 2: Specify PDSCH for performance requirements for 64QAM

- Agreement:

  - FFS whether specify PDSCH requirements for 64QAM in Rel-17, operators’ feedback on the demand are encouraged.

Issue 2-6: SCS/CBW set

- Proposals: to confirm the following tentative agreements:

  - Option 1: Do not specify requirements with additional 30KHz SCS for NTN.

  - Option 2: Specify requirements with additional 30KHz SCS for NTN

Issue 2-3: K-offset value

- Proposals: Only specify the k_offset values for LEO-600. Encourage TE vendors to provide the input on the feasibility of supporting a large k_offset value referring to 30o elevation angle for NTN-TDLA test cases and refers to 90° elevation angle for NTN-TDLC test cases.

  - Option 1: Provide the specific values of k_offset for HARQ disabled and HARQ process of 32

- Agreement:

  - K_offfset = [8 slots] applied for all HARQ configurations

Issue 2-5: Modulation order

- Proposals: to confirm the following tentative agreements:

  - Option 1: Do not specify PDSCH for performance requirements for 64QAM

  - Option 2: Specify PDSCH for performance requirements for 64QAM

- Agreement:

  - FFS whether specify PDSCH requirements for 64QAM in Rel-17, operators’ feedback on the demand are encouraged.

Issue 2-6: SCS/CBW set

- Proposals: to confirm the following tentative agreements:

  - Option 1: Do not specify requirements with additional 30KHz SCS for NTN.

  - Option 2: Specify requirements with additional 30KHz SCS for NTN

RAN4 #104-e agreed a WF with several topics [2]:

K_offset values have been discussed based on the four option in the previous meeting and the agreement above has been made. There is also an agreement that the K-offset should be at least twice the satellite one-way delay. It has also been proposed that K_offset should depend on elevation angle, and the K-offset corresponding to LEO-600 and 30 degree elevation angle has been chosen. We are also interested to hear if TE vendors have comments to this choice.

It should be noted that this value of K-offset corresponds to already agreed 15kHz SCS. Issue need to be discussed again if 30kHz SCS is introduced.

Observation 1: K-offset tentatively agreed is assuming already agreed SCS of 15kHz.

Proposal 1. Remove square brackets and confirm agreements of K_offset equal to 8 slots for 15kHz SCS

Proposal 2. Scale K_offset value with SCS if 30kHz SCS is introduced

Modulation order for NTN PDSCH demodulation requirements has been discussed and agreement reached to further discuss if 64QAM requirements are necessary. 64QAM support in general has been discussed and an agreement has been made in RAN4 #102-e to make 64QAM as optional per band. However if we look at the link budget results in TR 38.821 [3], they clearly show that SNRs required by 64QAM do not exist in any of the cases with 2GHz carrier frequency. Hence this topic should not be prioritized in discussions.

Observation 2: The link budget results in TR 38.821 [3] clearly show that SNRs required by 64QAM do not exist in any of the cases with 2GHz carrier frequency.

Proposal 3. Do not specify PDSCH requirements for 64QAM in Rel-17.

There is agreement to select 15kHz SCS and 10MHz bandwidth and consider 30kHz SCS with 20MHz bandwidth as additional case. Existing specification has only one RMC with 64QAM for 30kHz PDSCH. Hence if 30kHz and QPSK/16QAM requirements are desired then new RMC:s would need to be introduced.

Observation 3: Existing specification has only one RMC with 64QAM for 30kHz PDSCH.

Proposal 4. Do not specify PDSCH requirements with additional 30kHz SCS for NTN.

# Conclusion

This contribution discusses aspects related to NTN PDSCH demodulation requirements for NTN and has the following observations:

Observation 1: K-offset tentavively agreed is assuming already agreed SCS of 15kHz.

Proposal 1. Remove square brackets and confirm agreements of K_offset equal to 8 slots for 15kHz SCS

Proposal 2. Scale K_offset value with SCS if 30kHz SCS is introduced

Observation 2: The link budget results in TR 38.821 [3] clearly show that SNRs required by 64QAM do not exist in any of the cases with 2GHz carrier frequency.

Proposal 3. Do not specify PDSCH requirements for 64QAM in Rel-17.

Observation 3: Existing specification has only one RMC with 64QAM for 30kHz PDSCH.

Proposal 4. Do not specify PDSCH requirements with additional 30kHz SCS for NTN.

# References

- RP-222556, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2214386, WF for NTN demodulation requirements - general and PDSCH, Qualcomm

- TR 38.821 v16.1.0, Solutions for NR to support non-terrestrial networks (NTN), 3GPP