3GPP TSG-RAN WG4 Meeting #104-bis-e 	       R4-2215551

# Electronic Meeting, 10 - 11 October 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NTN SAN PUCCH demodulation requirements

Agenda item: 	4.2.7.2.2

Document for:	Discussion

# Introduction

The NTN WI is presented in [1], where the following RAN4 objectives are defined:

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite Note 2: The spectrum usage on the service link for HAPS might be a different spectrum allocation than for Satellite.

In this contribution we present discussion of some open issues of SAN PUCCH demodulation requirements for NTN.

# Discussion

Agreement from way forward [2]:

Antenna configuration

Candidate options:

- Option 1: RAN4 consider SAN PUCCH format 0 and format 2 requirements for only 2Rx configuration. Corresponding manufacture declarations and applicability rules should be further discussed.

- Option 2: 2Rx only for PUCCH requirement

- Option 3: 1Rx only for PUCCH requirement

In our view, SNR requirements for the short PUCCH formats are high, especially with only one Rx, and according to link budget calculations such high SNRs are not very common in NTN. For the results see our accompanying contribution [3]. Therefore, we are fine to limit short formats to 2Rx only.

Observation 1 : Due to high SNR requirement, PUCCH format 0 and 2 may have only 2Rx requirements.

Proposal 1. RAN4 to agree with option 1 and consider SAN PUCCH format 0 and format 2 requirements for only 2Rx configuration.

# Conclusion

This contribution discusses aspects related to NTN SAN demodulation requirements and has the following observation and proposal:

Observation 1 : Due to high SNR requirement, PUCCH format 0 and 2 may have only 2Rx requirements.

Proposal 1. RAN4 to agree with option 1 and consider SAN PUCCH format 0 and format 2 requirements for only 2Rx configuration.

# References

- RP-222556, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2214387 WF on NTN SAN demodulation requirements, Huawei

- R4-2215550, Simulation results for NTN SAN PUCCH demodulation, Nokia