3GPP TSG-RAN WG4 Meeting #104Bis-e	R4-2215565

Oct 2022

Title:	DMRS bundling CA aspects

Source:	Qualcomm Incorporated

Agenda item:	4.4.1.1

Release:	Rel-17

Work Item:	NR_cov_enh

Responsible WG:	RAN1

Document for:	Approval

# 1.   Introduction

RAN#97 Agreed WF [39] contains the guidance from plenary to WGs:

- Issue#1: Support of DMRS bundling in case of UL operation over multiple carriers

Proposal1: On the issue of supporting DMRS bundling in case of UL operation over multiple
     carriers, WGs are to continue with their work to resolve the issue (for radio configurations
     included in R4-2211225) before RAN#98 under the assumption of no RAN1 specification
     change.

RAN1 LS reply [40] listed the cases with the associated notion:

-	DL CA with “additional” UL carrier configured with SRS only (i.e. no PUCCH/PUSCH configured)

-	FR1 inter-band UL CA with DMRS bundling

-	SUL with DMRS bundling

RAN1 has been evaluating the feasibility and conditions supposing there is no RAN1 spec impact. RAN1 will inform RAN4 the updates in the next RAN1 meeting.

With these, RAN4 can proceed with the technical discussion on how to agree UL CA support within the bounds of current ran1 specifications which will discuss in this paper.

# 2. 	Discussion

## 2.1	DL CA with SRS only carrier

The DL CA part defines DMRS bundling requirements to apply “with one uplink carrier assigned to one NR band”. The item in the LS “DL CA with “additional” UL carrier configured with SRS only” means there can be one “regular” carrier and one “SRS only” carrier for uplink. In our view, this is a subset of the uplink CA and if uplink CA requirements are written as proposed in [41,42], then no changes are needed.

Observation: The “SRS only” carrier requirements can be covered by uplink CA requirements.

## 2.2	UL CA with DMRS bundling

If UE is required to maintain power for the duration of the bundle, and there is a transmission scheduled during the bundle for another carrier, then UE may need to lower power during the bundle on a carrier with the bundle. UE therefore needs to have knowledge of the scheduling for both bands PUSCH preparation time before the transmission of the bundle for the whole duration of the bundle. This would put constraints on the network or UE behaviour would need to be defined when there is an interrupting transmission. However, if UE supports powerClassPerBand then it can be assumed UE does not need to share power and the band with the bundle is unaffected by the grants on another uplink. However, since RAN1 aspects may not be clear for this situation it maybe better left for future discussions. RAN4 can clarify with no changes to requirements in RAN4 or RAN1, that DMRS bundling is supported and current requirements apply for

- For inter-band UL CA no transmission is scheduled for other uplink carriers for the duration of the scheduled bundle

- TX switching SUL option

- SUL

Proposal 1: Ran4 to define DMRS bundling requirement applicability to cover inter-band UL CA when no transmission is scheduled for other uplink carriers for the duration of the scheduled bundle

We have provided draft CR [41, 42] to define this condition in ran4 specifications by clarifying current requirements applicability for the uplink CA case without concurrent transmissions on two bands.

## 2.3	The DMRS configuration for one carrier

The CR for FR1+FR2 case confines the applicability of the DMRS bundling to one band at a time with the text in TS 38.101-3:

“DMRS bundling requirements, as specified in clause 6.4.2.5 in TS 38.101-1 [2] and clause 6.4.2.6 in TS 38.101-2 [3], apply when one uplink band and CC is configured for DMRS bundling at a time. If UE needs to apply P-MPR as described in 6.2.4 of TS 38.101-1 or TS 38.101-2 during a granted DMRS bundle on any uplink CC, phase continuity is not expected to be maintained in the bundle”

The limitation highlighted is somewhat ambiguous of origin. It is not clear why only one band can be configured at a time if transmissions are not concurrent. The UE requirements and also behaviour should be based on granted transmissions and bundles.

Proposal 2: Change the condition for FR1+FR2 uplink CA to cover same case is proposed for FR1: when no transmission is scheduled for other uplink carriers for the duration of the scheduled bundle

We have provided a draft CR to remove the corresponding unnecessary limitations [43].

## 2.4	Capability aspects for CA

CA has been enabled with DMRS bundling, DL CA and UL CA between FR1 and FR2. This have been communicated to RAN1 in LS [36] but it is not clear the text in LS was received. We would like to emphasis that the capabilities should be per band per band combination as discussed and proposed in last meeting.

Proposal 3:

Add the following constraints to UE capability FGs 30-4a, 4b, 4c, and 4d (the four back-to-back cases):

Note: This capability is applicable in the following multi-carrier scenarios:

- FR1+FR2 CA/DC with one band on FR1 and another band on FR2. DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time.

- SUL

- FR1+FR1 Inter-band CA involving two or more FR1 bands where UE does not expect concurrent uplink transmissions on any two carriers

- All carriers belong to a single TAG

Proposal 4: UE capability FGs 30-4a, 4b, 4c, and 4d are signalled at per band and per band combination granularity

Proposal 5: Add the following constraint to 30-4h (non-back-to-back case): “Note: This capability is only applicable when UE is configured with single uplink carrier.”

# Conclusion

We made following observation:

Observation: The “SRS only” carrier requirements can be covered by uplink CA requirements.

And Following proposals:

Proposal 1: Ran4 to define DMRS bundling requirement applicability to cover inter-band UL CA when no transmission is scheduled for other uplink carriers for the duration of the scheduled bundle

Proposal 2: Change the condition for FR1+FR2 uplink CA to cover same case is proposed for FR1: when no transmission is scheduled for other uplink carriers for the duration of the scheduled bundle

Proposal 3:

Add the following constraints to UE capability FGs 30-4a, 4b, 4c, and 4d (the four back-to-back cases):

Note: This capability is applicable in the following multi-carrier scenarios:

- FR1+FR2 CA/DC with one band on FR1 and another band on FR2. DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time.

- SUL

- FR1+FR1 Inter-band CA involving two or more FR1 bands where UE does not expect concurrent uplink transmissions on any two carriers

- All carriers belong to a single TAG

Proposal 4: UE capability FGs 30-4a, 4b, 4c, and 4d are signalled at per band and per band combination granularity

Proposal 5: Add the following constraint to 30-4h (non-back-to-back case): “Note: This capability is only applicable when UE is configured with single uplink carrier.”

# References

[1]	R4-2100014, ”LS on PUCCH and PUSCH repetition”, RAN1, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[2]	R4-2103393, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[3]	R4-2103363, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[4]	R4-2105417, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#98-Bis-e

[5]	R4-2105418, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition” Huawei, RAN4#98-Bis-e

[6]	R4-2107611, “Reply LS on PUCCH and PUSCH repetition”, RAN1, RAN4#99e

[7]	R4-2107880, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#99e

[8]	R4-2107881, “WF on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, RAN4#99e

[9]	R4-2111194, “Reply LS to RAN1 latest question on phase discontinuity”, Ericsson, RAN4#99e

[10]	R4-2111706, “LS on joint channel estimation for PUSCH and PUCCH”, RAN1, RAN4#100-e, Online, Aug 2021

[11] 	R4-2114991, “Reply LS on PUCCH and PUSCH transmissions”, Qualcomm, RAN4#100-e, Online, Aug 2021

[12]	R4-2114992, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, HiSilicon, RAN4#100-e, Online, Aug 2021

[13]	R4-2117006, “Reply LS on PUCCH and PUSCH repetition”. RAN1, RAN4#101e, Nov 2021

[14]	R4-2120002, ”Reply LS on PUCCH and PUSCH transmissions”, RAN4 (Qualcomm Incorporated), 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[15]	R4-2120003, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[16]	R4-2119494, “UE requirement for phase continuity”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[17] R4-2200339, “draft CR for EVM based requirements”, Qualcomm, RAN4#101Bis-e, Jan, 2022

[18]	R4-2119495, “Simulation results and criterias for setting requriements for UE”, Qualcomm Incorporated, RAN4#101e, Nov 2021

[19]	R4-2202418, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[20]	R4-2202368, “Reply LS on Maximum duration for DMRS bundling” RAN4(Qualcomm), 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[21]	R4-2200020, “Updated RAN4 RF work plan for NR coverage enhancements WI”, China Telecom, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[22]	R4-2200338, “Discussion on UE requirement for JCE”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #101Bis-e, Jan 2022

[23]	R4-2206537, “Reply LS on Maximum duration for DMRS bundling”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[24]	R4-2206538, “CR on UE RF requirements for DMRS bundling in TS 38.101-1”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[25]	R4-2206539, “CR on UE RF requirements for DMRS bundling in TS 38.101-2”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[26]	R4-2206593, “WF on issues for maintenance of NR coverage enhancements”, Ericsson. 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[27]	R4-2206580, “Reply LS on Length of Maximum duration”, China Telecom, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[28]	R4-2206541, “CR on measurment for DMRS bundling in TS 38.101-1”, Ericsson, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[29]	R4-2206542, “CR on measurment for DMRS bundling in TS 38.101-2”, Ericsson, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[30]	R4-2206538, “CR on UE RF requirements for DMRS bundling in TS 38.101-1”, Huawei, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[31]	R4-2206539, “CR on UE RF requirements for DMRS bundling in TS 38.101-2”, Huawei, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[32]	RP-220931, “Status report for WI NR coverage enhancements; rapporteur: China Telecom”, RAN1, RAN#95e, March 2022

[33]	R4-2207659, “CR 38.101-1 DMRS for CA”, Qualcomm, RAN4#103e, May 2022

[34]	R4-2207658, Discussion on additional requirements for DMRS bundling, Qualcomm, RAN4#103e, May 2022

[35]	R4-2210548, “WF on DMRS bundling for CA, SUL and FR2-2”, China Telecom, 3GPP TSG-RAN WG4 Meeting # 103-e, Electronic Meeting, May 09 - May 20, 2022

[36]	R4-2211225, “Reply LS to RAN1/RAN2 on DMRS bundling”, MediaTek, 3GPP TSG-RAN WG4 Meeting #103-e, Electronic Meeting, May 09 - May 20, 2022

[37]	R4-2214041, “CR DMRS bundling for DL CA”, Qualcomm, RAN4#104e, Aug 2022

[38]	R4-2210215, “Handling of UL CA for DMRS bundling”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #103-e, Electronic Meeting, May 09 - May 20, 2022

[39]	RP-222592, “Moderator Summary Rel-17 CovEnh”, RAN1 chair(Samsung), RAN#97e, Online, Sept 2022

[40]	R4-2215305 (R1-2208212), “Reply LS to RAN4 on DMRS bundling”, RAN1(MTK), RAN4#104Bis-e, Oct 2022

[41]	R4-2215566, “CR for CA for 38.101-1”, Qualcomm, RAN4#104Bis-e, Oct 2022

[42]	R4-2215567, “CR for CA for 38.101-2”, Qualcomm, RAN4#104Bis-e, Oct 2022

[43]	R4-2215568,  “CR 38.101-3 DMRS config limitation removal”, Qualcomm, RAN4#104Bis-e, Oct 2022