3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215602

# Electronic Meeting, Oct 10 – Oct 19, 2022

Agenda item:	6.9.3

Source:	Apple

Title:	On RRM requirements for FR1-FR1 NR-DC

Document for:	Discussion

# Introduction

In RAN4 #104e meeting, RRM requirements for FR1-FR1 NR-DC scenarios has been discussed. The WF of eFeRRM has been agreed in [1], and the open issues were also captured.

## Sub-topic 3-1 General Issue 3-1-1: How to reflect the applicability of the existing requirement for FR1+FR1 NR-DC Agreement: - Rel-18 RRM requirements need to remove the clarification or limitation of only supporting FR1-FR2 NR-DC - FFS - Proposal 1: Apart from the requirements in R16/R17 mentioned in the WID, RAN4 to discuss the applicability rules of other R16/R17 requirements for FR1+FR1 NR-DC with/without changing on existing requirements. - Proposal 2: - RAN4 to firstly agree on the list of features that shall be included in the table1. - RAN4 to agree on the baseline requirement clause and formulation based on the agreed list of features that included in the table1. - RAN4 shall have separate applicability discussion of the Scheduling availability, CSSF impacted based on the agreed list of features that included in the table1. Issue 3-1-2: Other features with FR1+FR1 NR-DC Agreement: - The impact of features such as NR-U, MG-enhancement and Redcap are not expected unless the WI scope was extended.

In this paper, we further discuss the RRM requirements for FR1-FR1 NR-DC scenario.

# Discussion on FR1+FR1 NR-DC RRM

## Number of serving carriers

Issue 3-2-1: Number of serving carriers

The options we discussed in last meeting were:

Agreement: - RRM requirements for supported number of serving carriers for NR-DC need be updated for FR1-FR1 NR-DC according to RF specification. - FFS on exact number of serving carriers for NR-DC. - Option 1: For FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell and 1 UL in SCell. - Option 2: Exact number of serving carriers will be updated until the late phase of Rel-18 when RF spec is in more stable.

The existing RRM requirement for number of serving carriers are defined as following, and it is only specified for FR1+FR2 NR-DC.

#### 3.6.2.4 Number of serving carriers for NR-DC Requirements for NR-DC are applicable for the UE configured with the following number of serving NR CCs: - up to 2 NR DL CCs in total in FR1, up to 8 NR DL CCs in total in FR2, with 1 UL in PCell, 1 UL in PSCell, and up to 1 UL in each SCell.

In TS38.101-1-h60, if FR1+FR1 NR-DC is used, the max number of serving cell could be found in the BC of “DC_n48C-n96E” (it has 6 CCs in total based on following CA BW classes table). On the UL, 3 CCs are supported for combination for FR1-FR1 NR-DC in TS38.101-1-h60 (e.g., DC_n46A-n48B).

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 34 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 34 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 34 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |

The requirement for NR-DC serving CC number shall be revised to:

#### 3.6.2.4 Number of serving carriers for NR-DC Requirements for NR-DC are applicable for the UE configured with the following number of serving NR CCs: - For FR1+FR2 NR-DC, up to 2 NR DL CCs in total in FR1, up to 8 NR DL CCs in total in FR2, with 1 UL in PCell, 1 UL in PSCell, and up to 1 UL in each SCell. - For FR1+FR1 NR-DC, up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell, and 1 UL in SCell.

In last meeting, companies mentioned that the exact number of CC can be further updated when the R18 RF spec become stable. We are fine to update the CC number in late phase of R18 but we also support to have values specified in RRM requirement based on current RF spec.

Proposal 1: For FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell and 1 UL in SCell. These values can be further updated if R18 RF spec introduces greater CC number.

## PSCell addition/release delay and conditional PSCell addition delay

Issue 3-2-4: Interruption requirement for PSCell addition/release for FR1+FR1 NR-DC

The options we discussed in last meeting were:

- FFS - Option 1(Apple, Intel, Xiaomi, OPPO, MTK, QC): For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused. - Option 2(Nokia, Ericsson, vivo): NR-DC interruption requirements need to be updated to support FR1-FR1 NR-DC. - Option 2a(vivo): When defining the interruption requirements, for the number of SCells for FR1-FR1 NR-DC, up to 3 SCell(s) in FR1 of PCell and up to 3 SCell(s) in FR1 of PSCell are configured, deconfigured, activated or deactivated.

In the interruption requirement for NR PSCell addition, the existing requirement is generic enough to cover the FR1+FR1 NR-DC (somewhat like FR1+FR1 inter-band CA), so no need to change the existing interruption requirement for FR1 PSCell addition or release.

However, in section 8.2.4 of current TS38.133, the introduction part of NR-DC interruption is defined as:

### 8.2.4 NR-DC: Interruptions #### 8.2.4.1 Introduction This clause contains the requirements related to the interruptions on PCell, PSCell and activated SCell if configured, when up to 1 SCell in FR1 and up to 7 SCell(s) in FR2 are configured, deconfigured, activated or deactivated or, a supplementary UL carrier or an UL carrier is configured or de-configured, or measurements on SCC with deactivated SCell in NR SCG, or [unrelated part omitted]

Based on the proposal 1, for FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell and 1 UL in SCell; thus, FR1+FR1 NR-DC could have 1 DL PCell, 1 DL PSCell and up to 4 DL SCell in FR1.

Proposal 2: For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused.

Proposal 3:

For FR1+FR1 NR-DC, the introduction part of interruption requirement in section 8.2.4 of TS38.133 can be revised as:

- up to 1 SCell in FR1 and up to 7 SCell(s) in FR2 are configured, deconfigured, activated or deactivated when operating in FR1-FR2 NR-DC, or,

- up to 4 SCell(s) in FR1 are configured, deconfigured, activated or deactivated when operating in FR1-FR1 NR-DC

Issue 3-2-5: conditional PSCell addition delay requirement for FR1+FR1 NR-DC

The options we discussed in last meeting were:

- FFS - Option 1(Apple, Xiaomi, vivo, Intel, OPPO, MTK, QC, Nokia, HW, vivo): For FR1+FR1 NR-DC case, existing conditional PSCell addition requirement can be reused. (section 8.9A TS38.133) - Option 2 (Ericsson): existing Conditional Pscell addition requirement (section 8.9A.2 TS38.133) can be used as a baseline for starting the discussion - Option 2a: study the potential enhancement for TUE_preparation

Regarding the conditional PSCell addition, the existing requirement has already somehow covered FR1+FR1 NR-DC, as following,

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms when PSCell is in FR1, and Tprocessing = 40 ms when PSCell is in FR2.

However, some clarification is needed to say that “The requirements in this clause shall apply for the UE configured with only PCell in FR1.”

Proposal 4: For FR1+FR1 NR-DC case, existing conditional PSCell addition requirement can be reused. (section 8.9A TS38.133).

## Scheduling availability

Issue 3-2-8: Scheduling availability requirement for RLM/BFD/CBD/L1-RSRP in FR1+FR1 NR-DC

The options we discussed in last meeting were:

Agreement: - Scheduling restrictions for UE performing RLM/BFD/CBD/L1-RSRP in case of FR1-FR1 NR-DC needs to be clarified/updated in the specification. - FFS on option 1a and 2 and 3. - Option 1a(Apple, OPPO, Intel, Xiaomi, QC, vivo): For RLM/BFD/CBD/L1-RSRP, the existing scheduling availability requirement for FR1 inter-band CA is used for FR1-FR1 NR-DC scenario. - Option 2(MediaTek, QC): Add inter-band NR-DC scenario in scheduling availability for both FR1+FR1 and FR1+FR2. - Option 3(Nokia): Other solutions are not excluded.

The existing scheduling availability requirement are used in both L1 measurement (including RLM/BFD/CBD/L1-RSRP) and L3 measurement(including intra-/inter-freq measurement without MG).

Regarding the scheduling availability requirement for L1 measurement, the following requirement is specified in current spec:

#### 8.1.7.2 Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1 For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to radio link monitoring based on SSB as RLM -RS. - The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for radio link monitoring. When intra-band carrier aggregation in FR1 is performed, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols. When inter-band carrier aggregation within FR1 is performed, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to radio link monitoring performed on FR1 serving PCell or PSCell in different bands.

We think the FR1 inter-band CA case is very much similar as FR1-FR1 NR-DC case, and therefore we propose to use the same scheduling availability requirement from FR1 inter-band CA case.

Proposal 5: For RLM/BFD/CBD/L1-RSRP, the existing scheduling availability requirement for FR1 inter-band CA is used for FR1-FR1 NR-DC scenario.

Issue 3-2-9: Scheduling availability requirement for L3 measurement in FR1+FR1 NR-DC

The options we discussed in last meeting were:

FFS: - Option 1(Apple, OPPO, Intel, Xiaomi, MTK, HW): - For intra-frequency measurement without MG, - the existing scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case is used for FR1-FR1 NR-DC scenario. - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario. - For inter-frequency measurement without MG, - No need to introduce scheduling availability requirement of UE performing measurements in TDD bands for FR1-FR1 NR-DC scenario unless such requirement is introduced for FR1 inter-band CA. - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

Regarding the scheduling availability requirement for L3 measurement, the following requirement is specified in current spec:

##### 9.2.5.3.1 Scheduling availability of UE performing measurements in TDD bands on FR1 [omit irrelevant part] When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols. When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair. ##### 9.2.5.3.2 Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 [omit irrelevant part] When intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

In TS38.306, it specified that, the capability IE of simultaneousRxTxInterBandCA can also used for NR-DC case.

simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca- ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101- 2 [3] and TS 38.101-3 [4].

Thus, in intra-frequency measurement without MG, the same scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case can be used for FR1-FR1 NR-DC scenario. Moreover, existing scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 didn’t have a case of FR1 inter-band CA, and therefore there is also no need to have it for FR1-FR1 NR-DC case.

For inter-frequency measurement without MG, no scheduling availability requirement is specified for FR1 inter-band CA, and we think we don’t need to specify scheduling availability requirement for FR1-FR1 NR-DC either unless such requirement is introduced for FR1 inter-band CA.

Proposal 6:

For intra-frequency measurement without MG,

- the existing scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case is used for FR1-FR1 NR-DC scenario.

- No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

For inter-frequency measurement without MG,

- No need to introduce scheduling availability requirement of UE performing measurements in TDD bands for FR1-FR1 NR-DC scenario unless such requirement is introduced for FR1 inter-band CA.

- No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

## Measurement restriction

Issue 3-2-10: Measurement restriction in FR1+FR1 NR-DC

The options we discussed in last meeting were:

FFS: - Option 1(MediaTek): Add inter-band NR-DC scenario in measurement restriction for both FR1+FR1 and FR1+FR2.

In our view the current measurement restriction is quite general for all cases, and therefore we think the existing measurement restriction requirement can also be used for FR1+FR1 NR-DC.

Proposal 7: the existing measurement restriction requirement can be reused for FR1-FR1 NR-DC scenario.

## CSSF

Issue 3-2-11: CSSF outside gap for FR1+FR1 NR-DC

The options we discussed in last meeting were:

Agreement: - For CSSF, add the scenario of FR1 PCell + FR1 PSCell in CSSFoutside_gap,i. - FFS on option 1a and 2. - Option 1a(Apple, Xiaomi, OPPO, vivo, MTK, Ericsson, HW): CSSF outside gap for FR1+FR1 NR-DC is: Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR1 PSCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap. FR1 + FR1 NR-DC (FR1 PCell and FR1 PScell); 1+NPCC_CSIRS; 2×( NSCC_SSB +Y+2xNSCC_CSIRS); 2x(1+ NPSCC_CSIRS) Note 2; N/A; N/A; 2x(NSCC_SSB +Y+2x NSCC_CSIRS ). - Option 2(Intel, QC): For FR1+FR1 NR-DC, further discuss whether similar rule to derive CSSFoutside_gap and CSSFwithin_gap based on FR1+FR2 NR DC can be re-used.

The existing RRM requirement for CSSF outside MG of NR-DC is specified as following:

Table 9.1.5.1.3-1: CSSFoutside_gap,i scaling factor for NR-DC mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PSCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap. FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) Note 1; 1+NPCC_CSIRS; 2×( NSCC_SSB +Y+2xNSCC_CSIRS); 2x(1+ NPSCC_CSIRS) Note 2; 2x(NSCC_SSB +Y+2x NSCC_CSIRS ); 2x(NSCC_SSB +Y+2x NSCC_CSIRS ).

For FR1 PSCC measurement, we also think the 2nd searcher shall be used for it but it needs to share the 2nd searcher with other FR1 SCCs in both MCG and SCG. The usage of 2nd searcher for PSCC can be 50%, and the SCCs and inter-frequency layer without MG in both MCG and SCG share the other 50% of the 2nd searcher. PCC always use the 1st searcher as same as FR1+FR2 NR-DC.

Proposal 8: For FR1-FR1 NR-DC scenario, FR1 PCC use 1st searcher, FR1 PSCC uses 50% of the 2nd searcher, and all the SCCs and inter-frequency layer without MG in both MCG and SCG use the other 50% of the 2nd searcher. The CSSF outside MG table is proposed as following.

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR1 NR-DC (FR1 PCell and FR1 PScell) | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | 2x(1+ NPSCC_CSIRS) Note 2 | N/A | N/A | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |

## HO with PSCell

The options we discussed in last meeting were:

Issue 3-2-13: HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR1 NR-DC) FFS: - Option 1(Apple, Xiaomi, OPPO, vivo, Intel, MTK, QC, HW, vivo): the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused, i.e., for HO with PSCell requirement, Tprocessing= 30 ms for sequential case and Tprocessing= 25 ms for parallel case should be added for FR1-FR1 NR-DC. - Option 2(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC Issue 3-2-14: HO with PSCell for FR1+FR1 NR-DC (FR1-FR2 NR-DC to FR1-FR1 NR-DC) FFS: - Option 1(Apple, OPPO, Intel, Xiaomi, QC), HW: the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused - Option 2 (vivo, MTK, QC): Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms. - Option 3(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC Issue 3-2-15: HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR2 NR-DC) FFS: - Option 1(Apple, OPPO, vivo, Intel, Xiaomi, MTK, QC, HW): - Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms. - Option 2(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC

The existing RRM requirement for HO with PSCell only cover the case for HO from FR1-FR2 NR-DC to FR1-FR2 NR-DC. The missing cases of NR-DC can be:

- HO with PSCell from FR1-FR1 NR-DC to FR1-FR1 NR-DC

- HO with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC

- HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC

- HO with PSCell from FR1-FR2 NR-DC to FR1-FR2 NR-DC (existing requirement)

Since we have proposal 2 for FR1-FR1 NR-DC PSCell addition and the HO with PSCell requirement for NR-DC to NR-DC will refer to section 8.9.2, there is no need to have any change to cover the HO with PSCell for FR1-FR1 NR-DC to FR1-FR1 NR-DC. Additionally, such requirement can also cover the HO with PSCell for FR1-FR2 NR-DC to FR1-FR1 NR-DC (the source NR-DC has both FR1 and FR2 serving CCs), and the processing time is Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 25 ms.

However, for HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC, the processing time in PSCell change part shall be revised since the old NR-DC mode has only FR1 serving CCs but new NR-DC mode needs to have FR2 PSCC. For HO with PSCell for FR1-FR1 NR-DC to FR1-FR2 NR-DC, we think the processing time is Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms.

Proposal 9:

For HO with PSCell from ‘FR1-FR1 NR-DC to FR1-FR1 NR-DC’ and ‘FR1-FR2 NR-DC to FR1-FR1 NR-DC’, the existing requirement in section 6.1.5.4 TS38.133 can be reused.

For HO with PSCell from ‘FR1-FR1 NR-DC to FR1-FR2 NR-DC’, the processing time in existing PSCell change delay requirement in section 6.1.5.4.2 TS38.133 shall be defined as following:

- Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms.

## SCG activation/deactivation

Issue 3-2-16: RACH based PSCell activation for FR1+FR1 NR-DC

The options we discussed in last meeting were:

Agreements: - SCG activation/deactivation requirements need to be updated to support FR1-FR1 NR-DC - FFS on option 1a and 1b - Option 1a(Apple, Xiaomi, OPPO, MediaTek, vivo, Intel, Ericsson, HW): if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms. - Option 1b(Ericsson): further study the potential enhancement in T∆

The existing RRM requirement for SCG activation is defined as following, and it is only specified for FR1+FR2 NR-DC.

### 8.17.2 SCG Activation Delay Requirement The requirements in this clause shall apply for the UE configured with one deactivated SCG in NR-DC and when PScell in one SCG is being activated. The delay within which the UE shall be able to activate the deactivated SCG depends upon the specified conditions. Upon receiving SCG activation command in slot n, the UE shall be capable to transmit PRACH preamble or PUCCH towards PSCell no later than in slot n+Tactivation_timeNR slot length , where: Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. When PSCell is activated from deactivated state, if any PSCell parameter is modified, Tprocessing = [20ms]. Otherwise, Tprocessing = [5 or 10ms]. Tsearch is the time for AGC settling and PSS/SSS detection. For RACH based PSCell activation, if the target cell is a known NR FR2 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR2 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. For RACH-less based PSCell activation, if RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR2 PScell. There are no requirements if PSCell is unknown. T∆ is time for fine time tracking and acquiring full timing information of the target PSCell. T∆ = 1*Trs ms. TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3]. When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH]. Trs is the SMTC periodicity of the PSCell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 In FR2, the PSCell is known if it has been meeting the following conditions: - During the last 5 seconds before the reception of the SCG activation command: - the UE has sent a valid measurement report for the PSCell being activated and - One of the SSBs measured from the PSCell being activated remains detectable according to the cell identification conditions specified in clause 9.3. - One of the SSBs measured from PSCell being activated also remains detectable during the PSCell activation delay Tconfig_PSCell according to the cell identification conditions specified in clause 9.3. otherwise it is unknown. If the UE is configured to perform BFD while the SCG is deactivated - The TCI state is known if the following conditions are met: - During the period from the PSCell deactivation to the completion of PSCell activation, while PSCell was deactivated, - UE has not detected beam failure - Otherwise, the TCI state is unknown. The PCell interruption specified in clause 8.2 is allowed only during the RRC reconfiguration procedure [2].

For SCG activation in FR1+FR1 NR-DC, the processing time can be same as for FR1+FR2 NR-DC (FR1-FR1 NR-DC processing shall not be longer than FR1+FR2 NR-DC case in our view).

Similar as FR1 PSCell addition in proposal 2, for RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms. For RACH-less based PSCell activation, the existing requirement can also apply for FR1+FR1 NR-DC case (no difference was observed).

Proposal 10: For SCG activation delay requirement in FR1+FR1 NR-DC, the following changes on existing requirement shall be added,

- for RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

Issue 3-2-17: Known condition for PSCell activation for FR1+FR1 NR-DC

The options we discussed in last meeting were:

FFS: - Option 1(Apple, Intel, Xiaomi, OPPO, MTK, QC, HW, vivo): In FR1, the PSCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

For the known/unknown PSCell condition, since Rx beam doesn’t need to consider for FR1 PSCell case (no need to limit the side condition as one specific SSB is detectable), the known condition from FR1 HO can be reused, that is: in FR1, the PSCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

Proposal 11: For SCG activation delay requirement in FR1+FR1 NR-DC, the PSCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

# Conclusion

In this paper, we further discuss the RRM requirements for FR1-FR1 NR-DC scenario.

Proposal 1: For FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell and 1 UL in SCell. These values can be further updated if R18 RF spec introduces greater CC number.

Proposal 2: For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused.

Proposal 3:

For FR1+FR1 NR-DC, the introduction part of interruption requirement in section 8.2.4 of TS38.133 can be revised as:

- up to 1 SCell in FR1 and up to 7 SCell(s) in FR2 are configured, deconfigured, activated or deactivated when operating in FR1-FR2 NR-DC, or,

- up to 4 SCell(s) in FR1 are configured, deconfigured, activated or deactivated when operating in FR1-FR1 NR-DC

Proposal 4: For FR1+FR1 NR-DC case, existing conditional PSCell addition requirement can be reused. (section 8.9A TS38.133).

Proposal 5: For RLM/BFD/CBD/L1-RSRP, the existing scheduling availability requirement for FR1 inter-band CA is used for FR1-FR1 NR-DC scenario.

Proposal 6:

For intra-frequency measurement without MG,

- the existing scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case is used for FR1-FR1 NR-DC scenario.

- No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

For inter-frequency measurement without MG,

- No need to introduce scheduling availability requirement of UE performing measurements in TDD bands for FR1-FR1 NR-DC scenario unless such requirement is introduced for FR1 inter-band CA.

- No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

Proposal 7: the existing measurement restriction requirement can be reused for FR1-FR1 NR-DC scenario.

Proposal 8: For FR1-FR1 NR-DC scenario, FR1 PCC use 1st searcher, FR1 PSCC uses 50% of the 2nd searcher, and all the SCCs and inter-frequency layer without MG in both MCG and SCG use the other 50% of the 2nd searcher. The CSSF outside MG table is proposed as following.

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR1 NR-DC (FR1 PCell and FR1 PScell) | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | 2x(1+ NPSCC_CSIRS) Note 2 | N/A | N/A | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |

Proposal 9:

For HO with PSCell from ‘FR1-FR1 NR-DC to FR1-FR1 NR-DC’ and ‘FR1-FR2 NR-DC to FR1-FR1 NR-DC’, the existing requirement in section 6.1.5.4 TS38.133 can be reused.

For HO with PSCell from ‘FR1-FR1 NR-DC to FR1-FR2 NR-DC’, the processing time in existing PSCell change delay requirement in section 6.1.5.4.2 TS38.133 shall be defined as following:

- Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms.

Proposal 10: For SCG activation delay requirement in FR1+FR1 NR-DC, the following changes on existing requirement shall be added,

- for RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

Proposal 11: For SCG activation delay requirement in FR1+FR1 NR-DC, the PSCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

# References

[1] R4-2214345, WF on R18 eFeRRM, Apple, RAN4 #104e