3GPP TSG-RAN WG4 Meeting # 104bis-e												   R4-2215610

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.10.2

Source:	Apple

Title:	On R18 gap enhancement - joint configuration of Pre-MG, NCSG and concurrent gaps

Document for:	Discussion

# Introduction

RAN4 had initial discussion on improvement on R18 gap enhancement - joint configuration of Pre-MG, NCSG and concurrent gaps with agreements captured in [1]. In this contribution, we continue discussion with focus on the agreements and way forward approved in [1].

# Discussion

The first issue is about scope and scenario:

## Sub-topic 2-1: Scope and scenarios ### Issue 2-1: Whether to define requirements for one Pre-configured MG and one NCSG < Way forward >: - RAN4 to clarify whether one Pre-configured MG and one NCSG is included in Case 1 and/or Case 2 - FFS to explicitly set the priority of different combinations ### Issue 2-2: Whether to define requirements for one Pre-configured NCSG < Agreement >: - RAN4 focuses on Case 1 and Case 2 first. RAN4 can continue discuss whether and when to discuss requirements for Pre-configured NCSG ### Issue 2-3: NR SA and MR-DC < Way forward >: - FFS whether to deprioritize MR-DC in Rel-18 ### Issue 2-4: 2G and 3G < Agreement >: - Deprioritize 2G/3G measurements in Rel-18 MGE WI ### Issue 2-5: MUSIM < Agreement >: - Concurrent MUSIM gaps and other existing measurement gap types is not in the scope of this WI ### Issue 2-6: MAC signalling for activation/deactivation/reconfiguration < Agreement >: - Applying MAC signaling for activation/deactivation/reconfiguration for non-positioning measurements via Pre-MG is out of scope of this WI ### Issue 2-7: Legacy measurement gaps < Way forward >: - Proponents are encouraged to clarify the proposal “consider legacy MG, Pre-MG, concurrent MG, and NCSG when we discuss joint MG requirements.” in the next meeting. ### Issue 2-8: Positioning measurement gaps < Agreement >: - Rel-17 MGE Pre-MG for RRC-based POS is in the scope of this WI < Way forward >: - FFS Rel-17 MAC-CE based ePOS gap is in or out of scope of this WI

The scope and scenario have been extensively discussed in the previous RANP#97e. Eventually an updated WID was approved in [2]. The updated objective is duplicated here for information:

- Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Prioritize at least joint requirements for UE configured with - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: Gaps that are configured for NTN are precluded in Case 1 and Case 2 - Note 2: The requirement discussions on the scenarios that NCSG is considered in Case 1 and that Pre-configured MG is considered in Case 2 will be started after RAN#99. - Note 3: Prioritization among other possible combinations of pre-configured MG, concurrent MG, NTN gaps and NCSG can be discussed after RAN#99 - Note 4: This WID does not include any inter-working with MUSIM gaps

With above update, RAN4 doesn’t need to discuss the following issue 2-1 in RAN4#104e-bis according to the note 2 and 3 above.

- Issue 2-1: whether to define requirements for one Pre-configured MG and one NCSG

- Issue 2-2: Whether to define requirements for one Pre-configured NCSG

One thing we would like to highlight is that Pre-MG + Pre-MG and NCSG + NCSG are literally in scope of case 1 and 2 (before and after RAN#97e). We provided some examples in previous RAN4 meeting on the possible use case [3]

Example of Pre-MG + Pre-MG

Figure 2 Pre-MG + Pre-MG

Since mobility control is handled on PCC, it is quite typical for NW to configure relatively short SMTC such as 20ms or 40ms. However, NW may choose to configure longer SMTC on SCC since mobility on SCC is less sensitive compared to PCC. In the above example, SMTC on PCC is 20ms while SMTC on SCC is 40ms. Due to load balance or power saving and other reason, NW may configure multiple BWP on both PCell and SCell. When PCell is working BWP1, UE can measure SSB on PCC w/o gap. When PCell is working on BWP2, UE needs MG1 to measure SSB on PCC. Similar situation on SCC. Sometimes gap is needed and sometimes gap is not needed to measure SSB on SCC. Actually R17 Pre-MG in CA already support this case, i.e. UE can use MG1 to measure SSB. However, if UE can use another MGP with longer MGRP and shorter MGL to measure SSB on SCC when MG1 is not activated (BWP1 in PCell is active), system throughput can be increased. Because MG2 in green blocks has less overhead than MG1 in grey blocks.

Example of NCSG + NCSG

If UE still has another spare RF chain to cover band B, UE can still use NCSG to measure band B. It is beneficial for NW to configure another NCSG for measuring band B, especially when SMTC on band A and B are not aligned. For instance:

Figure 4 NCSG + NCSG

Proposal 1: Pre-MG + Pre-MG is considered in case 1.

Proposal 2: NCSG + NCSG is considered in case 2.

The next issue is about whether to deprioritize MR-DC.

Issue 2-3: NR SA and MR-DC

< Way forward >:

- FFS whether to deprioritize MR-DC in Rel-18

One of the challenges to support MR-DC is on existing information exchange over Xn interface. Take Pre-MG for example, the ON/OFF status of Pre-MG can be dynamically changed (in DCI or MAC level). It is challenging for MN and SN to exchange information in such a dynamical level. Even for concurrent gaps, once MN or SN updates the association between MO and certain gap pattern, it may need to inform the other node such that both MN and SN can know how many carriers are expected to be measured within each gap patterns. This would increase Xn signalling overhead.

Proposal 3: deprioritize MR-DC in R18 for joint configuration of Pre-MG, NCSG and concurrent gaps.

Another open use case is R17 MAC-CE based ePOS gap.

Issue 2-8: Positioning measurement gaps

< Agreement >:

- Rel-17 MGE Pre-MG for RRC-based POS is in the scope of this WI

< Way forward >:

- FFS Rel-17 MAC-CE based ePOS gap is in or out of scope of this WI

Similar with Pre-MG, status of ePOS can also be dynamically changed between ON and OFF. However, from functionality point of view, the intention of ePOS gap is for positioning, rather than legacy RRM measurement. Therefore, we consider it as equal priority as NTN gap and MUSIM gaps. It has been agreed that MUSIM gaps related enhancement will be covered by dedicated WI for MUSIM. According to the new note 4 in the WID [2], NTN gaps will be discussed after RAN#99. We propose to revisit ePOS gap after RAN#99 as well.

Proposal 4: feasibility of joint configuration of R17 MAC-CE based ePOS gap and other gaps can be discussed after RAN#99.

The second topic is how to inherit R17 framework.

## Sub-topic 2-2: Inherit Rel-17 framework ### Issue 2-9: Which Rel-17 Pre-MG principle(s) should be revisited < Way forward >: - FFS: For UE autonomous mechanism, only the measurements associated to the concerned pre-MG are used for the rule checking - FFS: For Network-controlled mechanism, only the bits corresponding to the concerned pre-MG are used for determining the status - FFS whether to consider the other potential enhancement ### Issue 2-10: Which Rel-17 concurrent gap principle(s) should be revisited < Way forward >: - FFS which Rel-17 concurrent gap principle(s) should be revisited

For case 1, UE is expected to be configured with at least one Pre-MG, together with another Pre-MG or a legacy gap. To make UE measurement behaviour clear, we expect NW to provide association between MOs and each gap pattern. With this assumption, the status of each Pre-MG can be determined based on the MOs which are associated to the concerned Pre-MG, covering both network-controlled mechanism and UE autonomous mechanism.

Note that association between MOs and NCSG in case 2 is also beneficial.

Proposal 5: when UE is configured with multiple gap patterns (including case 1, 2 and other possible combination), network shall provide clear association between MOs and each gap pattern.

Regarding issue 2-10, which R17 concurrent gap principle(s) should be revisited. We think most principle can be reused, including association mechanism between MO and gap pattern, framework of collision between multiple gap patterns (including proximity condition) and even the number of gaps (gap combination configurations defined in Table 9.1.8-1)

Proposal 6: the following concurrent gap principles can be reused for joint configuration in R18

- Association mechanism between MO and gap pattern

- Framework of collision between multiple gap patterns

- Number of gap combination configuration (similar with TS38.133 table 9.1.8-1)

The third topic captured in the WF [2] is about max number of supported gaps.

## Sub-topic 2-3: Max number of supported gaps ### Issue 2-11: Max # of gaps for Case 1 (Pre-configured MG and multiple concurrent MGs) < Agreement >: - For the max number of gaps for Case 1 (Pre-configured MG and multiple concurrent MGs), the Rel-17 conclusions will be taken as the baseline. < Way forward >: - FFS whether to increase the max number - FFS whether to introduce new UE capability for # of supported Pre-MG ### Issue 2-12: Max # of gaps for Case 2 (NCSG and multiple concurrent MGs) < Agreement >: - For the max number of gaps for Case 2 (NCSG and multiple concurrent MGs), the Rel-17 conclusions will be taken as the baseline. < Way forward >: - FFS whether to increase the max number - FFS whether to introduce new UE capability for # of supported NCSG ### Issue 2-13: Detail combinations < Way forward >: - Companies are encouraged to bring high-level principle about the possible gap combinations before going into details

We suggest starting from same assumption in R17 concurrent gaps design, as captured in TS38.133 table 9.1.8-1, as captured in proposal 6 above.

The next topic is on collision definition and handling.

## Sub-topic 2-4: Collision definition and handling ### Issue 2-14: Required changes for Pre-MG on collision < Agreement >: - For Case 1 (Pre-configured MG and multiple concurrent MGs), the baseline requirement considers collisions on Pre-MG is only considered when Pre-MG is activated. < Way forward >: - FFS whether Pre-MG priority can be further decided by the associated Mos being measured or other signalling parameters ### Issue 2-15: Required changes for NCSG on collision < Agreement >: - When determining if a collision occurs between a NCSG instance and another gap instance, the baseline requirement considers the total NCSG duration, including both the VILs and the ML. < Way forward >: - FFS other potential enhancements regarding spare RF chains, interruption due to collision on RRT, revised proximity condition, … etc. ### Issue 2-16: general gap collision handling < Agreement >: - On gap collision handling, take priority rule and overlapping rules from Rel 17 concurrent gaps as the baseline < Way forward >: - FFS whether to consider gap sharing rule

Definition of collision in case 1 and 2 have been agreed under issue 2-14 and 2-15. We are open for further enhancement, if justified.

As for how to handle collision, it has been agreed to take priority rule and overlapping rules from R17 as baseline and further study gap sharing rule. We support considering gap sharing rule. Actually, it has already been supported in R17 NTN gaps design. Since RAN4 may also discuss joint configuration of NTN gap and other gap (after RAN#99), it may cause compatibility issue if gap sharing mechanism is not supported in joint configuration. Besides, as more and more gap patterns can be jointly configured in future, it may be more challenging for network to clearly determine priority for each gap pattern. One of the possible scenarios to configure some gap patterns with equal priority. In this scenario, gap sharing rule is beneficial.

Proposal 7: RAN4 can further consider gap sharing rule to handle gap collision after priority based solution is stable (e.g. after RAN#99).

The last topic is on RRM requirements

## Sub-topic 2-5: Measurement delay requirements ### Issue 2-17: Required changes for Pre-MG on related delays < Way forward >: - FFS whether the measurement requirements with concurrent MGs defined in Rel-17 can be reused except that only activated gaps are considered when defining CSSF - FFS whether the pre-configured gap activation delay requirements need to be extended, when the activation procedures of multiple pre-configured gaps overlap. ### Issue 2-18: Required changes for NCSG on measurement delay < Way forward >: - FFS any change for the NCSG measurement reporting delay requirements is needed ### Issue 2-19: Available IDLE RF chain in measurement < Way forward >: - Proponents are encouraged to clarify the proposal “requirements need to be distinguished if the UE has at least one idle RF chain available or not under NCSG + NCSG case.”

Starting from Pre-MG, we think most of existing requirements can be reused. For instance, let’s consider Pre-MG plus one legacy gap. When the Pre-MG is activated, it has no difference compared with concurrent gaps. Once the Pre-MG is deactivated, existing requirements for single gap apply.

Proposal 8: for case 1, measurement requirements with concurrent MGs defined in Rel-17 can be reused except that only activated gaps are considered when defining CSSF.

As for NCSG, so far we don’t observe any necessary change on NCSG measurement reporting delay requirements. We are open for further discussion, if any issue identified.

# Conclusion

In this contribution, we continue discussion on joint configuration of Pre-MG, NCSG and concurrent gaps. After discussion, the following conclusions are provided:

Proposal 1: Pre-MG + Pre-MG is considered in case 1.

Proposal 2: NCSG + NCSG is considered in case 2.

Proposal 3: deprioritize MR-DC in R18 for joint configuration of Pre-MG, NCSG and concurrent gaps.

Proposal 4: feasibility of joint configuration of R17 MAC-CE based ePOS gap and other gaps can be discussed after RAN#99.

Proposal 5: when UE is configured with multiple gap patterns (including case 1, 2 and other possible combination), network shall provide clear association between MOs and each gap pattern.

Proposal 6: the following concurrent gap principles can be reused for joint configuration in R18.

Proposal 7: RAN4 can further consider gap sharing rule to handle gap collision after priority based solution is stable (e.g. after RAN#99).

Proposal 8: for case 1, measurement requirements with concurrent MGs defined in Rel-17 can be reused except that only activated gaps are considered when defining CSSF.

# References

- R4-2214346, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek Inc.

- RP-222337, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation

- R4-2211907, On R18 pre-configured MGs, multiple concurrent MGs and NCSG, Apple