3GPP TSG-RAN WG4 Meeting # 104bis-e                               R4-2215629

Electronic Meeting, 10th – 19th August 2022

Source: 	Apple

Title: 	Further consideration on intra-band non-collocated CA/EN-DC

Agenda Item:	6.1.1.2

Document for:	Discussion

# Introduction

- Discuss reference UE architecture and RF impacts due to the power imbalance

- Agree on the reference UE architecture at least for 2-layer MIMO case for NR-CA

- Discuss 2-layer MIMO case for NR-CA

- Start initial discussion of 4-layer MIMO case for EN-DC and NR-CA

There were extensive discussion [1] on intra-band non-collocated CA/EN-DC and the workplan and WF [2] have been approved in the last RAN4 meeting. It is planned to have the following discussion in RAN4#104bis-e.

This contribution will further discuss the open issue for 2MIMO layer per band and start to discuss the case 4MIMO layer per band.

# Discussion

## 2 MIMO layer per band

UE reference architecture

< Issue 2-2-1 (a): UE RF architecture>

- Total four RF antenna is assumed.

- Reuse UE RF architecture of inter-band non-contiguous DC_42_n77/78 EN-DC Type-2 (i.e. 2 layer/2 Rx Chain per CC, total 4 Rx Chain) as the baseline.

< Issue 2-2-1 (b): Meaning of number of Rx Chain>

- Clarify meaning of number of Rx Chain in the case of total 4 Rx Chain as follows

  - Antenna/RF paths as baseline

  - Further clarify in next meeting with consideration on other aspect, such as BB

The following are agreed for the case of 2 MIMO layer per band. Further clarifications are needed for the meaning of number of Rx chain with regard to the understanding on BB part.

Corresponding to 25dB power imbalance, the time difference from 2 non-collocated CCs that UE need to support is 4.6us. This already exceeds the CP length for 30kHz SCS which is the main interest SCS for band n77 and n78. From timing perspective, separate FFT will be needed to handle signals from the 2 CCs. It thus means the UE RF architecture should separate the reception on 2 CCs from RF to baseband.

Proposal: It is proposed to adopt the UE architecture for DC_42_n77/78 EN-DC type 2 for NR CA type 2 UE supporting 2 MIMO layer per CC.

- i.e. 2 layer/2Rx Chain per CC, total 4 Rx Chain with separate FFT for each Rx chain

It has been discussed [3] and agreed in [2] to introduce signaling to differentiate 2 type of UE, where type 1 is the current UE implementation with maximum 6dB power imbalance and type 2 UE is the UE supporting 25dB power imbalance. The signaling design could be as shown in the table below. After we concluded all the requirement this can be sent to RAN2.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandNonColocatedCADL-r18 Indicates the UE supports FDD-FDD or TDD-TDD intra non-colocated CA operation with additional REFSENS requirement in xx in 38.101-1, and MRTD <=33us according to 7.6.4 in 38.133. (i.e. Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD intra/inter-band operation with NR CA MRTD<3 us according to 7.6.4 in 38.133 [5] (i.e. Type 1 UE). | BC | No | N/A | FR1 only |

Impact on time difference

On top of proposal 1, the hardware capability for intra-band non-collocated CA is similar as the case for inter-band CA. As such, it allows UE to handling larger time difference between CCs although the needed MRTD corresponding to 25dB power imbalance is 4.6us. currently the MRTD requirement for CA is summarized as below. It’s feasible to either reuse the 33 us MRTD for intra-band non-collocated case assuming the architecture in proposal 1.

| Scenario | Maximum receive timing difference (µs) | Note |
| --- | --- | --- |
| Intra-band CA | 3 | Collocated case |
| Inter-band CA | 33us |  |

Observation 1: Based on the reference architecture in proposal 1, it is feasible to reuse 33us requirement.

Proposal 2: The exact value (< =33us) for MRTD requirement can be further discussed and confirmed in RRM session.

On “Async CA” scenario

In the last meeting the scenario of Async CA was discussed but leave as FFS. We think mentioning “async CA” is misleading since the slot boundary should be aligned at least for CA deployment even though the starting of the frame might not be aligned. Further looking at the RRM spec, we only specified requirement for synchronized case even for the inter-band CA case. E.g. 33 us is composed of 33 propagation delay difference and 3 us cell phase synchronization at gNB. Given all the relevant bands are TDD bands and synchronization should be considered. We don’s think there is any value to further discuss this scenario. The discussion on “Async CA scenario” shall not be pursued in this WI.

Proposal 3: “Async CA” is misleading concept. No further discussion should be pursued on this.

## 4 MIMO layer

Depending on the number of layers supported in each CC, the following cases are relevant for 4MIMO layer cases,

For EN-DC,

- 2MIMO layer in Band 42 + 4 MIMO layer in Band n77/n78

- 4MIMO layer in Band 42 + 2 MIMO layer in Band n77/n78

- 4MIMO layer in Band 42 + 4 MIMO layer in Band n77/n78

For intra-band non-collocated CA,

- 2MIMO layer in Band 77/78 + 4 MIMO layer in Band n77/n78

- 4MIMO layer in Band 77/78 + 2 MIMO layer in Band n77/n78

- 4MIMO layer in Band 77/78 + 4 MIMO layer in Band n77/n78

It is noted that power imbalance is defined between CCs rather than between layers. The power imbalance at UE will not change for 4 MIMO layer and 2 MIMO layer from a deployment point of view. The question left is whether it is feasible from UE implementation point of view and whether we need to develop requirement for >2 MIMO layer based on 25dB power imbalance. Given the discussion in section 2 and [4], under 25dB power imbalance, the increasing of MIMO layer on each CC just means increasing of the number of Rx chains. Our view is that it is not feasible due to the limitation by form factor and cost.

Proposal: 4MIMO layer per CC with 25dB power imbalance is not feasible.

Further check if the power imbalance could be reduced to reasonable level for 4MIMO layer case.

# Summary

This contribution presented our views on the open issue for intra-band non-collocated EN-DC/CA. The following observations and proposals are concluded.

Proposal: It is proposed to adopt the UE architecture for DC_42_n77/78 EN-DC type 2 for NR CA type 2 UE supporting 2 MIMO layer per CC.

- i.e. 2 layer/2Rx Chain per CC, total 4 Rx Chain with separate FFT for each Rx chain

Observation 1: Based on the reference architecture in proposal 1, it is feasible to reuse 33us requirement.

Proposal 2: The exact value (< =33us) to be specified for MRTD requirement can be further discussed in RRM session.

Proposal 3: “Async CA” is misleading concept. No further discussion should be pursued on this.

Proposal 4: 4MIMO layer per CC with 25dB power imbalance is not feasible.

# References

[1] RP-2214246, Email discussion summary for [104-e] [135] NonCol_intraB, KDDI

[2] R4-2214458, WF on Support of intra-band non-collocated EN-DC/NR-CA deployment, KDDI

[3] R4-2212098, Architecture impact and options for non-collocated DC_42_n77, Skyworks solutions

[4] R4-2211914, On intra-band non-colocation EN-DC/CA, Apple