3GPP TSG-RAN WG4 Meeting # 104bis-e                               R4-2215634

Electronic Meeting, 10th – 19th August 2022

Source: 	Apple

Title: 	Further discussion on ATG UE requirements

Agenda Item:	6.13.3

Document for:	Discussion

# Introduction

The following issues for ATG UE requirement were discussed in RAN4#104-e meeting [1] but no consensus has been reached. A WF was agreed in [2].

  - Doppler/timing pre-compensation assumption

  - Power class for ATG UE

  - UE requirement type

  - power control

  - power control accuracy

This contribution will present our views for these highlighted issues.

# Discussion

## Doppler/timing pre-compensation

The high moving speed of ATG UE is up to 1200km/h. It will cause up to more than 2*3k doppler frequency shift when UE doing initial access at the cell edge. This may cause not negligible impact. The UE may need to do some pre-compensation for the Tx frequency.

As discussed in the last RAN4 meeting [1], to do the pre-compensation the UE need to know the ATG UE location information from system information. This may need to involvement of other working groups. At least we need to check with RAN2 whether the NTN signaling developed in Rel-17 is applicable to ATG.

Proposal 1: UE frequency pre-compensation for initial access is needed RAN2 need to be involved in the WI.

## Power class for ATG UE

The following options were discussed and need further discussion in this meeting.

  - Option1: A separate UE power class is defined for ATG UE

  - Option2: not to define the power class for ATG UE similar as IAB-MT

  - Option3: Consider further whether to set a power level in the requirement, or set a requirement on power accuracy with a declared power (subject to a maximum limit)

Although there are different options to define the UE output power, it seems common understanding that the ATG UE power level will be a difference one from the existing power class. The question then is whether we need to define it and we need to fix it in the standard now.

Regarding option 2, we don’t think it is workable approach for ATG UE both from network scheduling and from interference point of view. management. IAB is fixed in location while ATG UE is still a moving device. The required power might be quite different depending on the type of aircraft it is mounted. The network need to know the UE Tx power for UL scheduling. We think either option 1 or option 3 could be considered.

If we can conclude one or more value on the maximum output power depending different aircraft or different altitude, Option 1 is preferred. It can be a set of maximum output power indicated to the network.

If we cannot conclude the maximum output power now, option 3 is preferred. It can be defined according to the custom demand. But the UE maximum power still needs to be reported to the network by signaling since it is a UE device rather than a network node. The benefit of this approach is the MPR/A-MPR requirement is not needed anymore.

Proposal 2: Either Option 1 or Option 3 is OK for ATG UE power definition.

## UE requirement type

Companies are encouraged to input on the following options in this meeting:

  - Option1: discuss whether 1-O and 1-H are appropriate for an AAS like UE equipment.

  - Option2: still use UE requirement structure and investigate what is specific aspects for ATG UE.

For ATG UE equipped with antenna array we think the beamforming gain should be considered. it can be AAS like requirement, but still needs to be limited to the UE requirement structure. The requirement structure of FR2 UE can be taken as the starting point and further check the antenna connect availability. If antenna connector is available, then the requirement definition can borrow hybrid AAS BS.

For ATG UE equipped with omni-directional antenna, the current FR1 UE requirement should be taken as the baseline.

Proposal 3: Depending on the ATG UE antenna type, either of the following way should be considered for requirement structure,

  - 3a: taking FR2 UE requirement structure but define hybrid BS like requirements where most of the requirement are still tested at antenna connector.

  - 3b: Take FR1 UE requirement structure as the baseline.

## Power dynamics

Companies are encouraged to input on the following options in this meeting:

  - Option1: RAN4 to examine whether power control requirement is needed, or whether the dynamic range of power control can be simplified

  - Option 2: Reduce the power control dynamic range according to the ATG scenario

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. It is used to reduce interference to other ATU UE signal reception at gNB.

As for ATG UE, we propose to consider a rough estimation as following. The minimum ouput power may be required when the aircraft flies over the ATG BS and only work on REFSENS level of ATG BS.

Pmin =PREFSENS,BS + CL

Assuming the UE height is 15km, the carrier frequency is 2000MHz and 25~30dB gain, the CL can be estimated according to the free space path loss model as below. The estimated minimum output power could be in the order of -25~-30dBm

PL = 32.5+20*log10(d)+20*log10(f)

Proposal 4: The minimum output power can be defined in the range of -25~-30dBm

Regarding UE off power, we think it can also be scaled accordingly by 10~15dB on top of the current -50dBm/MHz.

# Summary

This contribution presented our view on the UE requirement for ATG UE. the following are concluded.

Proposal 1: UE frequency pre-compensation for initial access is needed RAN2 need to be involved in the WI.

Proposal 2: Either Option 1 or Option 3 is OK for ATG UE power definition.

Proposal 3: Depending on the ATG UE antenna type, either of the following way should be considered for requirement structure,

  - 3a: taking FR2 UE requirement structure but define hybrid BS like requirements where most of the requirement are still tested at antenna connector.

  - 3b: Take FR1 UE requirement structure as the baseline.

Proposal 4: The minimum output power can be defined in the range of -25~-30dBm

# References

[1] R4-2214247, Email discussion summary, CMCC

[2] RP-2215147, WF on UE requirements for ATG, Apple

[2] R4-2211917, Overview on ATG UE requirements, Apple