3GPP TSG-RAN WG4 Meeting#104-bis-e                	R4-2215639

Electronic Meeting, 10 October – 19 October 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUSCH demodulation performance of Rel-17 NR coverage enhancements: simulation results

Agenda item: 	4.4.2.1

Document for: 	Discussion

Introduction

In this contribution, we provide the results of our simulation campaign considering the agreements in previous meetings captured in [1][2][3] for PUSCH TBoMS.

PUSCH enhancements

TBoMS

Test parameters

We provide in this section the simulation results of TBoMS using the parameters below:

Table 1: TBoMS simulation parameters.

| Parameters | TBoMS (FDD and TDD) |
| --- | --- |
| Metric | SNR at 70% throuhput |
| MCS | MCS2 |
| Resource allocation | 5 PRB |
| Number of slots | 2 slots for TDD 4 slots for FDD |
| TDD pattern / FR | 30 kHz : 7D1S2U, S=6D:4G:4U For 15/60/120kHz SCS, 3D1S1U, S=10D:2G:2U |
| DMRS | DMRS 1+1 only and no PT-RS configured |
| Inter Intra slot frequency hopping | Disabled |
| Channel condition | higher speed channel model TDLB100-400 Low for FR1 and TDLA30-300 Low for FR2 |
| Antenna configuration | 1T2R |
| RV sequence | [0 2 3 1] |
| PUSCH mapping type | Cover both PUSCH mapping type A and type B for FR1 // Type B for FR2 |
| HARQ | 4 |
| Repetition | No repetition |
| transform precoding | CP-OFDM |
| DMRS/PTRS | DMRS 1+1 only and no PT-RS configured |
| DM-RS configuration type | 1 |
| DM-RS duration | single-symbol DM-RS |
| Additional DM-RS position | pos1 |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
| DM-RS port | 0 |
| DM-RS sequence generation | NID0=0, nSCID =0 |
| Start symbol | 0 |
| Allocation length | 14 for FR1 //10 for FR2 |
| Frequency density (KPT-RS) | Disabled |
| Time density (LPT-RS) | Disabled |

Simulation results: SNR at 70% throughput

The Table 2 gives the SNR at 70% throughput for two TBoMS configurations: TBoMS over 2 slots and TBoMS over 4 slots using Rx, different SCS, and both PUSCH mapping type 1 and PUSCH mapping type B.

Table 2: SNR at 70% throughput in case of TBoMS.

| TBoMS over |  |  |  |  | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| 4 available slots | 1T2R | FDD 15kHz SCS | TDLB100-400 Low | PUSCH Mapping type A | -4,37 | -1,87 |
|  |  |  |  | PUSCH Mapping type B | -4,15 | -1,65 |
| 2 available slots |  | TDD 15kHz SCS |  | PUSCH Mapping type A | -4,52 | -2,02 |
|  |  |  |  | PUSCH Mapping type B | -4,38 | -1,88 |
| 4 available slots |  | FDD 30kHz SCS |  | PUSCH Mapping type A | -4,38 | -1,88 |
|  |  |  |  | PUSCH Mapping type B | -4,36 | -1,86 |
| 2 available slots |  | TDD 30kHz SCS |  | PUSCH Mapping type A | -4,30 | -1,80 |
|  |  |  |  | PUSCH Mapping type B | -4,26 | -1,76 |
| 2 available slots |  | TDD 60kHz SCS | TDLA30-300 Low | PUSCH Mapping type B | -4,40 | -1,30 |
| 2 available slots |  | TDD 120kHz SCS |  | PUSCH Mapping type B | -4,20 | -1,10 |

Conclusion

In this contribution we have provided the results of our simulation campaign on PUSCH TBoMS for NR_Cov_enh Rel-17 BS demodulation performance requirements.

References

- R4-2203030 WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2207210 WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2210666 WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement