3GPP RAN WG4 Meeting #104bis		R4-2215647

Electronic meeting, October 10 – 19th, 2022

Agenda item:	6.28.1

Source:	Apple

Title:	Update of the regulatory requirements and summary of NS values

WI/SI:	NR_unlic_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which has been approved recently in different regulatory regions. Since not all the countries and regions were addressed during the corresponding Rel-16 and Rel-17 WI [2], RAN#96 meeting approved a new WI [3] with the main motivation of enabling LPI and VLP in countries not covered by previous efforts.

During the last RAN4#104 meeting a contribution with overview of the 6GHz regulatory rules for the LPI and VLP modes was presented [4] with the intention of identifying which existing NS values can be reused and which new NS values are still needed. However, since during the TSG RAN#97 meeting another regulatory updated was submitted for TR 37.890 [5] covering New Zealand, Malaysia and Japan, in this discussion paper we present our further considerations on which NS values we need to support these countries.

# 2	Overview of the 6GHz regulatory rules

As mentioned in the Introduction part, there was a regulatory status update submitted to the TSG RAN#97 meeting covering countries, such as New Zealand, Malaysia and Japan. Here we provide a more detailed overview of new requirements with a summary of which NS flags can be re-used.

-	New Zealand. New Zealand regulatory requirements are identical to Australia allowing both LPI and VLP modes in 5925-6425MHz range.

-	Malaysia. Malaysian regulations allow both LPI and VLP modes in the 5925 – 6425MHz range with the maximum Tx power and PSD requirements identical to EU/CEPT, but unlike EU/CEPT there are no emission limits.

-	Japan. Japanese regulations also allow unlicensed spectrum usage in the range of 5925-6425MHz for both LPI and VLP modes with the maximum Tx power of 23dBm and 14dBm, respectively. And even though the Japanese requirements do not define explicitly PSD, it is clearly indicated that the minimum operating channel size is 20MHz, which implicitly sets the maximum PSD limit. And it is also worth noting that Japanese requirements clearly indicate that only 20, 40, 80, and 160MHz channels are allowed.

As can be seen from the summary, all countries can be supported with the existing 3GPP band n102. New Zealand LPI and VLP modes can be supported with the same NS flags as Australia, i.e. no new flags are needed. Even though Malaysian regulations are close to EU/CEPT, there are no out-of-band emission limits meaning that actual requirements are less stringent and can be ensured with the NS flags already assumed for other countries: NS_01 for LPI and a new "flat 6dB back-off" flag for VLP. And the same approach can be also taken for Japan.

Proposal 1:	Re-use 3GPP band n102 for the LPI/VLP operation in New Zealand, Malaysia and Japan.

Proposal 2a:	For the LPI operation in New Zealand, Malaysia and Japan, NS_01 can be re-used.

Proposal 2b:	For the VLP operation in New Zealand, the corresponding NS flag from Australia can be re-used.

Proposal 2c:	For the VLP operation in Malaysia and Japan, the corresponding "flat 6dB back-off" NS flag can be re-used.

Speaking of the Japanese regulations, they limit the operating channels only to 20, 40, 80 and 160MHz. 3GPP NR-U core functionality does not support 160MHz channel, but 60MHz and 100MHz channels are defined for NR-U, which apparently shall not be used in Japan. Thus, we suggest discussing further whether these restrictions should be reflected in the 3GPP core specifications or whether we can leave it to the network configuration.

Proposal 2d:	Since Japanese regulations allow only 20, 40, 80 and 160MHz channels, it should be discussed further whether these restrictions will be reflected in 3GPP specifications or left for the network configuration.

Based on the presented considerations, an updated summary of NS values is presented in the Table 2-1 below, where the highlighted parts are new countries and NS values. As can be seen from the table, New Zealand can use same NS flags as Australia. Malaysia and Japan can use NS_01 for LPI and "flat 6dB back-off" NS flag for VLP.

Table 2-1: Updated NS values for PC5.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | NS_y1 (new) |
| UK | N/A | NS_01 | NS_y2 (new) |
| Morocco | N/A | NS_01 | NS_y2 (new) |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | N/A |
| Canada | NS_54 | NS_59 | NS_y3 (new) |
| Brazil | N/A | NS_53 | NS_y4 (new) |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | NS_y4 (new) |
| Costa Rica | N/A | NS_01 | NS_y2 (new) |
| Colombia | N/A | NS_53 | N/A |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | NS_y1 (new) |
| Australia | N/A | NS_01 | NS_y5 (new) |
| New Zealand | N/A | NS_01 | NS_y5 (new) |
| Malaysia | N/A | NS_01 | NS_y2 (new) |
| Japan | N/A | NS_01 | NS_y2 (new) |

Proposal 3:	Endorse an updated summary of NS values for PC5 (subject for further changes).

# 3	Conclusions

In this contribution we have presented summary of the regulatory requirements of the 6GHz band LPI and VLP modes for the countries that were not address by previous WIs.  Based on the presented considerations existing band n96 and n102 can be re-used to support the LPI and VLP mode. However, new NS flags are needed for the 3GPP bands n96 and n102 because there are several countries that have quite different regulatory requirements (the associated A-MPR values can be found in [4]).

Proposal 1:	Re-use 3GPP band n102 for the LPI/VLP operation in New Zealand, Malaysia and Japan.

Proposal 2a:	For the LPI operation in New Zealand, Malaysia and Japan, NS_01 can be re-used.

Proposal 2b:	For the VLP operation in New Zealand, the corresponding NS flag from Australia can be re-used.

Proposal 2c:	For the VLP operation in Malaysia and Japan, the corresponding "flat 6dB back-off" NS flag can be re-used.

Proposal 2d:	Since Japanese regulations allow only 20, 40, 80 and 160MHz channels, it should be discussed further whether these restrictions will be reflected in 3GPP specifications or left for the network configuration.

Proposal 3:	Endorse an updated summary of NS values for PC5 (subject for further changes).

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP-213180, "Revised WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- RP-221813, "New WID: Introduction of evolved shared spectrum bands", Apple Inc.

- R4-2212338, "Overview of the regulatory parameters and potential NS values", Apple Inc.

- RP-222173, "Regulatory update for the 6GHz frequency range", Apple Inc.

Annex A: Summary of the regulatory requirements

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
|  | UAE | LPI (see 4.1.5) | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI (see 4.1.7) | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP (see 4.1.7) |  | 14 dBm |  |  |
|  | Saudi Arabia | LPI (see 4.1.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 10dBm/MHz |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  |  | VLP (4.2.5) |  | 17 dBm |  |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |
|  | Hong Kong | LPI (see 4.3.3) | 5925 – 6425MHz | 23dBm | 10dBm/MHz | In accordance with ETSI EN 303 687 |
|  |  | VLP (see 4.3.3) |  | 14dBm | 1dBm/MHz |  |
|  | Australia | LPI (see 4.3.4) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.4) |  | 14dBm | 1dBm/MHz |  |
|  | New Zealand | LPI (see 4.3.5) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.5) |  | 14dBm | 1dBm/MHz |  |
|  | Japan | LPI (see 4.3.6) | 5925 – 6425MHz | 23dBm |  |  |
|  |  | VLP (see 4.3.6) |  | 14dBm |  |  |
|  | Malaysia | LPI (see 4.3.7) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.3.7) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) |  |