3GPP RAN4 Meeting #104bis-e		R4-2215653

Online meeting, October 10 – 19, 2022

Agenda item:	6.14.2.1

Source:	Apple

Title:	On TRP TRS methodology enhancements

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

Following the conclusion of the Rel-17 RAN4-led work item on the Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) [1], [2], 3GPP approved a new Rel-18 work item on the Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies [3] with the following work objectives:

### 4.1 Objective of Core part WI The following objectives are considered in this WI: - Enhancements of TRP TRS test methodology - Specify necessary enhancement of the anechoic-chamber based test methodology (i.e. reference test methodology) to support (test methodology defined in TR 38.834 is the baseline): - UE with NR 2Tx configuration - Case 1: TxD (i.e., TxD capability supported) - Case 2: single layer UL-MIMO (i.e., codebook-based capability supported) - Study proper configuration from UE implementation and test system feasibility perspective - Define test case applicability for case 1 and case 2 - RedCap devices (focused on wearable device only) - Wrist-worn device as 1st priority - Considering UEs with antenna configurations of 1 Tx, 1 Rx, 2 Rx - Forearm phantom should be specified - Consider the coordination with CTIA on this aspect - Other phantoms are not precluded - UE with DL/UL carrier aggregation configuration (2nd priority) - Independent measurements of each CC can be the baseline approach - Limited to Inter-band CA with up to 2DL/2UL cases - Study and, if feasible, specify reverb-chamber based test methodology to support SA and EN-DC TRP TRS testing: - Define test methodology for UE type with 1Tx and 2Tx configuration - Test scenarios support: at least browsing mode, talk mode, and wrist-worn mode - Perform harmonization activity with reference test method - Define harmonization framework and pass/fail criteria - Perform harmonization test campaign according to the framework - Note: the harmonization between anechoic chamber and Reverb-chamber based method can be applied independently for the conditions between 1Tx and 2Tx, among browsing mode, talk mode, and wrist-worn mode. - If reverb-chamber based test methodology is found harmonized with the reference test method, then further discuss applicability of reverb-chamber based test methodology for requirements conformance testing - Develop the Measurement Uncertainty (MU) assessment of the above newly defined test methodologies and test configurations [RAN5]: - Measurement Uncertainty (MU) aspects will be handled by RAN5 and the conclusions can be captured in a separate section of the TR - Browsing mode, talk mode, and wrist-worn mode should be considered - Study and specify the testing time reduction methodology for TRP and TRS testing During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9 WG1, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic

### 4.2 Objective of Performance part WI Specify the performance part framework (general for both 1Tx and 2Tx requirements work): - Lab alignment outcome and performance campaign framework in Rel-17 should be baseline with the following considerations: - Decide whether lab alignment is needed for new test bands, new test scenarios and EN-DC configuration - Handheld UE type is the first priority - Consider UE size 1 and size 2 - UE size 1 is the first priority - Consider both browsing mode and talk mode - Further prioritization not precluded - Consider Power class 2 and 1.5 for 2Tx requirements, Power class 3 and 2 for 1Tx requirements - Requirement applicability of TxD and single-layer UL MIMO of the same UE shall be specified - Consider further prioritization of test cases - Taking following as a starting point to further discuss which information needed including for information disclosure: - [Number of models tested by the labs] - Number of vendors that produced the models - percentage of tested devices per vendor - Percentage of models per production year - [Power Class of the devices] - Note 1: Size 1 (wide, width >72mm and ≤92mm), Size 2 (narrow, width ≥56mm and ≤72mm); - Note 2: Browsing mode (hand phantom only), Talk mode (head and hand phantom); - Specify TRP TRS requirements and recommended tolerance for UE with NR 2Tx for handheld UE based on enhanced reference test method and defined performance part framework - Specify the requirements and test tolerance for UE with SA mode - Band n41, n77 and n78, as the first priority - FDD bands are not precluded - SA with 1 CC is the first priority - Specify TRP TRS requirements and recommended tolerance for UE with NR 1Tx (SA and EN-DC) based on reference test method and defined performance part framework - Specify the requirements and recommended tolerance for UE - SA: n3, n5, n7, n8, and n28 with PC3, n1, n77 and n79 with PC2 and PC3 - For talk mode requirements, n41 and n78 with PC2 should also be additionally considered - Further prioritization on band list not precluded - EN-DC: NR SA bands related EN-DC band combinations are the first priority, only NR carrier requirement will be specified - Rel-17 band combination conclusions in TS 38.161 should be the basis - For each newly added NR band to the scope, at least one example band combination shall be defined

In the contribution we share our initial views on the scope of test methodology enhancements for the anechoic chamber (reference) methodology.  These inputs concern the objectives related to test methodology development for UEs with 2Tx configuration, RedCap devices, DL/UL CA, and test time reduction.

# 2	Discussion

## 2.1	TRP measurement of devices capable of 2 Tx

As defined on TS 38.211 Table 6.3.1.5-1, for codebook-based transmission precoding matrix W index for single-layer transmission using two antenna ports. TPMI indexes  0, 1 and 2 are related to non-coherent UL MIMO, while the TPMI indexes 2, 3,4 and 5 are related to coherent UL MIMO.

Observation 1:	TPMI indexes 0 and 1 are single port transmission schemes.

Proposal 1:	Consider to not adopting TPMI indexes 0 and 1 individually, since these single port antenna schemes are less relevant to 2Tx TRP.

Proposal 2:	Define different TRP requirements for coherent and non-coherent UL MIMO capable devices.

Proposal 3:	To not consider TPMI 2 and only focus on TPMI 0 and 1 for non-coherent devices considering the linear combination of separately measurement of TPMI 0 + TPMI 1 results.

Proposal 4:	Enable TPMI indexes for coherent UL MIMO testing.

Proposal 5:	Define a methodology to optimize EIRP based on TPMI selection, FFS.

## 2.2	TRP and TRS measurement of RedCap devices

The antenna system on wearable devices is developed since inception prioritizing the user case, a correspondent phantom is adopted during the design process and through the OTA radiated performance assessment. Free space is de-prioritized or not considered, since most of these devices are dedicated to limited user-cases not including free space conditions.

The CTIA Test Plan 01.72 Near-Field Phantoms V4.0, defines on section 2.3 forearm phantom requirements. These phantoms are commercially available and had been adopted for years on wearable devices certification process.

Observation 2:	RedCap devices, particularly wearables, can be developed adopting less complex user cases. Requiring simple fixed sized phantoms e.g.: forearm or chest phantoms. Rather than more complex test configurations adopting hands phantoms which may vary dimension and grip  depending on the DUT width dimensions..

Proposal 6:	Submit an LS to CTIA requesting more information on forearm phantoms defined on CTIA Test Plan 01.72 Near-Field Phantoms V4.0, section 2.3.

## 2.3	TRP and TRS measurement of devices configured with UL/DL CA

Considering the very large scope of the Rel-18 work item on TRP/TRS enhancements overall, it is quite notable that the UL/DL CA objective was already recognized by RAN to be an “extra” item in the WID.  Our understanding of the intention of this objective is that it is one of best-effort completion.

Beyond the workload arguments, RAN4 should also consider that OTA requirements in CA configurations add little value on top of requirements defined for single carrier.  A great example of this is the applicability rule for the verification of TRP/TRS in EN-DC mode for devices which support both SA and NSA.  TS38.161 captures this as follows:

## 4.3 Applicability rules for testing of FR1 SA and NSA UEs The applicability and test coverage rules for Non-Standalone (NSA) only capable devices shall include the following: - For each NR band supported by the device, test the UE in EN-DC mode using any one example configuration containing that NR band or configuration declaration decision tree as per recommended TRP/TRS test procedures in this specification. The applicability and test coverage rules for Standalone (SA) and NSA (EN-DC) capable devices shall include the following: - For each NR band in a device, test the UE in Standalone Mode as per the TRP/TRS test procedures in this specification. - This shall also fulfil coverage for all EN-DC FR1 minimum performance requirements for that NR band and need not be retested in EN-DC mode.

Observation 3:	RAN4 has already concluded that for a given NR band under test, the single-carrier verification of OTA performance in stand-alone mode is sufficient and is prioritized over the same verification in an EN-DC configuration.

Proposal 7:	RAN4 should conclude work on all first-priority methodology enhancement objectives before starting discussion related to TRP and TRS measurement of devices configured with UL/DL CA.

## Test Time Reduction

We would like to propose some areas for study towards potential test time reduction for NR FR1 TRP TRS testing. Currently these proposed methods for evaluation apply to the methods described in TR 38.834 and referenced in TS 38.161 but can be extended to additional scope being brought into the NR FR1 TRP TRS WID.

Observation 4:	As per the principles of TRP TRS testing, it is well known that TRS measurements are significantly more time consuming than TRP measurements. Therefore, while looking at test time reduction for both measurements, the bigger need seems to be for TRS.

Observation 5:	Investigate possible test time reduction using alternate Test Procedures for FR1 TRP/TRS Measurements:

Other SDOs such as CTIA have described several alternate test methodologies which could potentially offer significant test time savings with NR FR1 TRP TRS measurements. It is recommended to evaluate these test procedures and plan to liaise with the referenced SDOs if RAN4 identifies such alternate test methods as potential candidates to leverage. Some examples are

  - Spiral Scan methodology : Reduce test time for TRP using continuous rotation of both theta and phi positioner during collection of data points. Power measurements on the measurement antenna for both theta and phi polarizations may be taken with simultaneously or in rapid succession [4]. Alternately each polarization may be measured for a complete spiral.

  - Single Point Offset Test Procedure: is intended to reduce the test time for TRP or TRS for devices supporting multiple technologies in the same frequency band. With the understanding that a similar antenna pattern is expected on both protocols/technologies owing to the same band of operation. The offset is calculated by determining the difference in TRP measurements from both test configurations (say A for first technology and B for second technology). This offset can be used to compute TRP for other points if one set of measurements is known. In the context of NR FR1 TRP TRS, this approach could be studied in case of several overlapping E-UTRA and NR bands. The concept is extended to Multiple single point offset (essentially applying same concept to larger number of test points). A possible approach is to test full suite on mid channel only but apply multiple single point offset methodology for low and high channel testing.

Observation 6:	Investigate possible Test time reduction by optimization of test cases:

  - Reduce number of test cases across Hand Only and Head and Hand by measuring LMH in HL/BHHR and mid only in BHHL and HR. Alternate approach could be to focus on mid channel only for BHHR/BHHL measurements

  - Explore fast TIS with optimized measurement grids. There is potential of slight increase in MU which will need to be evaluated further. Feedback from test equipment vendors is requested.

Proposal 8:	RAN4 to further evaluate test time savings approaches starting with the items in observations 4, 5 and 6. As needed, liaising shall be pursued with other SDOs where leverage of test methods is deemed possible.

# Conclusions

In the contribution we share our initial views on the scope of test methodology enhancements for the anechoic chamber (reference) methodology.  These inputs concern the objectives related to test methodology development for UEs with 2Tx configuration, RedCap devices, DL/UL CA, and test time reduction.  The following observations and proposals are presented:

Observation 1:	TPMI indexes 0 and 1 are single port transmission schemes.

Observation 2:	RedCap devices, particularly wearables, can be developed adopting less complex user cases. Requiring simple fixed sized phantoms e.g.: forearm or chest phantoms. Rather than more complex test configurations adopting hands phantoms which may vary dimension and grip  depending on the DUT width dimensions..

Observation 3:	RAN4 has already concluded that for a given NR band under test, the single-carrier verification of OTA performance in stand-alone mode is sufficient and is prioritized over the same verification in an EN-DC configuration.

Observation 4:	As per the principles of TRP TRS testing, it is well known that TRS measurements are significantly more time consuming than TRP measurements. Therefore, while looking at test time reduction for both measurements, the bigger need seems to be for TRS.

Observation 5:	Investigate possible test time reduction using alternate Test Procedures for FR1 TRP/TRS Measurements:

Observation 6:	Investigate possible Test time reduction by optimization of test cases:

Proposal 1:	Consider to not adopting TPMI indexes 0 and 1 individually, since these single port antenna schemes are less relevant to 2Tx TRP.

Proposal 2:	Define different TRP requirements for coherent and non-coherent UL MIMO capable devices.

Proposal 3:	To not consider TPMI 2 and only focus on TPMI 0 and 1 for non-coherent devices considering the linear combination of separately measurement of TPMI 0 + TPMI 1 results.

Proposal 4:	Enable TPMI indexes for coherent UL MIMO testing.

Proposal 5:	Define a methodology to optimize EIRP based on TPMI selection, FFS.

Proposal 6:	Submit an LS to CTIA requesting more information on forearm phantoms defined on CTIA Test Plan 01.72 Near-Field Phantoms V4.0, section 2.3.

Proposal 7:	RAN4 should conclude work on all first-priority methodology enhancement objectives before starting discussion related to TRP and TRS measurement of devices configured with UL/DL CA.

Proposal 8:	RAN4 to further evaluate test time savings approaches starting with the items in observations 4, 5 and 6. As needed, liaising shall be pursued with other SDOs where leverage of test methods is deemed possible.

# 4	References

- RP-222269, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, 3GPP RAN #97, September 2022

- RP-222152, "Status report for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC); rapporteur: vivo," 3GPP RAN #97, September 2022

- RP-222669, “Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies,” vivo, 3GPP RAN #97, September 2022