3GPP RAN WG4 Meeting #104bis-e		R4-2215661

Online, October 10th – 19th, 2022

Agenda item:	5.22.2

Source:	Apple

Title:	MSD analyses for PC2 FDD bands with 2Tx

WI/SI:	HPUE_NR_FR1_FDD_R18

Release:	Rel-18

Document for:	Approval

# 1	Introduction

The basket WID for NR FR1 PC2 FDD bands has been approved in September plenary meeting [1]. The initially proposed bands include n2, n5, n8, n13, n25, n26, n28, n66, n71, and n85 where most of them are expectedly subject to further sensitivity degradation due to increased UL interference caused by 3dB higher maximum output power from PC3 to PC2. In this contribution, we provide our MSD analysis results for n2, n5, n8, n25, n26, n28, n71, and n85 based on 2Tx PC2 configuration.

# Discussion

The 2Tx MSD analysis was based on the PC3 REFSENS requirements to first extract the Tx noise, assuming antenna isolation is 10 dB. The Tx noise was then added to thermal noise for both main and diversity Rx paths, followed by MRC to calculate the equivalent total noise for 2Tx configuration. The MSD was then derived by taking the difference between the 2Tx PC2 equivalent total noise and the PC3 equivalent total noise. To account for the PA reverse IMD when 2PA are transmitting simultaneously, additional 1dB Tx noise contribution was assumed in our analysis.

Table 2-1 summarizes MSD results based on our analyses.

| Band | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n2 | 0 | 0 | 0 | 0 | 0.2 | 6.7 | 6.6 | 7.3 | N/A | N/A |
| n5 | 0 | 0 | 0 | 6.2 | 6.8 | N/A | N/A | N/A | N/A | N/A |
| n8 | 0 | 0 | 0.9 | 6.2 | N/A | N/A | 8.1 | N/A | N/A | N/A |
| n25 | 0 | 0 | 0 | 0 | 0.1 | 6.9 | 6.8 | 7.5 | 7.9 | N/A |
| n26 | 0 | 0 | 0 | 5.0 | 6.7 | 7.5 | N/A | N/A | N/A | N/A |
| n28 | 0 | 0 | 0 | 2.1 | 7.3 | 8.4 | N/A | N/A | N/A | N/A |
| n71 | 0 | 0 | 1.0 | 6.2 | 6.7 | 7.1 | 7.5 | N/A | N/A | N/A |
| n85 | 0 | 0 | 7.6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 2-1 2Tx PC2 MSD relative to PC3 REFSENS

Proposal 2: RAN4 to take the MSD values in Table 2-1 into consideration for n2, n5, n8, n25, n26, n28, n71, n85 2Tx PC2 MSD relative to PC3 REFSENS.

# 3	Conclusion

In this contribution, we provide our MSD analysis results for n2, n5, n8, n25, n26, n28, n71, and n85 based on 2Tx PC2 configuration.

Proposal 1: Proposal 2: RAN4 to take the MSD values in the table below into consideration for n2, n5, n8, n25, n26, n28, n71, n85 2Tx PC2 MSD relative to PC3 REFSENS.

| Band | Channel BW (MHz) / MSD (dB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| n2 | 0 | 0 | 0 | 0 | 0.2 | 6.7 | 6.6 | 7.3 | N/A | N/A |
| n5 | 0 | 0 | 0 | 6.2 | 6.8 | N/A | N/A | N/A | N/A | N/A |
| n8 | 0 | 0 | 0.9 | 6.2 | N/A | N/A | 8.1 | N/A | N/A | N/A |
| n25 | 0 | 0 | 0 | 0 | 0.1 | 6.9 | 6.8 | 7.5 | 7.9 | N/A |
| n26 | 0 | 0 | 0 | 5.0 | 6.7 | 7.5 | N/A | N/A | N/A | N/A |
| n28 | 0 | 0 | 0 | 2.1 | 7.3 | 8.4 | N/A | N/A | N/A | N/A |
| n71 | 0 | 0 | 1.0 | 6.2 | 6.7 | 7.1 | 7.5 | N/A | N/A | N/A |
| n85 | 0 | 0 | 7.6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

# 4	References

RP-222625 “New WID: High power UE for FR1 for FDD single band(s) with power class 2”, China Unicom, 3GPP TSG-RAN Meeting #97-e, Online, September 12th – 16th, 2022