3GPP RAN WG4 Meeting #104bis-e		R4-2215665

Online, October 10th – 19th, 2022

Agenda item:	6.3.3

Source:	Apple

Title:	On FR1 2UL inter-band CA UE coexistence requirements

WI/SI:	FS_SimBC

Release:	Rel-18

Document for:	Approval

# 1	Introduction

The FR1 uplink inter-band CA (two bands) spurious emission for UE coexistence requirements in Table 6.5A.3.2.3-1 in TS 38.101-1 [1] has been derived based on the requirements for single carrier operation from each constituent UL band as specified in Table 6.5.3.2-1 in TS 38.101-1. The protected bands and frequency ranges for a band combination however are not necessary to cover all the protected bands and frequency ranges from each constituent band due to the reason that one frequency band may be deployed in certain regions and countries, while the other frequency band may not be deployed in all the same regions and countries. Therefore, the protected bands and frequency ranges for a band combination in principle should be specified based on the intersection set from each constituent band coexistence requirements.

Along the line of the Rel-18 SI to study on the simplification of band combination specifications for NR and LTE [2], in last RAN4 meeting, it has been proposed that the 2UL inter-band CA UE coexistence requirements is specified only with a normative text as “For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements are the intersection set from each constituent band coexistence requirements as specified in Table 6.5.3.2-1.” without an explicit coexistence table [3]. The benefits for not having an explicit coexistence table for band combinations can be perceptibly realized to not only simplify the contents of the technical specifications (more than 18 pages reduction in TS 38.101-1), but also to save time and efforts on manually checking the errors and the associated CR processes. While the proposal above had received substantial supports as a potential way forward on handling the 2UL inter-band CA UE co-existence requirements in last RAN4 meeting, there were also concerns on the possible exceptions which may not be covered by the intersection set from each constituent band [4]. The discussions were then concluded with an approved WF with the following objectives [5]:

- Companies are encouraged to investigate whether the 2UL inter-band CA UE co-existence requirements can be specified based on the intersection set of the protected bands and frequency ranges from each constituent band without an explicit coexistence table.

- Investigate on how to handle the intersection band/ranges with different requirements from each constituent band.

- Identify if there are exceptions not applicable to the principle of the intersection set and how to handle them in either a generic or a combination specific approach.

In this contribution, we follow up the above WF to share our views on how 2UL inter-band CA UE co-existence requirements spec. structure may potentially be simplified based on the intersection set of the protected bands and frequency ranges from each constituent band.

# Discussion

## 2.1	Single-band UE coexistence requirements

To investigate whether the 2UL inter-band CA UE co-existence requirements can be specified based on the intersection set of the protected bands and frequency ranges from each constituent band, we have revisited the single-band spurious emission for UE coexistence requirements where they can basically be divided into the following categories:

- General 3GPP band protection

- PHS system protection

- Other non-3GPP RATs protection

For general 3GPP band protection, the requirements also depend on how close the protected band to the UL aggressor band in frequency. When the protected band is not in frequency proximity to the UL aggressor band, the spurious emission requirement is typically specified as -50 dBm/MHz. When the protected band is adjacent to the UL aggressor band, the protected band may be divided into a few sub-ranges with different emission requirements to account for the UE ACLR/SEM characteristics and the lack of filter rejection in practical UE implementation, as is illustrated in Table 2-1.

| NR Band | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1, n84 1920 – 1980 MHz (≤ 54 RB) | B39 | 1880 | - | 1895 | -40 | 1 | 27 |
|  |  | 1895 | - | 1915 | -15.5 | 5 |  |
|  |  | 1915 | - | 1920 | +1.6 | 5 |  |
| n7 2500 – 2570 MHz (≤ 54 RB) | B38 | 2570 | - | 2575 | +1.6 | 5 | 21 |
|  |  | 2575 | - | 2595 | -15.5 | 5 |  |
|  |  | 2595 | - | 2620 | -40 | 1 |  |
| n38 2570 – 2615 MHz (≤ 54 RB) | B7 | 2620 | - | 2645 | -15.5 | 5 | 22 |
|  |  | 2645 | - | 2690 | -40 | 1 |  |
| n39, n98 1885 – 1920 MHz (≤ 54 RB) | B3 | 1805 | - | 1855 | -40 | 1 | 33 |
|  |  | 1855 | - | 1880 | -15.5 | 5 |  |
| n65 1920 – 1980 MHz (≤ 54 RB) | n39 | 1900 | - | 1915 | -15.5 | 5 | 27 |
|  |  | 1915 | - | 1920 | +1.6 | 5 |  |

Table 2.1-1 UE coexistence spurious emission requirements for adjacent bands

For the protected band in the frequency proximity but not adjacent to the UL aggressor band, the spurious emission requirement can be relaxed from -50 dBm/MHz to a higher emission level without the need of additional UL power reduction. These cases including certain FDD self DL band protection are summarized in Table 2-2.

| NR Band | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n18 | B28 | 758 | - | 799 | -50 | 1 |  |
|  |  | 799 | - | 803 | -40 | 1 |  |
|  | B18/B19 | 860 | - | 890 | -40 | 1 |  |
|  | B8 | 945 | - | 960 | -50 | 1 |  |
| n28, n83 | B28 | 758 | - | 773 | -32 | 1 |  |
|  |  | 773 | - | 803 | -50 | 1 |  |
| n41 | B40 | FDL_low | - | FDL_high | -40 | 1 |  |
| n71 | B29 | FDL_low | - | FDL_high | -38 | 1 |  |
| n74 | B74 | 1475 | - | 1488 | -28 | 1 | 42 |
|  |  | 1475 | - | 1488 | -50 | 1 | 45 |
|  |  | 1475.9 | - | 1510.9 | -35 | 1 | 46 |
|  |  | 1488 | - | 1518 | -50 | 1 |  |

Table 2.1-2 UE coexistence requirements for non-adjacent bands with frequency proximity

For NR bands required to protect PHS system have been indicated by NOTE 8 in Table 6.5.3.2-1 in 38.101-1 which are summarized in Table 2-3.

| NR Band | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n3, n5, n8, n18, n26, n28, n34, n40, n41, n74, n77, n78, n79, n95 | PHS | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |

Table 2.1-3 NR bands which are required to protect PHS system

It is worth noting that since the PHS frequency range is within B39 band range, for NR bands which are required to protect both B39 with a spurious emission requirement at -50 dBm/MHz and PHS system, the protection to B39 should be sufficient to cover the PHS system protection as the former requirement is tighter. In this case, it may not be necessary to list PHS for protection by those NR bands.

On the other hand, PHS is a rather old phone system where the service may have been terminated in many countries or near end of life. Therefore, RAN4 should consider whether it is meaningful to retain the PHS system protection in NR UE coexistence requirements.

Observation 1: PHS is a rather old phone system where the service may have been terminated in many countries or near end of life.

Proposal 1: RAN4 to clarify whether it is necessary to protect both PHS system and B39 at the same time or if it is necessary to retain PHS system protection in NR UE coexistence requirements at all.

For other non-3GPP RATs protection, only a handful requirements are specified which are summarized in Table 2-4.

| NR Band | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n13 | Public Safety narrow band | 769 | - | 775 | -35 | 0.00625 |  |
|  |  | 799 | - | 805 | -35 | 0.00625 |  |
| n14 | Public Safety narrow band | 769 | - | 775 | -35 | 0.00625 |  |
|  |  | 799 | - | 805 | -35 | 0.00625 |  |
| n28 | DTV | 470 | - | 694 | -42 | 8 | 35 |
|  |  | 470 | - | 710 | -26.2 | 6 | 34 |
|  |  | 662 | - | 694 | -26.2 | 6 |  |
| n74 |  | 1400 | - | 1427 | -32 | 27 | 41 |

Table 2.1-4 Other non-3GPP RATs protection in UE coexistence requirements

It can be seen that the above non-3GPP RATs protection is only specified for the bands which are in frequency proximity to the protected frequency ranges which is different from the general 3GPP band protection and PHS system protection where all the coexisting bands are listed. On the other hand, if the 3GPP UE coexistence requirements have been defined based on the coexistence system simulations with similar types of UEs and networks, RAN4 may need to consider whether the above non-3GPP RATs protection should be included in the UE coexistence requirements or specified as additional spurious emission requirements.

Observation 2: The non-3GPP RATs protection is only specified for the bands which are in frequency proximity to the protected frequency ranges.

## 2.2	2UL inter-band CA UE coexistence requirements

Based on the analysis in section 2.1, we can conclude that for general 3GPP band protection, the the 2UL inter-band CA UE co-existence requirements can be specified based on the intersection set of the protected bands and frequency ranges from each constituent band. If the protected intersection band has different requirements from each constituent UL aggressor band, only the more relaxed requirements (due to either adjacent or frequency proximity) need to be specified, otherwise, the UE could fail the tighter requirement from one of the UL transmissions. For example, for UL CA_n1-n3, the following requirements are currently specified for single-band and UL CA coexistence requirements.

| NR Band | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1, n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n78, n79, n100, n104 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15, XX |
|  | Frequency range (B39) | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range (B39) | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range (B39) | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| n3, n80 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79, n100, n104 |  |  |  |  |  |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 52, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |

| NR CA Combination | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range (B39) | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range (B39) | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range (B39) | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |

Table 2.2-1 UE coexistence requirements for n1, n3, and CA_n1-n3

It can be seen that the UE coexistence requirements for UL CA_n1-n3 is basically specified based on the intersection set of the protected bands and frequency ranges from each constituent band. However, Band 45 and Band n104 which also belong to the intersection set somehow are not included. In our view, Band n104 could be a miss for CA_n1-n3 in the current specifications as the band was just recently introduced. For Band 45, clarification is needed as whether this is a miss in the current specifications or an exception from the principle of intersection set for the combination. On the other hand, Band 45 actually is not needed to be included in both n1 and n3 protection band list as it is a subset of Band 50 which can already cover the requirement for Band 45.

Observation 3: For UL CA_n1-n3, Band 45 and Band n104 which also belong to the intersection set are not currently included in the protected band list.

Observation 4: Band n104 protection could be a miss for CA_n1-n3 in the current specifications as the band was just recently introduced.

Observation 5: Band 45 is not needed to be included in both n1 and n3 protection band list as it is a subset of Band 50 which can already cover the requirement for Band 45.

For another UL CA example, CA_n1-n5, the following requirements are currently specified for single-band and UL CA coexistence requirements.

| NR Band | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1, n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n78, n79, n100, n104 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15, XX |
|  | Frequency range (B39) | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range (B39) | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range (B39) | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| n5, n89 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 18, 19, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 NR Band n79 |  |  |  |  |  |  |
|  | E-UTRA Band 41, 52, 53 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |

| NR CA Combination | Protected Band | Frequency Range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n5 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 41, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |

Table 2.2-2 UE coexistence requirements for n1, n5, and CA_n1-n5

It can be seen that the UE coexistence requirements for UL CA_n1-n5 is exactly specified based on the intersection set of the protected bands and frequency ranges from each constituent band.

For PHS system protection, it should be of no difference from the general 3GPP band protection, meaning that it is specified for the UL band combination only when the frequency range is protected by both of the constituent bands. Nonetheless, in our view what is more important is for RAN4 to clarify as whether it is necessary to protect both PHS system and B39 at the same time or if it is necessary to retain PHS system protection in NR UE coexistence requirements at all.

For other non-3GPP RATs protection, they are probably the only exceptions from the intersection set of the protected bands as they should be protected by the UL band combination as long as one of the constituent bands is required to protect them. On the other hand, as we have assessed in section 2.1, RAN4 may need to consider whether the non-3GPP RATs protection should be included in the UE coexistence requirements or specified as additional spurious emission requirements.

Observation 6: The non-3GPP RATs protection are the only exceptions from the intersection set of the protected bands as they should be protected by the UL band combination as long as one of the constituent bands is required to protect them.

Proposal 2: RAN4 to consider whether the non-3GPP RATs protection should be included in the UE coexistence requirements or specified as additional spurious emission requirements.

In summary, based on our assessment above, we can conclude that the protected bands and frequency ranges for a 2UL inter-band UL CA combination have already been specified based on the intersection set from each constituent band coexistence requirements except for non-3GPP RATs protection. And under this principle, we propose RAN4 to consider replacing the 2UL inter-band CA UE coexistence requirements with a normative text to simplify the contents of the technical specifications and save time and efforts on manually checking the errors and the associated CR processes.

Observation 7: The protected bands and frequency ranges for a 2UL inter-band UL CA combination have already been specified based on the intersection set from each constituent band coexistence requirements except for non-3GPP RATs protection.

Proposal 3: RAN4 to consider replacing the 2UL inter-band CA UE coexistence requirements with a normative text to simplify the contents of the technical specifications and save time and efforts on manually checking the errors and the associated CR processes.

# 3	Conclusion

In this contribution, we revisit the single-band UE coexistence requirements and share our views on how 2UL inter-band CA UE co-existence requirements spec. structure may potentially be simplified based on the intersection set of the protected bands and frequency ranges from each constituent band.

Observation 1: PHS is a rather old phone system where the service may have been terminated in many countries or near end of life.

Observation 2: The non-3GPP RATs protection is only specified for the bands which are in frequency proximity to the protected frequency ranges.

Observation 3: For UL CA_n1-n3, Band 45 and Band n104 which also belong to the intersection set are not currently included in the protected band list.

Observation 4: Band n104 protection could be a miss for CA_n1-n3 in the current specifications as the band was just recently introduced.

Observation 5: Band 45 is not needed to be included in both n1 and n3 protection band list as it is a subset of Band 50 which can already cover the requirement for Band 45.

Observation 6: The non-3GPP RATs protection are the only exceptions from the intersection set of the protected bands as they should be protected by the UL band combination as long as one of the constituent bands is required to protect them.

Observation 7: The protected bands and frequency ranges for a 2UL inter-band CA combination have already been specified based on the intersection set from each constituent band coexistence requirements except for non-3GPP RATs protection.

Proposal 1: RAN4 to clarify as whether it is necessary to protect both PHS system and B39 at the same time or if it is necessary to retain PHS system protection in NR UE coexistence requirements at all.

Proposal 2: RAN4 to consider whether the non-3GPP RATs protection should be included in the UE coexistence requirements or specified as additional spurious emission requirements.

Proposal 3: RAN4 to consider replacing the 2UL inter-band CA UE coexistence requirements with a normative text to simplify the contents of the technical specifications and save time and efforts on manually checking the errors and the associated CR processes.

# 4	References

3GPP TS 38.101-1 V17.6.0 (2022-06)

RP-221790 “Revised SID: Study on simplification of band combination specification for NR and LTE”, ZTE Corporation et al, 3GPP TSG-RAN Meeting #96, Budapest, Hungary, June 6th – 9th, 2022

R4-2212357 “On FR1 2UL inter-band CA coexistence requirements”, Apple, 3GPP TSG-RAN WG4 Meeting #104-e, August 15th – 26th, 2022

R4-2214242 “Email Discussion Summary for [104-e][131] FS_SimBC”, Moderator (ZTE), 3GPP TSG-RAN WG4 Meeting #104-e, August 15th – 26th, 2022

R4-2214448 “WF on FR1 2UL inter-band CA coexistence requirements”, Apple, 3GPP TSG-RAN WG4 Meeting #104-e, August 15th – 26th, 2022