3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215703

# Electronic Meeting, October 10 – October 19, 2022

Title:	Discussion on full degree of rotation freedom with 2AoA

Source:	Samsung

Agenda item:	6.5.2

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, it was agreed to further discuss the feasibility of supporting full degree of rotation freedom with 2AoA, as captured in the approved WF [1]:

Issue 1-1: Work plan

- Proposals

  - Option 1: To adopt the workplan Rel-18 FR2 OTA testing enhancements study item in R4-2214814

  - Option 2: Provide the comments if any

- Agreement: Endorse the latest draft work plan

  - RAN4 aims to further discuss and document the feasibility of supporting the full degree of rotation freedom with 2AoA and check the status by RAN4 #106 meeting.

In this contribution, we share our views on the interpretation of full degree of rotation freedom with 2AoA.

2. 	Discussion

There have been extensive discussion on full degree of rotation freedom with 2AoA, however, it seems not very clear what the exact meaning of full degree of rotation freedom with 2AoA is. Obviously it is not possible to achieve ideal full freedom if there is no grid granularity, e.g., there will be mechanical interference or signal blocking when the angle separation between the 2AoA is too small.

First of all there will be measurement grid for the 2AoA test with some granularity, e.g. 15deg for spherical coverage. Full degree of rotation freedom does not require each probe to be located any direction. It is enough to locate the probes on any measurement grid point.

Figure.1: illustration when AoA1 and AoA2 are close to each other

When the probes for the 2AoA are located at two grid points near the pole region for constant step size grid, such case means the two AoA are very close in solid angle. In the core requirement of Multi-Rx Chain DL, there is minimum angle separation discussion undergoing. Such case is potentially excluded by core requirement, because there would be severe performance degradation due to worse spatial isolation and hence spatial multiplex would not work well. Even there would be no minimum angle threshold defined in core requirement, a minimum angle threshold could be defined in this SI to avoid probes mechanical interference or to avoid signal blockage.

With above understanding, we would like to clarify that the meaning of full degree of rotation freedom with 2AoA for feasibility study: full degree of rotation freedom with 2AoA means that the two probes can be located onto any pair of different measurement grid points with the exception when the angle separation between the 2AoA is smaller than a minimum solid angle threshold.

Proposal 1:	RAN4 to study the feasibility of full degree of rotation freedom based on the following interpretation: full degree of rotation freedom with 2AoA means that the two probes can be located onto any pair of different measurement grid points with the exception when the angle separation between the 2AoA is smaller than a minimum solid angle threshold.

Legacy rotation system for 1AoA is usually to rotate DUT instead of rotating probe for convenience. For 2AoA case, it may be not enough to only rotate DUT since the relative direction between probes may also need to be adjusted depending on core requirement definition. It is necessary to have the same understanding in the group that the 2AoA directions are with reference to DUT.

Proposal 2:	when discussing the full degree of rotation freedom with 2AoA, the 2AoA directions are with reference to DUT.

3. 	Conclusion

Proposal 1:	RAN4 to study the feasibility of full degree of rotation freedom based on the following interpretation: full degree of rotation freedom with 2AoA means that the two probes can be located onto any pair of different measurement grid points with the exception when the angle separation between the 2AoA is smaller than a minimum solid angle threshold.

Proposal 2:	when discussing the full degree of rotation freedom with 2AoA, the 2AoA directions are with reference to DUT.

4. 	References

- R4-2214357,    “WF on NR FR2 OTA testing enhancements”, Qualcomm