3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215704

# Electronic Meeting, October 10 – October 19, 2022

Title:	Initial discussion of TRP TRS on NR 2Tx UE

Source:	Samsung

Agenda item:	6.14.2.1

Document for:	Discussion

1.	Introduction

It was agreed to enhance the FR1 TRP TRS in Rel-18 in RAN plenary September meeting and the new WID [1] shows that 2Tx related test method and requirements are one of major objectives:

  - Specify necessary enhancement of the anechoic-chamber based test methodology (i.e. reference test methodology) to support (test methodology defined in TR 38.834 is the baseline):

    - UE with NR 2Tx configuration

      - Case 1: TxD (i.e., TxD capability supported)

      - Case 2: single layer UL-MIMO (i.e., codebook-based capability supported)

        - Study proper configuration from UE implementation and test system feasibility perspective

      - Define test case applicability for case 1 and case 2

In this contribution, we discuss the test method for NR 2Tx UE and some suggestion for work plan is provided as well.

2. 	Discussion

2.1	test method

NR 2Tx UE depends on UE’s capability including Tx Diversity and single layer UL MIMO as indicated in the WID [1]. For single layer UL MIMO, it depends on the TPMI code book the UE supports. For two antenna port transmission, the TPMI table is shown as below (same as Table 6.3.1.5-1 in TS 38.211):

Table 1: Precoding matrix W for single-layer transmission using two antenna ports.

| TPMI index | W (ordered from left to right in increasing order of TPMI index) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 5 | 1210 | 1201 | 1211 | 121-1 | 121j | 121-j | - | - |

For TPMI=0 and TPMI=1, there will be 3dB power scaling loss except if it is reported as full power TPMI by corresponding ULFPTx feature.

TPMI=2~5 is full power TPMI, however, non-coherent UE could not support them except fullpowerMode1 UE which supports TPMI=2.

Tables in TS 38.212 show the TPMI availability for different UEs:

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower, or if transform precoder is disabled, maxRank = 1, and and ul-FullPowerTransmission is not configured or configured to fullpowerMode2 or configured to fullpower Bit field mapped to index; codebookSubset = fullyAndPartialAndNonCoherent; Bit field mapped to index; codebookSubset = nonCoherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2. 3; 1 layer: TPMI=3. 4; 1 layer: TPMI=4. 5; 1 layer: TPMI=5. 6-7; reserved. Table 7.3.1.1.2-5A: Precoding information and number of layers, for 2 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission = fullpowerMode1, or if transform precoder is disabled, maxRank = 1, and ul-FullPowerTransmission = fullpowerMode1 Bit field mapped to index; codebookSubset= nonCoherent. 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2. 3; Reserved. (from TS 38.212)

Based on above tables, single layer UL MIMO have two kinds of waveforms, one is DFT-s-OFDM (transform precoder is enabled), the other is CP-OFDM (transform precoder is disabled). There is quite big MPR for CP-OFDM waveform, so only DFT-s-OFDM waveform is applicable for TRP test. It is similar case for Tx diversity.

Proposal 1:	DFT-s-OFDM rather than CP-OFDM is selected for TRP test of NR 2Tx UEs

For 2 antenna ports transmission, there are both 1Tx TPMI (TPMI=0~1) and 2Tx TPMI (TPMI=2~5). According to the WID, the test method enhancement is for NR 2Tx configurations, so 1Tx TPMI is not in scope.

Proposal 2:	1Tx TPMI (TPMI=0~1) is not in the work item scope for NR 2Tx configurations

From above tables from TS 38.212, it could be summarized that 2Tx TPMI=2 could be supported by coherent UEs and fullpowerMode1 non-coherent UEs.

Observation 1:	2Tx TPMI=2 could be supported by coherent UEs and fullpowerMode1 non-coherent UEs

For non-coherent UE case, the phase variation between two antenna ports are not predictable, leading to a time-varying radiation pattern.

For coherent UE case, the relative phase error for coherent UL MIMO is specified as 40 degrees with many side conditions in clause 6.4D.4 of TS 38.101-1:

For coherent UL MIMO, Table 6.4D.4-1 lists the maximum allowable difference between the measured relative power and phase errors between different antenna connectors in any slot within the specified time window from the last transmitted SRS on the same antenna connectors, for the purpose of uplink transmission (codebook or non-codebook usage) and those measured at that last SRS. The requirements in Table 6.4D.4-1 apply when the UL transmission power at each antenna connector is larger than 0 dBm for SRS transmission and for the duration of time window. Table 6.4D.4-1: Maximum allowable difference of relative phase and power errors in a given slot compared to those measured at last SRS transmitted Difference of relative phase error; Difference of relative power error; Time window. 40 degrees; 4 dB; 20 msec. The above requirements when all the following conditions are met within the specified time window: - UE is not signaled with a change in number of SRS ports in SRS-config, or a change in PUSCH-config - UE remains in DRX active time (UE does not enter DRX OFF time) - No measurement gap occurs - No instance of SRS transmission with the usage antenna switching occurs - Active BWP remains the same - EN-DC and CA configuration is not changed for the UE (UE is not configured or de-configured with PSCell or SCell(s)) - When UE is not configured with uplink switching; or when UE is configured with uplink switching, and ‘fullCoherent’ codebook subset is supported in the corresponding carrier according to the capability uplinkTxSwitching-PUSCH-TransCoherence and/or uplinkTxSwitching2T2T-PUSCH-TransCoherence; or when UE is configured with uplink switching, ‘nonCoherent’ codebook subset is supported in the corresponding carrier according to the capability uplinkTxSwitching-PUSCH-TransCoherence and/or uplinkTxSwitching2T2T-PUSCH-TransCoherence, and uplink switching is not triggered by the switching mechanisms specified in sub-clause 6.1.6 of TS 38.214 [10] between last transmitted SRS and scheduled transmission.

Observation 2:	the phase variation between two antenna ports is quite large for both non-coherent UEs and coherent UEs

Based on observation 2, the configurations for 2Tx conduction test may not work for long time OTA test.

It can also be observed that some UEs supporting 2Tx (e.g. supporting TPMI=2) may also support 1Tx configuration, i.e., single antenna port transmission without Tx diversity. It seems necessary to distinguish the following two cases:

- Case a): UEs not only supporting 2Tx TPMI, but also supporting single antenna port transmission to achieve required maximum output power without Tx diversity, i.e., with at least one full power PA;

- Case b): UEs having to rely on 2Tx TPMI and/or Tx diversity to achieve required maximum output power, i.e., with no full power PA.

From current WID it is not very clear if above Case a) is in scope or not. It is proposed to discuss if UEs with at least one full power PA is in scope or not.

Proposal 3:	It is proposed to discuss if UEs with at least one full power PA are in scope or not.

In our understanding, it is not necessary to include UEs with at least one full power PA for the 2Tx configuration test for TRP. The test method has already been introduced in Rel-17 as 1Tx.

For the TRS test of NR 2Tx UE, there is no performance difference expected between 1Tx and 2Tx configuration though there are reference sensitivity requirements for Tx Diversity and UL MIMO respectively in TS 38.101-1. The latest trend indicated in FR2 is that the receiver requirement for UL MIMO has been removed recently, refer to details in CR [2]:

<<Start of Change>> 7.3D Void 7.4D Void 7.5D Void 7.6D Void <<End of Change>

From that sense, we think that the TRS test and requirement of NR 2Tx UE can be still based on 1Tx configuration as long as 1Tx configuration is available, for better test stability.

Proposal 4:	TRS test and requirement of NR 2Tx UE can be still based on 1Tx configuration as long as 1Tx configuration is available.

2.2	work plan

Multi-antenna UE test method has been extensively discussed in Rel-17 but no consensus achieved, thus this objective is left over to Rel-18. The technical difficulty lies in the phase variation during the long OTA test which leads to a time-varying radiation pattern. That is the difference between conduction test and OTA test.

To avoid repeating the same experience again at the end of Rel-18, we would highlight the following agreement achieved at the end of Rel-17, captured in the approved WF [3]:

Issue 1-2-1: How to evaluate multiple antenna system on UE?

Agreements:

- For multi-antenna system, the target is to develop the test methodologies with multi-antenna active if feasible.

- Test mode is a back-up solution for multi-antenna system measurement, and can be considered when no other valid test method identified at a specific time point in work plan.

Following the agreement, we propose to set up a specific time point in Rel-18 TRP TRS work plan to consider test mode if no other valid test method identified then.

Proposal 5:	it is proposed to set up a specific time point in Rel-18 TRP TRS work plan to consider test mode if no other valid test method identified, according to previous agreement.

3. 	Conclusion

Proposal 1:	DFT-s-OFDM rather than CP-OFDM is selected for TRP test of NR 2Tx UEs

Proposal 2:	1Tx TPMI (TPMI=0~1) is not in the work item scope for NR 2Tx configurations

Observation 1:	2Tx TPMI=2 could be supported by coherent UEs and fullpowerMode1 non-coherent UEs

Observation 2:	the phase variation between two antenna ports is quite large for both non-coherent UEs and coherent UEs

Proposal 3:	It is proposed to discuss if UEs with at least one full power PA are in scope or not.

Proposal 4:	TRS test and requirement of NR 2Tx UE can be still based on 1Tx configuration as long as 1Tx configuration is available.

Proposal 5:	it is proposed to set up a specific time point in Rel-18 TRP TRS work plan to consider test mode if no other valid test method identified, according to previous agreement.

4. 	References

- RP-222669,    “New WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies”, vivo

- R4-2213734,    “Draft CR for TS 38.101-2 Revision on UL MIMO receiver characteristics”, Huawei, Hisilicon

- R4-2214358,    “WF on FR1 TRP TRS for UE with multi-antenna and test time reduction”, OPPO