3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215705

# Electronic Meeting, October 10 – October 19, 2022

Title:	Necessity and feasibility of hand phantom MIMO OTA test method

Source:	Samsung

Agenda item:	6.15.3

Document for:	Discussion

1.	Introduction

It was agreed to enhance the FR1 and FR2 MIMO OTA in Rel-18 in RAN plenary September meeting and the new WID [1] shows that the necessity and feasibility study of FR1 hand phantom MIMO OTA test method is one of the scope:

  - FR1 MIMO OTA test methodology enhancement

    - Study, and if necessary, specify necessary enhancements of the FR1 MIMO OTA test methodology (test methodology defined in TS38.151 is the baseline) for the following device types in free space based on operators’ request.

      - For example: Tablet

      - Quiet zone of 20cm and 16-probe MPAC system should be unchanged

    - Study, and if necessary and feasible, specify enhancements of the FR1 MIMO OTA test methodology for Smartphone in browsing mode using hand phantom.

      - Test methodology defined in TR38.827 and TS38.151 is the baseline

In this contribution, the comparison between free space and hand phantom is provided and our view on necessity and feasibility of hand phantom MIMO OTA is presented.

2. 	Discussion

2.1	applicability rule

According to the WID objective [1] for FR1 MIMO OTA, possible enhancement for hand phantom shall take the test methodology defined in TR38.827 and TS38.151 as baseline, which means big change should be avoided. The basic FR1 test methodology in TR38.827 and TS38.151 is based on 2D MPAC with 16 uniformly-spaced dual-polarized probes.

Figure 1 (figure A.2.1-1 of TS38.151): MPAC system layout for NR FR1 MIMO OTA testing

Observation 1:	possible enhancement of FR1 MIMO OTA should reuse the 2D MPAC system layout with 16 uniformly-spaced dual-polarized probes specified in TR38.827 and TS38.151.

The test zone specified in TR38.827 and TS38.151 is 20cm considering the size of commercial mobile phones. However, when hand phantom is added, the diameter of DUT plus hand phantom will probably be larger than the 20cm test zone.

Observation 2:	the diameter of DUT plus hand phantom will probably be larger than the 20cm test zone specified in TR38.827 and TS38.151

Based on 2D MPAC test method, the overall performance of a UE should be averaged from several UE orientations. For free space MIMO OTA, the TRMS is averaged from three UE orientations: DMP, DML, and DMSU.

Figure 2: FS DMP, FS DML, FS DMSU

When hand phantom is considered, the DMSU orientation is not reasonable scenario. So fewer orientation scenarios are applicable for hand phantoms. Antenna performance is 3D OTA performance. When the antenna performance is verified with 2D test system, limited UE orientations will make the final TRMS not fairly averaged so that good UE and bad UE are not well distinguished.

Observation 3:	applicable hand phantom UE orientation is fewer which bring unfair averaged TRMS so that good UE and bad UE are not well distinguished.

Hand phantom for DMP orientation is similar as that of TRP TRS, however, hand phantom for DML is rather complicated. Refer to the analysis in TR37.977 as following:

Given a 2D ring of symmetrically distributed probes (as in Clause 6): - The XZ plane is similar to the DML mode except for the additional 45 degrees pitch in the DML case; - For the phantom case the tilt of the DML case is very relevant since the interaction of the phantoms with the antennas will depend on it – see Figure E.2-2

Depending on different hand phantom and different tilt of the phone, there will be many cases yielding different measurement results, summarized as following table:

|  | Hand phantom cases | DUT tilt cases |
| --- | --- | --- |
| hand phantom for DML | Left hand | Left tilt |
|  |  | Right tilt |
|  | Right hand | Left tilt |
|  |  | Right tilt |
|  | Both hands | Left tilt |
|  |  | Right tilt |

With so many hand grip scenarios, it is huge burden for measurement to only get the test result for DML orientation.

Observation 4:	hand phantom for DML orientation is complicated and bring large burden for test.

Even test huge test efforts would be made, according to observation 3, it is questionable if a fair TRMS result could be obtained to quantify the UE’s 3D OTA performance in practical usage.

As to UE performance in practical network, the motivation for installing more receiving antennas is not only to improve data throughput, but also to improve the communication robustness. The MIMO rank and MCS will adaptively vary with practical conditions. It is normal to fall back MIMO ranks and MCS to alleviate hand or body blockage. It is different case for TRP TRS. Hand phantom TRP TRS is essential for UE performance since call drop occurs in weak signal area if phantom TRP TRS performance is not good.

Observation 5:	MIMO rank and MCS fall back is normal UE behaviour to alleviate phantom blockage while SISO OTA with hand phantom is more essential.

Based above observations, we think it is a good combination to verify mobile phone’s OTA performance, i.e., free space MIMO OTA + phantom TRP TRS.

Proposal 1:	The OTA performance can be fully verified with the combination of free space MIMO OTA + phantom TRP TRS

3. 	Conclusion

Observation 1:	possible enhancement of FR1 MIMO OTA should reuse the 2D MPAC system layout with 16 uniformly-spaced dual-polarized probes specified in TR38.827 and TS38.151.

Observation 2:	the diameter of DUT plus hand phantom will probably be larger than the 20cm test zone specified in TR38.827 and TS38.151

Observation 3:	applicable hand phantom UE orientation is fewer which bring unfair averaged TRMS so that good UE and bad UE are not well distinguished.

Observation 4:	hand phantom for DML orientation is complicated and bring large burden for test.

Observation 5:	MIMO rank and MCS fall back is normal UE behaviour to alleviate phantom blockage while SISO OTA with hand phantom is more essential.

Proposal 1:	The OTA performance can be fully verified with the combination of free space MIMO OTA + phantom TRP TRS

4. 	References

- RP-222668,    “R18 New WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs”, CAICT, OPPO, Keysight Technologies