3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215710

Electronic Meeting, Oct. 10-19, 2022

Agenda item:	6.8.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on FR2 multi Rx chain DL reception

Document for:	Discussion

- Introduction

At the last RAN4 #104-e meeting, the discussion of Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception was started and workplan and WF were approved [1][2]. During last meeting, possible enhancement aspects are raised based on objective, however almost all issues are still diverse.

According to the agreed workplan, this contribution tries to conclude the study/feasibility/necessity of the RRM work and proposes core RRM requirements.

- Discussion

  - RRM study assumption of multi Rx chain for FR2

Enhanced NR FR2 UEs with multi-beam simultaneous reception and multiple RX chains can provide a meaningful performance improvement in FR2 improving both demodulation performance (4-layer DL MIMO), RRM performance and improve RF spherical coverage. This work item aims to introduce the requirements for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier to achieve improved RF, RRM and UE demodulation performance.

During last meeting, the discussion direction was quite diverse since each company has different view of assumption of this study. Here we again overview the justification and objective and try to fix the direction of RRM discussions. The part of justification is duplicated as follows:

One of the discussion points raised in previous meeting is whether RRM requirements should be related to the 4-layer MIMO study or not. The justification above seems to describe performance improvement of Demod/RRM/RF separately. Therefore RRM discussion have not to be related to 4-layer MIMO study directly, i.e., pure RRM enhancement study thanks to multi Rx chain should be discussed.

Observation 1: The justification in the WID seems to describe performance improvement of Demod/RRM/RF separately.

Proposal 1: RRM discussion have not to be related to 4-layer MIMO study directly, i.e., pure RRM enhancement study thanks to multi Rx chain should be discussed.

Based on this observation, we again propose which RRM aspects can be enhanced by using multi RF chain.

  - Clarification of scope

In WF, all the remaining issues are listed as proposals. However some of these proposals should be treated in RF/Demod or as out of scope.

Proposal 2: The issues listed below should be out of scope of RRM session:

- Issue 1-1-1-3: TAG assumption for uplink transmission

  - This should be out of scope. The scope of this WI is DL reception.

- Issue 1-1-1-4: Whether to define activation delay from a single antenna panel to multi-antenna panels

  - This needs RF study conclusion. The previous RF discussion agreed that FFS whether the single panel should be excluded. Thus, first this should be discussed in RF session and multi-antenna panel should be the baseline for RRM study.

- Issue 1-1-1-5: RRM impact of the UE behaviour using a single antennal panel

  - This needs RF study conclusion. The previous RF discussion agreed that FFS whether the single panel should be excluded. Thus, first this should be discussed in RF session and multi-antenna panel should be the baseline for RRM study.

- Issue 1-1-1-6: Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel

  - This should be out of scope. Spatial MIMO condition is not directly related to RRM study.

- Issue 1-1-1-9: How to handle FR2 SCell activation delay reduction by multi-Rx chain simultaneous reception

  - This should be out of scope. SCell activation delay should be handled in RRM enhancement WI.

- Issue 1-1-2-2: Simultaneous L3 measurements and L1 measurements

  - This should be out of scope. L1 measurement enhancement and L3 measurement (if necessary) should be sepalately studied first.

- Issue 1-1-2-4: Whether and how to define power saving related requirements

  - This should be out of scope. Power saving related requirements are not covered by WID.

- Issue 1-1-2-7: Accuracy assumption when defining RRM core requirements

  - This should be out of scope of core requirements.

- Issue 1-1-2-9: Requirements applicability for serving cells

  - This should be out of scope so far. If time allows, it will be discussed later.

- Issue 1-1-6-3: UE behaviour and capability of handling Rx signal level difference between two channels

  - This should be out of scope. This should be discussed in RF session.

- Issue 1-1-7: UE architectures

  - This should be out of scope. This should be discussed in RF session.

- Issue 1-2-3-4: Requirements enhancement for inter-cell L1-RSRP measurement

  - This should be out of scope so far. If time allows, it will be discussed later.

- Issue 1-2-3-5: Number of cells with PCI different from serving cells for inter-cell BM

  - This should be out of scope so far. If time allows, it will be discussed later.

  - L1 measurement related issues

Based on above proposals, the RRM work should assume multi panel utilization for RRM enhancements. It means that UE can receive different signals simultaneously from different direction and then these measurement and scheduling restrictions can be mitigated. In other words, the UE which has multi Rx chain can receive different QCL-Type D RSs simultaneously, and then measured RS samples, measurement restriction and scheduling availability can be mitigated. Based on this observation, the following RRM enhancements can be studied.

    - Measurement delay

For both SSB based and CSI-RS based L1-RSRP measurement, the measurement delay consists of following factors:

- Number of samples N

  - For SSB based, N=8 (fixed value)

  - For CSI-RS based, especially for periodic/semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. Otherwise N=1.

- Scaring factor P

For both number of samples and scaring factor, it can be relaxed if UE has multi Rx chain and receives signals from different direction because these factors are originally for UE beam sweeping consideration. Thus these factors should be studied for multi Rx chain UE.

Proposal 3: New number of samples N and scaring factor P defined in L1-RSRP measurement requirements should be studied for multi Rx UE.

    - Measurement restriction

Measurement restriction for SSB based L1-RSRP

[…]

For FR2, when the SSB for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

[…]

Measurement restriction for CSI-RS based L1-RSRP

[…]

For FR2, when the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and the other CSI-RS. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

-	The CSI-RS for L1-RSRP measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in q1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction.

[…]

In the current spec, following measurement restrictions are specified for both SSB based and CSI-RS based.

These measurement restrictions are defined based on that UE cannot receive multiple RSs simultaneously. However, if UE has multi Rx chain, these restrictions can be removed or relaxed.

Proposal 4: Measurement restrictions for both SSB based and CSI-RS based L1-RSRP should be studied whether it can be removed or relaxed.

    - Scheduling availability during L1-RSRP measurement

Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to L1-RSRP measurement.

-	For the case where RS for L1-RSRP measurement is CSI-RS which is QCLed with active TCI state for PDCCH/PDSCH and not in a CSI-RS resource set with repetition ON, and N=1 applies as specified in clause 9.5.4.2

-	There are no scheduling restrictions due to L1-RSRP measurement performed based on the CSI-RS.

-	In non-HST scenario, for FR2-1 or the reference symbols to be measured for L1-RSRP is not using 480 kHz SCS or 960 kHz SCS on FR2-2, tthe UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on

-	symbols corresponding to the SSB indexes configured for L1-RSRP measurement, and/or

-	symbols corresponding to the periodic CSI-RS resource configured for L1-RSRP measurement, and/or

-	symbols corresponding to the semi-perssitent CSI-RS resource configured for L1-RSRP measurement when the resource is activated, and/or

-	symbols corresponding to the aperiodic CSI-RS resource configured for L1-RSRP measurement when the reporting is triggered.

[…]

In HST scenario, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on

-	symbols corresponding to the SSB indexes configured for L1-RSRP measurement and 1 data symbol before each consecutive SSB symbols to be measured for L1-RSRP and 1 data symbol after each consecutive SSB symbols to be measured for L1-RSRP, and/or

-	symbols corresponding to the periodic CSI-RS resource configured for L1-RSRP measurement and 1 data symbol before each periodic CSI-RS resource to be measured for L1-RSRP and 1 data symbol after each periodic CSI-RS for L1-RSRP measurement symbols to be measured for L1-RSRP, and/or

-	symbols corresponding to the semi-persistent CSI-RS resource configured for L1-RSRP measurement and 1 data symbol before each semi-persistent CSI-RS resource to be measured for L1-RSRP and 1 data symbol after each semi-persistent CSI-RS resource to be measured for L1-RSRP when the resource is activated, and/or

-	symbols corresponding to the aperiodic CSI-RS resource configured for L1-RSRP measurement and 1 data symbol before each aperiodic CSI-RS resource to be measured for L1-RSRP measurement and 1 data symbol after each aperiodic CSI-RS resource to be measured for L1-RSRP measurement when the reporting is triggered.

[…]

In the current spec, following scheduling restrictions are specified for FR2-1 L1-RSRP measurement.

These scheduling restrictions are defined based on that UE cannot receive/transmit RSs and data simultaneously. However, if UE has multi Rx chain, these restrictions can be removed or relaxed.

Proposal 5: Scheduling restrictions for both SSB based and CSI-RS based L1-RSRP should be studied whether it can be removed or relaxed.

  - RLM and BFD/CBD requirements

The basic structure of measurement requirements for RLM and BFD/CBD are same as that of L1-RSRP measurements. Therefore the same studies for L1-RSRP measurements are needed for specifying the requirements for multi Rx UE.

Proposal 6: New number of samples N and scaring factor P defined in RLM and BFD/CBD requirements should be studied for multi Rx UE.

Proposal 7: Measurement restrictions for both SSB based and CSI-RS based RLM and BFD/CBD should be studied whether it can be removed or relaxed.

Proposal 8: Scheduling restrictions for both SSB based and CSI-RS based RLM and BFD/CBD should be studied whether it can be removed or relaxed.

In addition for BFD/CBD, the requirements for multi TRP case are sepalately specified with multi TRP specific parameters. Especially the scaling factor PTRP is defined as fixed value 2 if resource set of each TRP are overlapped. This value can be relaxed when UE has multi Rx chain.

Proposal 9: The scaling factor PTRP should be studied for multi Rx UE whether it can be relaxed.

  - L3 measurement delay

    - Cell Re-selection delay

About L3 measurement delay, there are several scaling factors related to FR2 beam sweeping as well. During cell re-celection procedure, UE measures candidata cell quality (SS-RSRP/SS-RSRQ). The minimum requirement of the evaluation durations are defined by DRX cycle length with a number of samples N1. The same reason of L1-RSRP measurement, it should be studied for multi Rx UE whether the N1 value can be reduced or not.

Proposal 10: New N1 value for cell re-selection should be studied for multi Rx UE.

    - Scaling factor for intra-frequency measurement/inter-frequency measurement without gap

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by SSB-ToMeasure and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

For intra-frequency measurement and inter-frequency measurement without gap, the scaling factor of Klayer1_measurement is specified as follows.

This scaling factor Klayer1_measurement is also introduced because of FR2 analog beam characteristics. Therefore if UE has multi Rx chain, it can be removed or relaxed.

Proposal 11: Scaling factor Klayer1_measurement should be studied whether it can be removed or relaxed.

- Conclusion

In this contribution, we proposed our views on FR2 multi Rx chain DL reception requirements.

Observation 1: The justification in the WID seems to describe performance improvement of Demod/RRM/RF separately.

Proposal 1: RRM discussion have not to be related to 4-layer MIMO study directly, i.e., pure RRM enhancement study thanks to multi Rx chain should be discussed.

Proposal 2: The issues listed below should be out of scope of RRM session:

- Issue 1-1-1-3: TAG assumption for uplink transmission

  - This should be out of scope. The scope of this WI is DL reception.

- Issue 1-1-1-4: Whether to define activation delay from a single antenna panel to multi-antenna panels

  - This needs RF study conclusion. The previous RF discussion agreed that FFS whether the single panel should be excluded. Thus, first this should be discussed in RF session and multi-antenna panel should be the baseline for RRM study.

- Issue 1-1-1-5: RRM impact of the UE behaviour using a single antennal panel

  - This needs RF study conclusion. The previous RF discussion agreed that FFS whether the single panel should be excluded. Thus, first this should be discussed in RF session and multi-antenna panel should be the baseline for RRM study.

- Issue 1-1-1-6: Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel

  - This should be out of scope. Spatial MIMO condition is not directly related to RRM study.

- Issue 1-1-1-9: How to handle FR2 SCell activation delay reduction by multi-Rx chain simultaneous reception

  - This should be out of scope. SCell activation delay should be handled in RRM enhancement WI.

- Issue 1-1-2-2: Simultaneous L3 measurements and L1 measurements

  - This should be out of scope. L1 measurement enhancement and L3 measurement (if necessary) should be sepalately studied first.

- Issue 1-1-2-4: Whether and how to define power saving related requirements

  - This should be out of scope. Power saving related requirements are not covered by WID.

- Issue 1-1-2-7: Accuracy assumption when defining RRM core requirements

  - This should be out of scope of core requirements.

- Issue 1-1-2-9: Requirements applicability for serving cells

  - This should be out of scope so far. If time allows, it will be discussed later.

- Issue 1-1-6-3: UE behaviour and capability of handling Rx signal level difference between two channels

  - This should be out of scope. This should be discussed in RF session.

- Issue 1-1-7: UE architectures

  - This should be out of scope. This should be discussed in RF session.

- Issue 1-2-3-4: Requirements enhancement for inter-cell L1-RSRP measurement

  - This should be out of scope so far. If time allows, it will be discussed later.

- Issue 1-2-3-5: Number of cells with PCI different from serving cells for inter-cell BM

  - This should be out of scope so far. If time allows, it will be discussed later.

Proposal 3: New number of samples N and scaring factor P defined in L1-RSRP measurement requirements should be studied for multi Rx UE.

Proposal 4: Measurement restrictions for both SSB based and CSI-RS based L1-RSRP should be studied whether it can be removed or relaxed.

Proposal 5: Scheduling restrictions for both SSB based and CSI-RS based L1-RSRP should be studied whether it can be removed or relaxed.

Proposal 6: New number of samples N and scaring factor P defined in RLM and BFD/CBD requirements should be studied for multi Rx UE.

Proposal 7: Measurement restrictions for both SSB based and CSI-RS based RLM and BFD/CBD should be studied whether it can be removed or relaxed.

Proposal 8: Scheduling restrictions for both SSB based and CSI-RS based RLM and BFD/CBD should be studied whether it can be removed or relaxed.

Proposal 9: The scaling factor PTRP should be studied for multi Rx UE whether it can be relaxed.

Proposal 10: New N1 value for cell re-selection should be studied for multi Rx UE.

Proposal 11: Scaling factor Klayer1_measurement should be studied whether it can be removed or relaxed.

References

[1] 3GPP, R4-2213053, vivo, Qualcomm, “Work plan for RRM requirement for NR FR2 multi-Rx chain DL reception”

[2] 3GPP, R4-2213433, vivo, “WF on FR2 multi-Rx RRM”.