3GPP TSG-RAN WG4 Meeting #104bis-e                                            	R4-2215726

Electronic Meeting, October 10 – October 19, 2022

Source:	Samsung

Title:	Discussion and simulation results on demodulation requirement for Enhancement on HST-SFN scenario

Agenda item:	4.5.3.1.1

Document for:	Discussion

- Introduction

In last RAN4 104-e meeting, test setup for PDSCH requirement for HST-SFN scheme A with single carrier has agreements in WF [1] as below. In this contribution, we share our simulation results and views for information.

- Sub-topic 2-2: Test Setup for PDSCH requirement for SFN scheme A with single carrier - Sub-topic 2-2-1: Maximum Doppler Shift Agreement: - Define PDSCH requirement with HST SFN scheme A with 870Hz Doppler shift for 15KHz SCS - Sub-topic 2-2-2: MCS and Rank Agreement: - Further discuss and decide in next Ran4 meeting with following options - MCS 13 with Rank 2 - MCS 17 with Rank 2 - It’s encouraged companies can provide results for both options under FDD and TDD modes. - RAN4 will decide based on the alignment outcome among companies’ results and whether peak TP can be achieved or not. - Sub-topic 2-2-3: UE capability Way forward: - The PDSCH demodulation requirements for HST-SFN Scheme A should be applicable for UE capable of “23-6-1 SFN scheme A (scheme 1) for PDSCH and PDCCH”

Besides, there is conclusion on TRP pre-compensation modelling for HST-SFN scheme B.  In this contribution we also provide our simulation results based on the agreed TRP pre-compensation modelling in [1].

- Sub-topic 2-1: Test Scope - Issue 2-1-1: Whether to define PDSCH requirement with HST-SFN scheme B Agreement: - Introduce PDSCH requirement with HST-SFN scheme B with test applicability rule - [If UE support HST-SFN scheme A and pass the test of HST SFN scheme A, it can skip the test of HST SFN scheme B] - Issue 2-1-2: Modelling of TRP pre- compensation Agreement: - Assume perfect modeling of TRP pre-compensation

- Simulation results of SFN scheme A

We evaluate SFN scheme A performance based on below simulation assumptions.

Channel model we used is similar as 3GPP38.101-4 B.3.2 HST-SFN [2], removing 2 taps.

- Simulation results for FDD and TDD

Fig.1 Throughput performance of MCS13 and MCS17

The SNR points at 70% of peak rate for different test cases are summarized in the following table：

Table 1: SNR point at 70% peak rate of different cases

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) point |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 13 | 870 | 2x2 | 9.3 |
| 2 | FDD | 13 | 870 | 2x4 | 5.8 |
| 3 | FDD | 17 | 870 | 2x2 | 14.2 |
| 4 | FDD | 17 | 870 | 2x4 | 11.0 |
| 5 | TDD | 13 | 1667 | 2x2 | 9.5 |
| 6 | TDD | 13 | 1667 | 2x4 | 6.6 |
| 7 | TDD | 17 | 1667 | 2x2 | 16.5 |
| 8 | TDD | 17 | 1667 | 2x4 | 13.7 |

Observation1: for HST-SFN scheme A, both FDD and TDD, MCS13 related cases could achieve peak throughput, while MCS17 related cases could not achieve peak throughput.

Observation2: for HST-SFN scheme A, both FDD and TDD, MCS17 related cases could get better throughput performance than 70% of peak throughput. For FDD MCS17 case could reach 98% of peak throughput, for TDD MCS17 case could reach 88% of peak throughput.

Proposal 1: for HST-SFN scheme A, MCS17 and rank2 should be used for FDD PDSCH requirement setup, while MCS13 and rank2 should be used for TDD PDSCH requirement setup.

Proposal 2: considering the impairment margin, SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme A with single carrier

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 17 | 870 | 2x2 | 16.7 |
| 2 | FDD | 17 | 870 | 2x4 | 13.5 |
| 3 | TDD | 13 | 1667 | 2x2 | 12.0 |
| 4 | TDD | 13 | 1667 | 2x4 | 9.1 |

- Simulation results of SFN scheme B

According the WF [1], Simulation assumption of scheme B is as same as scheme A, except the Channel model. This document provide simulation result of perfect modeling of TRP pre-compensation.

Fig.2 Throughput performance of MCS13 and MCS17

Table 2: SNR point at 70% peak rate of different cases

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) point |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 13 | 870 | 2x2 | 8.7 |
| 2 | FDD | 13 | 870 | 2x4 | 5.4 |
| 3 | FDD | 17 | 870 | 2x2 | 12.8 |
| 4 | FDD | 17 | 870 | 2x4 | 9.9 |
| 5 | TDD | 13 | 1667 | 2x2 | 8.9 |
| 6 | TDD | 13 | 1667 | 2x4 | 6.3 |
| 7 | TDD | 17 | 1667 | 2x2 | 14.5 |
| 8 | TDD | 17 | 1667 | 2x4 | 12.1 |

Observation3: for HST-SFN scheme B, both FDD and TDD, MCS13 related cases could achieve peak throughput, while MCS17 related cases could not achieve peak throughput.

Observation4: for HST-SFN scheme B, both FDD and TDD, MCS17 related cases could get better throughput performance than 70% of peak throughput. For FDD MCS17 case could reach 98% of peak throughput, for TDD MCS17 case could reach 90% of peak throughput.

Proposal 3: for HST-SFN scheme B, MCS17 and rank2 should be used for FDD PDSCH requirement setup, while MCS13 and rank2 should be used for TDD PDSCH requirement setup.

Proposal 4: considering the impairment margin, SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme B with single carrier

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 17 | 870 | 2x2 | 15.3 |
| 2 | FDD | 17 | 870 | 2x4 | 12.4 |
| 3 | TDD | 13 | 1667 | 2x2 | 11.4 |
| 4 | TDD | 13 | 1667 | 2x4 | 8.8 |

- Conclusion

In this contribution, we provide simulation results for PDSCH demodulation requirement for Enhancement on HST-SFN deployment, based on the agreements in RAN4 #104-e meeting. We compare different test cases under FDD and TDD mode. According to the above results, we observe that:

Observation1: for HST-SFN scheme A, both FDD and TDD, MCS13 related cases could achieve peak throughput, while MCS17 related cases could not achieve peak throughput.

Observation2: for HST-SFN scheme A, both FDD and TDD, MCS17 related cases could get better throughput performance than 70% of peak throughput. For FDD MCS17 case could reach 98% of peak throughput, for TDD MCS17 case could reach 88% of peak throughput.

Proposal 1: for HST-SFN scheme A, MCS17 and rank2 should be used for FDD PDSCH requirement setup, while MCS13 and rank2 should be used for TDD PDSCH requirement setup.

Proposal 2: considering the impairment margin, SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme A with single carrier

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 17 | 870 | 2x2 | 16.7 |
| 2 | FDD | 17 | 870 | 2x4 | 13.5 |
| 3 | TDD | 13 | 1667 | 2x2 | 12.0 |
| 4 | TDD | 13 | 1667 | 2x4 | 9.1 |

Observation3: for HST-SFN scheme B, both FDD and TDD, MCS13 related cases could achieve peak throughput, while MCS17 related cases could not achieve peak throughput.

Observation4: for HST-SFN scheme B, both FDD and TDD, MCS17 related cases could get better throughput performance than 70% of peak throughput. For FDD MCS17 case could reach 98% of peak throughput, for TDD MCS17 case could reach 90% of peak throughput.

Proposal 3: for HST-SFN scheme B, MCS17 and rank2 should be used for FDD PDSCH requirement setup, while MCS13 and rank2 should be used for TDD PDSCH requirement setup.

Proposal 4: considering the impairment margin, SNR point at 70% peak rate for PDSCH requirement for HST-SFN scheme B with single carrier

| Case Number | Duplex | MCS level | Maximum Doppler(Hz) | Antenna Configuration | SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 17 | 870 | 2x2 | 15.3 |
| 2 | FDD | 17 | 870 | 2x4 | 12.4 |
| 3 | TDD | 13 | 1667 | 2x2 | 11.4 |
| 4 | TDD | 13 | 1667 | 2x4 | 8.8 |

Reference

[1] R4-2214397, “WF on UE demodulation and CSI requirements for FeMIMO”, Samsung, 3GPP RAN4 #104-e meeting

[2] 3GPP TS 38.101-4 v17.3.0, “NR User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 17)”