3GPP TSG-RAN WG4 #104bis-e	R4-2215729

# Electronic Meeting, Oct 10th – 19th, 2022

Agenda item:	9.1

Source: 	Spreadtrum Communication

Title: 	BWP operation without bandwidth restriction

Document for:	Discussion

Introduction

RAN2 sent the LS [1] to ask questions regarding BM/RLM/BFD in the case of SSB outside active BWP. RAN4 discussed the questions and came up with several options. Then the reply LS [2] collecting the options was submitted to RAN 97-e for decision, still RAN tasks RAN4 to make further analysis of the options.

The conclusion in RAN97-e is copied as below:

- 	No new solution for FG 6-1a shall be added to Rel-17

- If CSI-RS based RLM/BM/BFD are supported by a UE, FG6-1a can work without any issue. FG1-7 (CSI-RS based

RLM) and FG 2-24 (SSB/CSI-RS for beam measurement) are mandatory with capability signalling features.

- 	No change to TU allocation for current RAN4 work in Q4 2022.

- RAN asks RAN4 to do a high level analysis of the options (copied below) in RAN4’s answer to Q2 in RP-221911

and report it to RAN#98 for RAN decision.

Options from RP-221911:

a) Perform BM/RLM/BFD based on CSI-RS within active BWP

b) Perform BM/RLM/BFD based on SSB outside active BWP

i)  UE’s capability to operate using larger BW covering SSB outside active BWP, or a UE that is equipped

with a separate RF chain

ii)  BM/RLM/BFD on SSB outside BWP are performed with shared MG or NCSG for L3 measurement, or

dedicated MG or NCSG for RLM/BFD/BM measurements.

c) NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

Discussion

The discussion paper presents Spreadtrum’s views on each options concluded in RAN 97e [3]

Perform BM/RLM/BFD based on CSI-RS within active BWP

According to [3], CSI-RS based RLM/BM/BFD is an optimum choice in release-17, most companies are positive to CSI-RS based RLM/BM/BFD since it is mandatory feature with no spec impact. However the OFDM-based system is vulnerable to time and frequency offset, how to achieve the same time and frequency tracking effect as it does when SSB is in the active BWP, additional CRS-RS for TRS might be required in the active BWP.

Observation 1: CSI-RS for TRS might be required for time and frequency tracking in the case of SSB outside active BWP.

Perform BM/RLM/BFD based on SSB outside active BWP

Larger bandwidth covering SSB outside active BWP leads to excessive power consumption, even though there is argument that higher throughput in a short time is actually power efficient in comparison with lower throughput in a longer time, because it assumes favorable traffic model, and ideal network algorithm to adaptive to the traffic model. For example, in the case of voice traffic model with continuous low data burst, we could foresee larger BW keeping on for the entire connection time.

Observation 2: Power efficiency with respect to the way of larger BW is both traffic model dependent and network algorithm dependent

Making use of vacant RF chain to perform BM/RLM/BFD leads to either excessive power consumption when the RF chain keeps on or causes traffic interruption due to RF chain on and off switching.

Observation 3: It’s hardware dependent and at the cost of either power efficiency or data interruption to perform BM/RLM/BFD based on SSB outside active BWP using spare RF chain.

Perform BM/RLM/BFD based on SSB outside active BWP with shared MG/NCSG with L3 measurement or dedicated MG/NCMG.

To accommodate the BM/RLM/BFD relevant L1 measurements into MGs/NCMG, there would be necessity to access the RS more frequently than L3 measurement, so it’s required to increase measurement opportunities.

Observation 4: To accommodate the BM/RLM/BFD relevant measurements into MGs/NCMG, it will impose data throughput loss on UE.

NCD-SSB approach

NCD-SSB would be configured for release-17 UE, it could also be applied by UEs to implement” FG 6-1a BWP without restriction” in a cost efficient way.

Observation 5: The NCD-SSB adopted by release-17 UE would be applied by UEs to support” FG 6-1a BWP without restriction”

Proposal 1: It’s proposed to study NCD-SSB for the UEs to support” FG 6-1a BWP without restriction”

Conclusion

In this paper we discussed our views on each options concluded in RAN 97-e when SSB used for BM/RLM/BFD is outside active BWP. We made the following observations:

.Observation 1: CSI-RS for TRS might be required for time and frequency tracking in the case of SSB outside the active BWP.

Observation 2: Power efficiency with respect to the way of larger BW is both traffic model dependent and network algorithm dependent

Observation 3: It’s hardware dependent and at the cost of either power efficiency or data interruption to perform BM/RLM/BFD based on SSB outside active BWP using spare RF chain.

Observation 4: To accommodate the BM/RLM/BFD relevant measurements into MGs/NCMG will impose data throughput loss on.

Observation 5: The NCD-SSB adopted by release-17 UE would be applied by UEs to support” FG 6-1a BWP without restriction”

Proposal 1: It’s proposed to study NCD-SSB for the UEs to support” FG 6-1a BWP without restriction”

References

- R4-2204009, “LS on BWP operation without bandwidth restriction”, TSG-RAN2

- RP-221911,“LS on Feature Group 6-1a “bwp-WithoutRestriction””

- RP-222630, “Summary for [97e-05-BWP-WithoutRestriction]”