3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215745

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			4.5.2.2

Source:			Samsung

Title:	Discussion on remaining issues of test cases for unified TCI state

Document for:		Discussion

- Introduction

In RAN4#104-e meeting, all unified TCI state test cases are defined. A WF on RRM performance part was approved in [1].

In this contribution, we provide our consideration of open issues and give our proposals.

- Discussion

For the joint TCI test cases, whether to configure pathloss RS is still FFS.

Pathloss RS configuration in joint TCI test case - Option 1: Pathloss RS is maintained - Option 2: Do not configure pathloss RS in joint TCI case

In our view, either option 1 or option 2 can work. We slightly prefer option 2 since pathloss RS is optional in TCI. PL-RS has already been tested in other test cases.

Proposal 1: For PL-RS configuration in joint TCI test case, prefer not to configure pathloss RS.

How to configure maintained PL-RS / NOT maintained PL-RS in the test case - Proposal 1: If an original PL-RS is configured, it is a maintained PL-RS; If a new RS is configured as PL-RS, it is a “not maintained PL-RS” - Proposal 2: Whether to maintain PL-RS is up to UE implementation. The value of delay in test requirement can be calculated based on the worst case. - Proposal 3: Before sending the TCI switch command, UE is expected to measure on the active TCI list and maintain the PL-RS. - Proposal 4: For non-maintain PL-RS, PL-RS is configured in TCI state and TCI state is unknown.

PL-RS is maintained or not maintained, there is no such definition in the RAN1/2 spec. But in the test cases set-up, if a new RS is configured as PL-RS, it is certainly that UE uses “not maintained PL-RS”. But if an original PL-RS is configured, is it not so clear it is maintained or not maintained PL-RS from UE implementation. We propose to only define the test cases for PL-RS is not maintained.

Proposal 2: In the test cases, only define the test cases for PL-RS is not maintained. For the test setup, configure a new RS as PL-RS, it is a “not maintained PL-RS”.

- Conclusion

In this contribution, we provide our consideration of unified TCI state test cases and our proposals are:

Proposal 1: For PL-RS configuration in joint TCI test case, prefer not to configure pathloss RS.

Proposal 2: In the test cases, only define the test cases for PL-RS is not maintained. For the test setup, configure a new RS as PL-RS, it is a “not maintained PL-RS”.

- Reference

[1] R4-2214483, WF on FeMIMO RRM Performance Requirement and Test Case, Samsung