3GPP TSG-RAN WG4 Meeting # 104-bis-e													R4-2215752

Electronic Meeting, 10– 19 Oct. 2022

Agenda item:			4.2.6.1

Source:	MediaTek Inc.

Title:	Discussion on RRM performance for NR NTN

Document for:	Discussion

# Introduction

In this contribution, measurement accuracy requirements for NTN RRM are discussed.

# Measurement accuracy requirements for NTN RRM

In the last meeting, the extra margin for measurement accuracy for the timing/frequency error caused by propagator model was discussed as in WF [1].

Issue 1-1: Margin assumption for evaluating measurement accuracy.

- Agreement

  - RAN4 considers propagator model error and timing/frequency error when defining the measurement accuracy.

    - FFS on the values of propagator model error and timing/frequency error

    - FFS on the definition of propagator model error

As discussed in the issue 1-1, although the timing/frequency error could be 0.29 us/46Hz with prediction 45s ahead based on the Eckstein Hechler model [2], it can be observed that the timing error is small comparing to the CP and the frequency error is also small. For example, the timing error of 0.29 us is ~6% CP of SCS 15 kHz. Thus, no significant impact on measurement accuracy is expected and the legacy measurement accuracy can be reused.

Observation 1: Although considering propagator model error, no significant impact on measurement accuracy is observed.

Proposal 1: Reuse the legacy TN measurement accuracy requirements for NTN.

# Summary

In this contribution, the measurement accuracy requirements for NTN RRM has been discussed, the observation/proposal are summarized as below:

Observation 1: Although considering propagator model error, no significant impact on measurement accuracy is observed.

Proposal 1: Reuse the legacy TN measurement accuracy requirements for NTN.

# Reference

- R4-2214473, “WF on performance part for NR NTN”, Xiaomi, RAN4#104-e meeting, August 2022.

- R4-2214267, “Email discussion summary for [104-e][215] NR_NTN_solutions_RRM_2”, Xiaomi, RAN4#104-e meeting, August 2022.