3GPP TSG-RAN WG4 Meeting # 104-bis-e													R4-2215753

Electronic Meeting, 10– 19 Oct. 2022

Agenda item:			7.5.6

Source:	MediaTek Inc.

Title:	RRM requirements for LTE NB-IoT/eMTC over NTN

Document for:	Discussion

# Introduction

A new Work Item of “LTE_NBIOT_eMTC_NTN_req” has been approved in RAN Plenary #95-e meeting to define requirements for R17 IoT NTN. In this contribution, we provide an overview on open issues of RRM requirements, as discuss in the WF [1] in the last RAN4 meeting, including the following aspects:

- General discussion for NTN IoT specific topics

- IDLE state mobility requirements

- CONNECTED state mobility requirements

- Timing requirements and RLM

- Measurement requirements

# General discussion for RRM requirements

The general approach for RRM Requirements is to follow similar principles as for NR NTN (alluded to in the WID), i.e. reuse the majority of existing TN requirements with necessary NTN specific modification.

For NGSO, the following NTN IoT specific functionality will be discussed in this section:

- T317 expiry and SIB31

- t-Service in SIB3 of the serving cell

- Discontinuous coverage and SIB32

Besides, the following remaining issues in the last meeting are also discussed.

- For NGSO, DRX/eDRX, HD-FDD applicability

- NTN related RAN4 UE features introduced in Rel-17 NR

- UE measurement capability of monitoring on number of NGSO satellites

- For NGSO, Doppler shift impact in Multiple NGSO satellites.

## T317 expiry and SIB31

In the last meeting, the impact of T317 expiry has been discussed. The below is an excerpt from TS 36.331, and it can be observed that the UE is required to acquire the SIB31 upon T317 expiry. Noting that it is for CONNCETED state and T317 is not available in IDLE.

#### 5.2.1.3	System information validity and notification of changes

....

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT). In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE 2:    Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

NOTE 3:    When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

### 7.3.1	Timers (Informative)

….

| T317 NOTE1 | Upon acquisition of SystemInformationBlockType31 |  | In RRC_CONNECTED mode, initiate acquisition of SystemInformationBlockType31 in accordance with 5.3.3.21. |
| --- | --- | --- | --- |

Observation 1: In CONNECTED state, UE is required to acquire the SIB31 upon T317 expiry. T317 is not available in IDLE state.

## t-Service in SIB3 of the serving cell

In the last meeting, the impact of t-Service has been discussed. The below is an excerpt from TS 36.304, and it can be observed that the UE is required to perform neighbouring cell measurement regardless S criteria when t-Service in SIB3/SIB3-NB of the serving cell is provided and applicable. Noting that the exact time to start measurements before t-Service is up to UE implementation.

// M1

#### 5.2.4.2	Measurement rules for cell re-selection

...

If t-Service is present in SystemInformationBlockType3 of the serving cell, UE shall perform intra-frequency, inter-frequency or inter-RAT measurements, before the time t-Service regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the exact time to start measurements before t-Service is up to UE implementation. UE shall perform measurements of higher priority inter-frequencies or inter-RAT frequencies regardless of the remaining service time of the serving cell.

// NB IoT

#### 5.2.4.2a	Measurement rules for cell re-selection for NB-IoT

...

If t-Service is present in SystemInformationBlockType3-NB of the serving cell, UE shall perform intra-frequency or inter-frequency measurements before the time t-Service regardless whether the serving cell fulfils Srxlev > SIntraSearchP or Srxlev > SnonIntraSearchP, and the exact time to start measurements before t-Service is up to UE implementation.

Proposal 1: In IDLE state, if t-Service of the serving cell is provided and applicable, UE is required to perform neighbouring cell measurement regardless S criteria. The exact time to start measurements before t-Service is up to UE implementation.

## Discontinuous coverage and SIB32

In the last meeting, whether and how to define the requirement for discontinuous coverage has been discussed.  The below is an excerpt from TS 36.304, and it can be observed UE doesn’t need to perform idle mode task if it is out of coverage is SIB32 is provided and applicable. Besides, the detection of out of coverage is up to UE implementation based on SAI (e.g. SIB32, SIB31, t-Service).

# 4	General description of Idle mode

## 4.1	Overview

…

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in coverage the UE shall perform all idle mode tasks.

Proposal 2: In IDLE state, if SIB32 is provided and applicable, UE is not required to perform any cell measurement while out of coverage in discontinuous coverage. The detection of out of coverage using satellite assistance information is up to UE implementation. IDLE state requirements apply when UE is once in coverage.

## NGSO, DRX/eDRX, HD-FDD applicability

Agreement in R17 NR NTN in RAN4 101-b [2]

- Whether and what DRX cycle length to configure is up to NW, but UE is not required to fulfil the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.

In general, the existing TN requirements related to DRX/eDRX, HD-FDD can be re-used as baseline, although eDRX and HD-FDD are not discussed in NR NTN. However, for LEO/NGSO, the limitation on DRX, eDRX (PTW) can be further discussed, e.g., long DRX cycle and short PTW may not be applicable for LEO/NGSO in Enhanced coverage (EC). Similar discussion can be found in R17 NR NTN as in [2], where UE is not required to fulfil the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. Same conclusion of NR NTN can be applied to NTN IoT.

Proposal 3: In LEO, whether and what DRX cycle length to configure is up to NW, but UE is not required to fulfil the requirements for DRX cycle length ≧ 2.56s.

## NTN related RAN4 UE features introduced in Rel-17 NR

In the last meeting, whether to introduce the following UE capability for LTE IoT NTN in Rel-18 has been discussed.

  - Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

  - Relaxed cell reselection on GEO

  - Parallel measurements on multiple LEO satellites

Regarding the enhanced RRM requirements, since the working time for this WI is short, we would suggest focusing on the baseline requirement, and the enhanced requirement can be discussed later, e.g. WI R18 NTN IoT Enh. Besides, what would be the enhanced requirement is unclear at this stage.

Similar, the capability of parallel measurements on multiple LEO satellites can be discussed later, e.g. WI R18 NTN IoT Enh. Since IoT device is considered as low complexity, it can assume UE measure one LEO satellite at a time as the baseline.

Proposal 4: The UE capability of Enhanced RRM requirements and parallel measurements on multiple LEO satellites can be postponed.

## Measurement capability on number of NGSO satellites

For NGSO deployment, to address the complexity of large Doppler shift of NGSO satellite, the additional limitation on the number of satellites the UE needs to monitor per carrier is specified in NR NTN. Thus, the similar measurement capability is also proposed for IoT NTN. However, in IoT NTN, discontinuous coverage can be additional supported, and UE is only required to monitor the satellite provided in SIB32 in the proper time, thus the number of satellite to be monitor can be 1. Besides, IoT device maybe with lower capability than NR devices, thus lower number in inter-frequency is proposed.

Proposal 5: If the discontinuous coverage is supported, the number of target satellites UE needs to monitor can be [1].

Proposal 6: For NB in IDLE and M1 in both IDLE and CONNCTED,

- for intra-frequency carrier, the number of target satellites UE needs to monitor is [2] including serving LEO satellite.

- for inter-frequency carrier, the number of target satellites UE needs to monitor is [1]

## For NGSO, Doppler shift impact in Multiple NGSO satellites

For LEO, in NR NTN discussion, the scaling factor of K_satellite has been considered to allow UE to perform the measurement in TDM manner, and IoT NTN can follow the same approach, while the definition of K_satellite can be further simplified. Because it has not need to consider SMTC structure in LTE and it can assume UE to UE measures on only one satellite at a time as IoT device is considered as low complexity, thus the K_satellite can be the number of NGSO satellites to be measured.

Proposal 7: For NGSO, consider a scaling factor of K_satellite, which is the number NGSO satellites to be measure. It can apply to the following requirement

- RRC Re-establishment and RRC release with redirection

- M1, intra-frequency measurement and inter-frequency measurement in CONNETED mode

- For NB/M1, intra-frequency measurement and inter-frequency measurement in IDLE mode

# IDLE state mobility requirements

The following requirements for NB/M1 IDLE mode will be discussed in this section:

- Cell Re-selection

- Maximum interruption in paging reception

- Channel quality report for UE Category M1 in idle mode

- WUS receptions

- Transmission using preconfigured uplink resources (PUR)

## Cell Re-selection for NGSO.

In the last meeting, it has been agreed that the existing requirement can apply to GEO, but more discussion is needed for LEO.

NGSO –scaling factor of K_satellite

#### 4.6.2A.2	Measurements of intra-frequency NB-IoT cells for UE category NB1/NB2 in normal coverage

- The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS36.304 within Tdetect,NB_Intra_NC x KSatellite when Treselection= 0 .

- The UE shall measure NRSRP at least every Tmeasure,NB_Intra_NC x KSatellite for intra-frequency cells that are identified and measured according to the measurement rules.

- For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Tevaluate,NB_intra-NC x KSatellite when Treselection = 0, cprovided that the cell is at least XdB better ranked, where ‘X’ is specified in Table 4.6.2.4-3. When evaluating cells for reselection, the side conditions for NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot apply to both serving and non-serving NB-IoT intra-frequency cells.

where KSatellite is a satellite specific scaling factor. KSatellite = 1, if GEO satellites are measured on the carrier; KSatellite= the number of NGSO satellites to be measured, if NGSO satellites are measured on the carrier

As discussed in the previous, the scaling factor of K_satellite can be considered. One example for NGSO intra-frequency NB-IoT cells for UE category NB1/NB2 in normal coverage is provided as below, where the Tdetect, Tmeasure,NB, Tevaluate are scaled up by a satellite specific scaling factor KSatellite, where KSatellite = 1, if GEO satellites are measured on the carrier; KSatellite  = the number of NGSO satellites to be measured, if NGSO satellites are measured on the carrier.

NGSO – cell detection time in intra-/inter-frequency measurement

LEO cells are moving fast, and the UE may stay within a cell around 6 – 130 secs, depending on the deployment scenario (as in Table 7.3.2.1.4-1: Time to HO for min/max cell diameter and varying UE speed, [5]). However, the detection time with DRX/eDRX of intra-/inter-frequency measurement could be relatively long, especially for Enhanced Coverage.

For example, as shown in Table 4.6.2.4-1 in TS 36.133, for NB in Enhanced Coverage, the cell detection time when SCH Ês/Iot of neighbouring cell < -6dB (i.e, -15≤ Q2 < -6) is larger than 260 secs, and thus the LEO cell would disappear when it has been detected. Similar if eDRX is configured, it would at least require ~225sec (11 eDRX cycles of 20.40s) to provide 406 measurement samples (assume PTW = 12.8s, eDRX = 20.48s, DRX = 0.32s), according to Table 4.6.2.4-2 in TS 36.133.

Table 4.6.2.4-1 : Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate, NB_intra_EC

| SCH Ês/Iot of neighboring cell: Q2 | DRX cycle length [s] | Tdetect,NB_Intra_ EC [s] (number of DRX cycles) | Tmeasure,NB_Intra_ EC [s] (number of DRX cycles) | Tevaluate,NB_intra_ EC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| -15≤ Q2 < -6 | 0.32 | 256 (800) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 266 (415) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 532 (415) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 532 (208) | 2.56 (1) | 15.36 (6) |
|  | 5.12 | 1063 (208) | 5.12 (1) | 20.48 (4) |
|  | 10.24 | 1063 (104) | 10.24 (1) | 30.72 (3) |
| Q2-6 | 0.32 | 26 (80) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | 29 (45) | 1.28 (2) | 10.24 (16) |
|  | 1.28 | 58 (45) | 1.28 (1) | 12.8 (10) |
|  | 2.56 | 59 (23) | 2.56 (1) | 15.36 (6) |
|  | 5.12 | 113 (22) | 5.12 (1) | 20.48 (4) |
|  | 10.24 | 113 (11) | 10.24 (1) | 30.72 (3) |

Table 4.6.2.4-2: Tdetect,NB_Intra_EC, Tmeasure,NB_Intra_EC and Tevaluate,NB_intra_EC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 2.56s periods) | Tdetect,NB_Intra_EC [s] (number of DRX cycles) | Tmeasure,NB_Intra_EC [s] (number of DRX cycles) | Tevaluate,NB_intraEC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥12.8 (5) | (406) | 1.28 (4) | 10.24 (32) |
|  | 0.64 | ≥12.8 (5) |  | 1.28 (2) | 10.24 (16) |
|  | 1.28 | ≥15.36 (6) |  | 1.28 (1) | 12.8 (10) |
|  | 2.56 | ≥17.92 (7) |  | 2.56 (1) | 15.36 (6) |
|  | 5.12 | ≥23.04 (9) |  | 5.12 (1) | 20.48 (4) |
|  | 10.24 | ≥33.28 (13) |  | 10.24 (1) | 30.72 (3) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |  |  |

On the other hand, for Normal Coverage, the issue is not significant, as the detection time can within 1 PTW to provide 20 measurement samples in general, e.g. DRX=0.32s and PTW>=6.4 sec), according to Table 4.6.2.2-2 in TS 36.133. Thus, for NC, the exiting TN requirement can be a baseline.

Table 4.6.2.2-2: Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 2.56s periods) | Tdetect,NB_Intra -NC [s] (number of DRX cycles) | Tmeasure,NB_Intra_NC [s] (number of DRX cycles) | Tevaluate,NB_intra_NC [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 2.56 (1) | (20) | 1.28 (4) | 2.56 (8) |
|  | 0.64 | ≥ 2.56 (1) |  | 1.28 (2) | 2.56 (4) |
|  | 1.28 | ≥ 5.12 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥ 7.68 (3) |  | 2.56 (1) | 5.12 (2) |
|  | 5.12 | ≥ 12.8 (5) |  | 5.12 (1) | 10.24 (2) |
|  | 10.24 | ≥ 23.04 (9) |  | 10.24 (1) | 20.48 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |  |  |

Proposal 8: For NB-IoT/eMTC cell re-selection requirement in NGSO,

- the existing delay requirements (Tdetect, Tmeasure, Tevaluate) can be scaled up by KSatellite

  - where KSatellite is the number NGSO satellites.

- For Normal Cover, the exiting TN requirement can be the baseline.

- For Enhanced Coverage intra-/inter-frequency measurement, the cell detection time (Tdetect) is not applicable for the case of -15≤ Q2 < -6.

NGSO - Cell stop serving time based cell reselection for Quasi-Earth Fixed satellites

#### Measurement and evaluation of serving cell for NB

…

If t-Service is provided and applicable, UE shall perform intra-frequency or inter-frequency measurements before the time t-Service regardless whether the serving cell fulfils Srxlev > SIntraSearchP or Srxlev > SnonIntraSearchP, and the exact time to start measurements before t-Service is up to UE implementation.

…

If t-Service is provided and applicable of the serving cell then the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 36.304 when any of the following conditions is fulfilled:

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within 40 s since time instance T1 provided that ‘t-Service’ > T1 or

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within 40 s since the time instance ‘t-Service’.

-	Where, T1 is the time instance in seconds when the UE has determined that the serving cell does not fulfil the cell selection criterion S.

Besides, for NGSO Quasi-Earth Fixed satellites, cell stop serving time (t-Servive) based cell reselection is supported and thus can be further considered in cell re-selection, i.e. ‘t-Service’ can be indicated in SIB as in TS 36.331. The following are excerpts from NR NTN cell re-selection as in 38.133 [4] with minor modifications and can be considered to add in the corresponding cell re-selection clauses in 36.133 for IoT NTN. One example on NB cell reselection with t-Service is provided as below.

Proposal 9: Cell stop serving time based cell reselection can be further considered for Quasi-Earth Fixed satellites.

## Maximum interruption in paging reception

It can be observed from the NR NTN Maximum interruption in paging reception as in 38.133 [4], as copied below, if the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, more attempts are allowed (i.e. K = 5), and it becomes 2.5 times comparing to GEO (i.e. K=2). By following the same logic, we propose longer maximum interruption for the case of different NGSO satellite.

Observation 2: In current 38.133 NR NTN, the maximum interruption in paging reception is extended if the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO.

#### 4.2C.2.5	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception.

At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms.

Where,

If the target cell belongs to the same satellite as the current one, and if the target cell is known, then K = 2.

If the target cell belongs to a different satellite than the current one and the target cell’s satellite is GEO, and if the target cell is known, then K = 2.

If the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, then K = 5 if the target cell is known.

…

The target cell is considered as known if it has been detectable during Tdetect,NR_Intra or Tdetect,NR_Inter, and the time span between SIB broadcasting cell stop time and the cell stop time is not less than Ttrigger. Otherwise, the target cell is considered as unknown, where Tdetect,NR_Intra, Tdetect,NR_Inter and Ttrigger are defined in 4.2C.2.3 and 4.2C.2.4. A longer interruption can be expected if the target cell is unknown.

It can also be observed that known condition based on Trigger is also specified to ensure the UE is provided with sufficient time for measurement if cell stop time in SIB is supported.  However, the explicit paging interruption for unknown case may not be necessary and it can simply capture the longer interruption is expected.

Proposal 10: For NB, the maximum interruption in paging reception for NTN cell reselection shall not exceed

- TSI-NB1-NC/EC + 100 ms,

  - the target cell’s satellite is GEO, or

  - the target cell’s satellite is NGSO and the target cell belongs to the same satellite as the current one

  - Note: same as the existing TN requirement, as in 4.6.2.7/4.6.2.7A

- TSI-NB1-NC/EC + [250] ms,

  - the target cell’s satellite is NGSO and the target cell belongs to the different satellite as the current one

Proposal 11: For M1, the maximum interruption in paging reception for NTN cell reselection shall not exceed

- TSI-EUTRA-M1-NC/EC + 50 ms,

  - the target cell’s satellite is GEO, or

  - the target cell’s satellite is NGSO and the target cell belongs to the same satellite as the current one

  - Note: same as the existing TN requirement, as in 4.7.2.1.5/4.7.2.2.5

- TSI-EUTRA-M1-NC/EC + [125] ms, if

  - the target cell’s satellite is NGSO and the target cell belongs to the different satellite as the current one

Proposal 12: If the cell stop time (i.e., t-serve) is applicable, and the time span between SIB broadcasting cell stop time and the cell stop time is less than Ttrigger, longer interruption is expected.

## Channel quality report for UE Category M1 in idle mode

In the last meeting, it has agreed the exiting requiement can apply for NTN GEO but NTN NGSO is not agreed yet. In our understanding, the transmission parameters are not necessary impacted by the NTN and can be reused as the baseline.

Proposal 13: Consider the existing requirement for Connected mode channel quality report in LEO as baseline. It can be updated if NTN specific impact has been identified.

## WUS receptions

In the last meeting, it has ageed the principles of defining WUS reception requirements is reused but the required number of repetitions are kept TBD. In our view the same number can be reused as baseline and it is fine to clarify the the side conditions are also the same.

Proposal 14: For WUS receptions, the existing required number of repetitions are re-used as baseline.

## Transmission using preconfigured uplink resources (PUR)

As discussed in the last meeting, the current TA validation may not be necessary for NTN UE because UE is requred to perform UE TA precompansation and the TA can be calculated based on Satellite location and the UE location. Since the legacy NRSRP change based method is optional, thus no impact on RAN2 specicification is expected, and network can simply not configure NRSRP change to the NTN UEs.

Besides, the Te requirement already covers PUR, as below an excerpt from TS 38.133,

7.20.2    Requirements

The UE initial transmission timing error shall be less than or equal to ±Te where the timing error limit value Te is specified in Table 7.20.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. ….

In other words, the timing accuracy has been ensured.

Regarding the impact of T317 expiry (regarding the Satellite assistance information) was also discussed in the last meeting. However, in our understanding that T317 is used in CONNECTED mode and it may not appropriate to define TA validation based on T317.

Regarding the impact of t-Service (serving cell stop time), it is not necessary to define the TA validation based on t-service, because PUR will not be able to be established as the serving cell has been stopped, i.e. no SIB has be provided.

Proposal 15: For the transmission using PUR, requirements on TA validation based on NRSRP change are not applicable to NTN. UE is allowed to transmit using PUR using the timing derived using the latest available N_TA, N_TA,common, N_TA,UE-specific values.

# CONNECTED state mobility requirements

The following CONNECTED state mobility requirements for NB/M1 IDLE mode will be discussed in this section:

- RRC Re-establishment

- RRC Connection Release with Redirection

## RRC Re-establishment and RRC release with redirection

In the last meeting, requriement for RRC Re-establishment and RRC release with redirection has beend disucssed, and the open issues are mainly for LEO scenarios, thus the existing TN requirements can be reused for GEO in our understanding.

Proposal 16: For GEO, the existing TN RRC Re-establishment and RRC release with redirection requirement can be reused as baseline.

For LEO, in NR NTN discussion, the scaling factor of K_satellite has been considered, and IoT NTN can follow the same approach, while the definition of K_satellite can be further simplified. Because it has not need to consider SMTC structure in LTE and it can assume UE to UE measures on only one satellite at a time as IoT device is considered as low complexity, thus the K_satellite can be the number of NGSO satellites to be measured.

Regarding the impact of t-service, as in the previous discussion, its impact is mainly on IDLE to trigger measurement regardless S criteria, thus impact has not observed for CONNECTED mode mobility.

Proposal 17: For NGSO, consider a scaling factor of K_satellite, which is the number NGSO satellites to be measure.

# Timing requirements and RLM

The following timing and RLM requirements for NB/M1 IDLE mode will be discussed in this section:

- UE transmit timing

- UL segmented transmission

- RLM (Radio link monitoring)

## UE transmit timing

It can be observed from TS 38.133 section 7.1C [4] on UE transmit timing for Satellite Access, the UE initial transmission timing error for 15 kHz is relaxed by 17*Ts, i.e. from 12*Ts (12*64*Tc) to be 29*Ts, by consideration the inaccuracy introduced by UE GNSS and estimation error of Satellite position based on signalled Ephemeris. By following the same principle, the relaxation of 17 Ts for IoT Te_NTN can be considered. However, relaxation may be not necessary for NB, since it has been relaxed. Besides, more than 17 Ts relaxation is not desirable because the timing requirement already be relaxed several times comparing the normal LTE and too large timing error will impact on the UL performance. The corresponding Te_NTN for NB and M1 are proposed below.

Proposal 18: For NB, it is fine to reuse the exiting Te for Te_NTN, i.e. Te_NTN = 80 Ts.

Proposal 19: For M1, relaxation of [17] Ts can be considered for Te_NTN.

- For M1 CE Mode A, Te_NTN: 24+[17] = [41] Ts

- For M1 CE Mode B, Te_NTN: 48+[17] =[65] Ts

When no repetition period is configured (or the configured repetition period is R=1)

In the legacy IoT timing requirement, when it is not the first transmission in a DRX cycle or there is no DRX cycle and When no repetition period is configured, or the configured repetition period is R=1, the rules for UE to adjust the UL timing as specified as below (take NB as an example, as in TS 36.133.)

### 7.20.2	Requirements // NB IoT

….

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for NPUSCH the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell except when the timing advance in clause 7.22 is applied such that the UE transmission timing error shall be less than or equal to ±Te, where the timing error limit value Te is specified in Table 7.20.2-1.

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate shall be 7*TS per second.

3)	The maximum aggregate adjustment rate shall be 58.33*TS per 200ms.

When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.

#### 7.1C.2.1	Gradual timing adjustment // NR NTN

When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE is required to adjust its timing to within Te_NTN. The reference timing shall be (NTA + NTA-offset + NTA,common + NTA,UE-specific)×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN.

2)	The minimum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common during the last one second, shall be Tp_NTN per second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common during the last 200ms, shall be Tq_NTN per 200 ms.

It can be observed that it is similar to the gradual timing adjustment in NR NTN, as in TS 38.133 section 7.1C.2.1 and copied below, because of UE pre-compensation on UL timing, the reference timing should take NTA,common + NTA,UE-specific into account. The same value of the exiting values of Tq /Tp  are applicable. Also similar clarification on the adjustments would be needed, i.e. to clarify with "apart from a change of NTA,UE-specific and NTA,common ….".

Proposal 20: For NB in NTN, clarify the rules for the timing adjustments as follows,

- When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 7*TS per second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 58.33*TS per 200ms.

- The legacy values of maximum amount of the magnitude of the timing change, the minimum aggregate adjustment rate, and the maximum aggregate adjustment rate can be reused.

- The above changes also apply M1.

When a repetition period is configured on the uplink for which R>1

### 7.20.2	Requirements // NB IoT

….

When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.

Besides, in the legacy IoT requirement, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period if R>1, an excerpt of 36.133 is copied as below.

However, the NTN UE needs to perform TA pre-compensation, based on satellite location and UE location, to align the received timing at eNB and thus this restriction should be removed for IoT NTN.

Proposal 21: Remove the follow restriction for IoT NTN, “When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.”

## UL Segmented Transmission

One incoming LS from RAN1 [3] regarding the UL Segmented Transmission for UL synchronization for IoT NTN, and a transmission gap between segments are introduced, as quoted as below:

LS on UL Segmented Transmission for UL synchronization for IoT NTN [3]

- RAN1 has discussed the following aspects and leaves it up to RAN4 to specify UL Segmented Transmission for UL synchronization for IoT NTN as follows:

  - UE applies segmented UL transmission according to duration configuration by the network

  - Different values (e.g., TA) for pre-compensation may be used per segment, where UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment

TP example

<TP1, Section 5.3.4, TS 36.211>

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the single UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity of Ngapprecompensation is configured by the higher layers based on the UE capability if signalled.

<END TP1>

From [3], it can be observed the UE capability on “Segmented UL transmission” are applicable to NB NGSO (NPUSCH), M1 GEO/NGSO (PUSCH/PUCCH) but not for NB GEO.

Observation 3: The UE capability on “Segmented UL transmission” are applicable to NB NGSO (NPUSCH), M1 GEO/NGSO (PUSCH/PUCCH) but not for NB GEO.

Regarding the RRM requirement for the UL Segmented Transmission for UL synchronization, because the UE specific TA for pre-compensation may be different for each segment and the UE specific TA is mainly captured in Timing requirement as in NR NTN. Thus, UE UL transmit timing requirements, i.e., Te_NTN seems a good place to cover it.

Proposal 22: Segmented UL transmission can be covered by NTN UE transmit timing requirements, i.e. Te_NTN. FFS whether and how to capture in RAN4.

## RLM in NGSO

Regarding the large Doppler shift prospective, as discussed in NR NTN, and thus the UE may not be able to measure both of serving cell and the intra-frequency cells if those cells are from different NGSO satellite.

However, no need to consider this case for NB because the intra-frequency cells are only serving cells in NB.

For M1 UE in LEO, it is preferrable to avoid introducing new UE capability of L1/L3 processing in parallel. The requirement can be defined based on the UE measures on one NGSO satellite at a time.

Regarding whether to evaluate the RLM as in non-DRX depending on T317 expiry or t-Service (cell stop time). In our understanding this optimization is already allowed in the current specification and can be up to UE implementation. It is not necessary to specify it.

Proposal 23: For RLM in NGSO,

- For NB NGSO, the existing TN RLM requirements apply

- For M1 NGSO,

  - For NGSO, define the RLM requirements based on UE measures on one NGSO satellite at a time, without introducing the UE capability of L1/L3 processing in parallel.

# Measurement requirements

The following measurement requirements for NB/M1 CONNECTED state will be discussed in this section:

- Measurement requirement for M1

- Measurement gap for M1

- Connected mode channel quality report for NGSO

Note that R16 NB-IoT does not support inter-frequency measurements in CONNCETED mode and hence it is not considred in R18 NB-IoT NTN requirements.

## M1, measurement rquirement in GEO.

Since GEO has not been impacted by large Doppler shift, serving cell stop time (t-Service), discontinuous coverage, thus the existing TN requirement can be re-used in general.

Proposal 24: For M1 in GEO, the existing TN measurement requirement can be reused as baseline.

## M1, measurement rquirement in NGSO.

Scaling factor for measurement with multiple LEO satellites

In the last meeting, it has been agreed to apply the scaling factor for measurement with multiple LEO satellites. However, the scaling factor Kmulti_SMTC specified in NR NTN (as in 9.2C.5 in 38.133) can not be applied directly, because there is no SMTC structure in LTE and can be simplified as below

K_satellite: it is a satellite specific scaling factor.

- K_satellite = 1, if GSO satellites are measured on the carrier

- K_satellite = TBD, if LEO satellites are measured on the carrier.

In our view, the K_satellite can be defined by assuming the UE measurement on only one satellite at a time, since IoT device is considered as low complexity, thus K_satellite can be the number of NGSO satellites to be measured.

Proposal 25: For M1 in NGSO, the delay requirements are scaled up by K_satellite, which is the number NGSO satellites to be measure.

## M1, Measurement gap

In the last meeting, it has been discussed whether to support 2 MGs. Because there is no SMTC structure in LTE, and the exiting gap of 6 ms can already cover NPSS/NSSS from different satellites, thus single MG can be sufficient.

Proposal 26: Support of multiple measurement gap is not considered.

## Connected mode channel quality report for NGSO

In the last meeting, it has agreed the exiting requiement can apply for NTN GEO but NTN NGSO is not agreed yet. In our understanding, similar to IDLE state, the transmission parameters are not necessary impacted by the NTN and can be reused as the baseline.

Proposal 27: Consider the existing requirement for Connected mode channel quality report in LEO as baseline. It can be updated if NTN specific impact has been identified.

# Summary

In this contribution, the RRM requirements for NB-IoT/eMTC over NTN has been discussed and views on the open issues are provided. The observations and proposals are summarized below:

Observation 1: In CONNECTED state, UE is required to acquire the SIB31 upon T317 expiry. T317 is not available in IDLE state.

Proposal 1: In IDLE state, if t-Service of the serving cell is provided and applicable, UE is required to perform neighbouring cell measurement regardless S criteria. The exact time to start measurements before t-Service is up to UE implementation.

Proposal 2: In IDLE state, if SIB32 is provided and applicable, UE is not required to perform any cell measurement while out of coverage in discontinuous coverage. The detection of out of coverage using satellite assistance information is up to UE implementation. IDLE state requirements apply when UE is once in coverage.

Proposal 3: In LEO, whether and what DRX cycle length to configure is up to NW, but UE is not required to fulfil the requirements for DRX cycle length ≧ 2.56s.

Proposal 4: The UE capability of Enhanced RRM requirements and parallel measurements on multiple LEO satellites can be postponed.

Proposal 5: If the discontinuous coverage is supported, the number of target satellites UE needs to monitor can be [1].

Proposal 6: For NB in IDLE and M1 in both IDLE and CONNCTED,

- for intra-frequency carrier, the number of target satellites UE needs to monitor is [2] including serving LEO satellite.

- for inter-frequency carrier, the number of target satellites UE needs to monitor is [1]

Proposal 7: For NGSO, consider a scaling factor of K_satellite, which is the number NGSO satellites to be measure. It can apply to the following requirement

- RRC Re-establishment and RRC release with redirection

- M1, intra-frequency measurement and inter-frequency measurement in CONNETED mode

- For NB/M1, intra-frequency measurement and inter-frequency measurement in IDLE mode

Proposal 8: For NB-IoT/eMTC cell re-selection requirement in NGSO,

- the existing delay requirements (Tdetect, Tmeasure, Tevaluate) can be scaled up by KSatellite

  - where KSatellite is the number NGSO satellites.

- For Normal Cover, the exiting TN requirement can be the baseline.

- For Enhanced Coverage intra-/inter-frequency measurement, the cell detection time (Tdetect) is not applicable for the case of -15≤ Q2 < -6.

Proposal 9: Cell stop serving time based cell reselection can be further considered for Quasi-Earth Fixed satellites.

Observation 2: In current 38.133 NR NTN, the maximum interruption in paging reception is extended if the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO.

Proposal 10: For NB, the maximum interruption in paging reception for NTN cell reselection shall not exceed

- TSI-NB1-NC/EC + 100 ms,

  - the target cell’s satellite is GEO, or

  - the target cell’s satellite is NGSO and the target cell belongs to the same satellite as the current one

  - Note: same as the existing TN requirement, as in 4.6.2.7/4.6.2.7A

- TSI-NB1-NC/EC + [250] ms,

  - the target cell’s satellite is NGSO and the target cell belongs to the different satellite as the current one

Proposal 11: For M1, the maximum interruption in paging reception for NTN cell reselection shall not exceed

- TSI-EUTRA-M1-NC/EC + 50 ms,

  - the target cell’s satellite is GEO, or

  - the target cell’s satellite is NGSO and the target cell belongs to the same satellite as the current one

  - Note: same as the existing TN requirement, as in 4.7.2.1.5/4.7.2.2.5

- TSI-EUTRA-M1-NC/EC + [125] ms, if

  - the target cell’s satellite is NGSO and the target cell belongs to the different satellite as the current one

Proposal 12: If the cell stop time (i.e., t-serve) is applicable, and the time span between SIB broadcasting cell stop time and the cell stop time is less than Ttrigger, longer interruption is expected.

Proposal 13: Consider the existing requirement for Connected mode channel quality report in LEO as baseline. It can be updated if NTN specific impact has been identified.

Proposal 14: For WUS receptions, the existing required number of repetitions are re-used as baseline.

Proposal 15: For the transmission using PUR, requirements on TA validation based on NRSRP change are not applicable to NTN. UE is allowed to transmit using PUR using the timing derived using the latest available N_TA, N_TA,common, N_TA,UE-specific values.

Proposal 16: For GEO, the existing TN RRC Re-establishment and RRC release with redirection requirement can be reused as baseline.

Proposal 17: For NGSO, consider a scaling factor of K_satellite, which is the number NGSO satellites to be measure.

Proposal 18: For NB, it is fine to reuse the exiting Te for Te_NTN, i.e. Te_NTN = 80 Ts.

Proposal 19: For M1, relaxation of [17] Ts can be considered for Te_NTN.

- For M1 CE Mode A, Te_NTN: 24+[17] = [41] Ts

- For M1 CE Mode B, Te_NTN: 48+[17] =[65] Ts

Proposal 20: For NB in NTN, clarify the rules for the timing adjustments as follows,

- When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 7*TS per second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA,UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be 58.33*TS per 200ms.

- The legacy values of maximum amount of the magnitude of the timing change, the minimum aggregate adjustment rate, and the maximum aggregate adjustment rate can be reused.

- The above changes also apply M1.

Proposal 21:

Observation 3: The UE capability on “Segmented UL transmission” are applicable to NB NGSO (NPUSCH), M1 GEO/NGSO (PUSCH/PUCCH) but not for NB GEO.

Proposal 22: Segmented UL transmission can be covered by NTN UE transmit timing requirements, i.e. Te_NTN. FFS whether and how to capture in RAN4.

Proposal 23: For RLM in NGSO,

- For NB NGSO, the existing TN RLM requirements apply

- For M1 NGSO,

  - For NGSO, define the RLM requirements based on UE measures on one NGSO satellite at a time, without introducing the UE capability of L1/L3 processing in parallel.

Proposal 24: For M1 in GEO, the existing TN measurement requirement can be reused as baseline.

Proposal 25: For M1 in NGSO, the delay requirements are scaled up by K_satellite, which is the number NGSO satellites to be measure.

Proposal 26: Support of multiple measurement gap is not considered.

Proposal 27: Consider the existing requirement for Connected mode channel quality report in LEO as baseline. It can be updated if NTN specific impact has been identified.

# Reference

- R4-2214350, “WF on LTE IoT NTN RRM requirements”, MediaTek Inc., RAN4#104-e meeting, August 2022.

- R4-2202637, “WF on NR NTN RRM requirements “, Qualcomm Inc., RAN4#101-bis-e meeting, Jan. 2022.

- R4-2211518, “LS on UL Segmented Transmission for UL synchronization for IoT NTN”, MediaTek Inc., RAN4#104-e, Aug. 2022.

- Draft of 3GPP TS 38.133 V17.7.0 (2022-09)

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.1.0 (2001-05)