3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215760

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item: 6.8.3.2

Source: MediaTek Inc.

Title: Discussion on simultaneous DL reception from different directions for L3 measurement

Document for: Discussion

- Introduction

In the last meeting, RAN4 has a preliminary discussion for FR2 multi-Rx and some open issues are identified in WF [1]. Besides, in this meeting, this WI is divided into four agenda items to be discussed : (1) general issues, (2) L3 measurement, (3) L1 measurement and (4) TCI state switching. Our view on second agenda item “L3 measurement” is provided in this paper.

- Discussion

The discussion of whether to enhance L3 measurement is triggered in the last meeting. For L3 measurement, it is a cell-level measurement and it is used for mobility. UE may regularly measure SSB on the frequency layer provided by RRC and perform handover procedure if the L3-RSRP measurement results of neighboring cell is better than the serving cell. However, to our understanding, there is no much gain if UE measures the SSB from neighboring more frequently with multi-panels. Fast L3 measurements are typically needed only when UE is moving faster. However, we do not see any difference in terms of UE mobility between UE with single activated panel and UE with multiple activated panels.

Proposal 1: Not to enhance L3 measurement requirement in R18 multi-panel WI.

- Summary

In this paper, the discussion of FR2 multiple panels is provided. We have the following proposal:

Proposal 1: Not to enhance L3 measurement requirement in R18 multi-panel WI.

- Reference

- R4-2214344, WF on FR2 multi-Rx RRM, vivo, RAN4 #104-e