3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215762

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item: 6.8.3.4

Source: MediaTek Inc.

Title: Discussion on simultaneous DL reception from different directions for TCI state switching

Document for: Discussion

- Introduction

In the last meeting, RAN4 has a preliminary discussion for FR2 multi-Rx and some open issues are identified in WF [1]. Besides, in this meeting, this WI is divided into four agenda items to be discussed: (1) general issues, (2) L3 measurement, (3) L1 measurement and (4) TCI state switching. Our view on fourth agenda item “TCI state switching” is provided in this paper.

- Discussion

In the following sections, below issues will be discussed sequentially.

- Basic assumption

- Known and unknown condition

- Switch delay

2.1 Basic assumption

In this section, we would like to discuss whether TCI state switch in R18 multi-Rx should be defined based on R15/R16 legacy TCI framework or R17 unified TCI framework. The corresponding WF [1] is provided as below for reference.

Issue 1-3-1-2: TCI framework for dual TCIs - Proposals - Option 1: Dual TCI state switching delay requirements shall base on Rel-15/16 TCI framework. - Option 2: To define requirements for TCI switching delay with dual TCI, both R15/R16 TCI framework and R17 TCI framework, i.e., unified TCI, are considered for UE supporting multi-Rx chain simultaneous reception.

To our understanding, currently, in RAN1, R17 unified TCI framework is not applicable to mTRP scenario, because 3GPP still does not support single PDCCH with 2 different TCI states. So, to our understanding, R17 unified TCI framework cannot be used for R18 multi-Rx because the intention of multiple panels is for mTRP scenario.

Proposal 1: In R18 multi-Rx, to define the TCI state requirement based on R15/R16 TCI framework.

2.2 Known/unknown condition of TCI state

In the legacy requirement, only one active panel is assumed. So, there is no cross panels problem. However, in R18 multi-Rx, it is a question whether known/unknown information can be shared cross different panels. For example, whether UE can regard target TCI state for panel #2 as known if UE only measured and reported for the target TCI state by using panel #1 before.

Our thinking on this issue is “should we care whether target TCI state is known to panel #2 if the TCI state is known to panel #1?” To our understanding, how to receive the signals is up to UE implementation. If UE transmitted L1 report for target TCI state based on a certain panel, the same panel will be used for later TCI state switch. So, as long as network indicates UE to switch to the target TCI state, UE behavior and the required delay are clear from both UE and network perspectives.

Proposal 2: For known and unknown condition of TCI state in R18 multi-Rx, the legacy R15/R16 requirement can be reused.

2.3 Switch delay

In the last meeting, some companies think the switch delay should additionally consider the transient time of cross panel switch. To our understanding, the cross panel switch means the transient time of panel power ON/OFF. However, in current TCI state switch delay requirement, such transient time is not considered.

Observation 1: For legacy TCI state switch delay requirement in TS 38.133, the transient time for panel power ON/OFF is not considered.

Besides, as far as we know, it seems no requirement for the transient time of panel power ON/OFF in RF session. Therefore, based on the above analysis, the following proposal is suggested.

Proposal 3: Reuse Rel-15/16 TCI state switch delay unless RF session achieves a new conclusion on panels ON/OFF switch time.

On the other hand, since we tend to not introduce the additional delay for the transient time of panel power ON/OFF. And considering that two panels should be able to work independently in terms of TCI state switch. So, to our understanding, the legacy TCI state switch delay requirement can be reused.

Proposal 4: For dual TCI state switch, the legacy TCI state switch delay requirement can be reused.

- Summary

In this paper, the discussion of FR2 multiple panels is provided. We have the following proposal:

Proposal 1: In R18 multi-Rx, to define the TCI state requirement based on R15/R16 TCI framework.

Proposal 2: For known and unknown condition of TCI state in R18 multi-Rx, the legacy R15/R16 requirement can be reused.

Observation 1: For legacy TCI state switch delay requirement in TS 38.133, the transient time for panel power ON/OFF is not considered.

Proposal 3: Reuse Rel-15/16 TCI state switch delay unless RF session achieves a new conclusion on panels ON/OFF switch time.

Proposal 4: For dual TCI state switch, the legacy TCI state switch delay requirement can be reused.

- Reference

- R4-2214344, WF on FR2 multi-Rx RRM, vivo, RAN4 #104-e