3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215765

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item: 4.5.1.2

Source: MediaTek Inc.

Title: Discussion on inter cell beam management

Document for: Discussion

- Introduction

According to [1], some issues have been concluded in the last meeting. In this paper, we would like to further discuss the open issues, e.g. applicability of FR 2-2, scheduling restriction for dynamic TDD, and SSB and PDCCH/PDSCH are overlapped on the same RE.

- Discussion

2.1 Applicability of FR 2-2

According to the current spec as below, we would like to further discuss whether R17 inter cell beam management is applicable for FR2-2 or not.

3.6.13 Applicability of requirements for FR2 Unless stated otherwise, the requirements for FR2 are applicable to both FR2-1 and FR2-2, except for the following cases: SFTD measurement requirements in clause 9.2.5.4, 9.3.8, 10.1.21 for FR2 are only applicable for FR2-1, CGI identification requirements in clause 9.11 for FR2 are only applicable for FR2-1, Inter-band CA requirements in all corresponding clauses for FR2 are only applicable for FR2-1.

To our understanding, R17 inter cell beam management is not applicable for FR2-2. The reason is because FR2-2 is not considered in current scheduling restriction requirement for non-serving cell (clause 9.13.6). Because scheduling restriction is a prerequisite of the measurement behaviour, to our understanding, it means whole measurement is not applicable. Therefore, the following proposal is suggested.

Proposal 1: FR 2-2 is not applicable to R17 inter cell beam management.

2.2 scheduling restriction for dynamic TDD

One open issue regarding scheduling restriction for dynamic TDD is discussed in the last meeting. To our understanding, the additional scheduling restriction is needed. The reason is because, as Figure 1, sometimes the flexible symbol may be overwritten as DL symbol for SSB transmission. In that case, the symbol which overlaps with SSB cannot be used for data transmission. In addition, considering TA, one additional symbol should be considered.

Figure 1. scheduling restriction for dynamic TDD

Therefore, based on above analysis, following proposal is suggested.

Proposal 2: Introduce scheduling restriction for dynamic TDD on serving cell UL symbols which fully or partially (because of TA) overlaps with the SSB for L1-RSRP measurement on cell with different PCI.

2.3 SSB and PDCCH/PDSCH are overlapped on the same RE

In the last meeting, there is one open issue as below.

Issue 2-6-2: Whether any clarification or update is needed in RAN4 spec when SSB and PDCCH/PDSCH are overlapped on the same RE - Option 1: - No. - Option 2: - Clarify that performance degradation is expected when overlapping happen in RAN4 spec. - Option 3: - Clarify that there is no UE requirement when overlapping happen in RAN4 spec. - Option 4: - Define scheduling restriction to avoid overlap between SSB and data on the same RE in RAN4 spec.

Because RAN4 sent a reply LS to RAN1 in the last meeting. So, to our understanding, whether to update RAN4 spec for overlap between SSB and data on the same RE should wait for RAN1 conclusion. If RAN1 defines any rate matching requirement in RAN1 spec, then RAN4 spec update is not needed. If no, RAN4 can further study how/whether to update the RAN4 spec.

Proposal 3: Whether to define the requirement of overlap between SSB and PDCCH/PDSCH in the same RE should wait for RAN1 conclusion.

- Summary

In this paper, the discussion of inter cell beam management is provided. We have the following proposal:

Proposal 1: FR 2-2 is not applicable to R17 inter cell beam management.

Proposal 2: Introduce scheduling restriction for dynamic TDD on serving cell UL symbols which fully or partially (because of TA) overlaps with the SSB for L1-RSRP measurement on cell with different PCI.

Proposal 3: Whether to define the requirement of overlap between SSB and PDCCH/PDSCH in the same RE should wait for RAN1 conclusion.

- Reference

- R4-2214482, WF on FeMIMO RRM requirements for inter-cell beam management, Huawei, HiSilicon, RAN4 #104-e