3GPP RAN WG4 Meeting #104-bis-e		R4-2215768

Electronic Meeting, October 10-19, 2022

Agenda item:	6.4.2

Source:	Murata Manufacturing Co., Ltd.

Title:	FR2 Multi-band BS

Document for:	Discussion

1	Introduction

In RAN#95, a SI on multi-band operation for FR2 was approved with the following objectives [1]. In this contribution, we want to share some consideration on the multi-band operation in FR2.

- mmWave multi-band BS Study the following aspects for FR2 multi-band BS: - Example bands: - 26+28 GHz: n258 + n261 - 28+39 GHz: n257/n261 + n260 - 26+40 GHz: n258 + n259/n262 - 28+40 GHz: n257/n261 + n259/n262 - Investigate the feasibility and performance of wideband RF and antenna architectures covering multiple FR2 bands - Investigate if FR1 multi-band methods are re-usable for FR2, and (if so) agree on the appropriate inter-RF BW gaps - Investigate if FR1 exceptions are acceptable for FR2 - Investigate whether a generic solution for all combinations within FR2-1 is possible and/or a solution for all or a part of the frequency range should be targeted - Frequency range 24-29 GHz which includes n257/n258/n261 - Frequency range 37-48 GHz which includes n260/n259/n262 - Study the definition of FR2 multi-band BS

2	multi-band BS considerations

2.1	RF frontend architecture

As explained by many contributors in RAN-#104-e, the feasibility of multi-band BS is highly dependent on Fractional Bandwidth (FBW) [2],[3]. For example, FBW of n258 + n261 combination is 15.6%. but in case of n258+n259, FBW reaches 56.8%. It is quite difficult to configure the multi-band RIB supporting across different frequency group like as n258 + n259 band combination.

A large FBW poses many challenges. For example, PA linearity and antenna wideband matching and so on. However, even if FR1, these issues are existed for multi-band base station which supports broadband frequency range. On the other hand, we need to consider that mmWave require using array antennas to achieve required communication distance.

As Nokia explained, array antenna has mainly two issues. The first one is that directivity deviation increases when array antennas are shared by different bands. For example, in case of n258+n259 combination, directivity deviation reaches 5dB, it is almost same as antenna element gain. And another problem is the beam direction. When Phase Shifter in the Beam-former (BF) is shared with two or more bands, the beam direction of either one band cannot be properly controlled [4]. From the viewpoint of impact on communication distance, the inability to properly perform beamforming might be the most significant issue.

Observation 1:	When Phase Shifter in the Beam-former (BF) is shared with two or more bands, the beam direction of either one band cannot be properly controlled. And from the viewpoint of impact on communication distance, the inability to properly perform beamforming might be the most significant issue.

Then we investigated the front-end architecture of a FR2 base station based on these contributions. Huawei presented four architecture proposals in RAN4-#104-e [5]. We added beam-former on it and analysed these architectures. Figure 2.1 is the revised four architectures.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 2.1. Architecture of FR2 multi-band BS RF Frontend.

Table 2. Comparison of FR2 multi-band BS RF Frontend architecture

|  | Definition of FR1’s multi-band RIB | Common active RF components | Expected band combination | Description |
| --- | --- | --- | --- | --- |
| (a) | Comply | PA, BF, ANT | n258 + n261 | - Applicable to support same frequency group combination, but difficult to support different frequency group |
| (b) | Partially comply | ANT | n257/n261 + n260 n258 + n259/n262 n257/n261 + n259/n262 | - Since each bands have independent PA and BF, possible to support different frequency group band combination. Compatible with stack antennas, etc. - A diplexer is required to divide Band A and B signals |
| (c) | Partially comply | PA, BF | n258 + n261 | - Due to PA performance, it is difficult to support different frequency groups. - May be used in Same frequency groups. In this case, filter is required to divide Band A and B signals. |
| (d) | Not comply | N/A | n258 + n261 n257/n261 + n260 n258 + n259/n262 n257/n261 + n259/n262 | - There are no common active RF components. - If the definition of FR1 is applied, it becomes a single-band RIB. |

It seems that (b) and (d) are feasible with multiband base stations which support different frequency groups. Because (a) and (c) are difficult to solve the restrictions on beam control due to sharing phase shifter in BF, even if PA performance across different frequency group can be improved. On the other hand, (b) and (d) have independent phase shifters corresponding to the lower frequency band and higher frequency band, so the problem of beam control can be smaller in spite of remaining element gap issue (directivity deviation, grating lobe etc.).

Comparing (b) and (d), (d) should not be regarded as an architecture having common active RF components, we think it is single-band RIB. Also, considering the concept of ACLR and ACS specifications for non-contiguous spectrum, it doesn’t make sense to define the configuration like (d) as multi-band RIB.

To summarize the above, the possible architectures that support Multi-band RIB are (a) and (b) for same frequency group, (b) for different frequency group. Since (c) has no independent phase shifter, it is difficult to solve the restriction on beam control for different frequency groups band combination. In case of same frequency group band combination, both (a) and (c) have a feasibility, but (a) has advantages in terms of cost and implementation.

Observation 2:	The possible architectures that support Multi-band RIB are (a) and (b) for the same frequency group, and (b) for a different frequency group.
(a) PA, BF, ANT work as common active RF components
(b) ANT works as common active RF components, PA, BF are independent for each band

Observation 3:	Base station which has no common active RF components should not be defined as multi-band RIB.

2.2	Filtering technology

In architecture (b), which realizes multiband support for different frequency groups, a diplexer is required between the antenna and PA (and LNA, although not shown in the figure). We studied to design this diplexer, so we present the overview. Figure 2.2 shows the simulation results of a diplexer for n257/258 and n260 multi-band combination. The insertion loss is about 1dB, and the attenuation is over 50dB.

Figure 2.2. n257/n258-n260 Diplexer simulation.

The dimensions of this diplexer are 4.0 mm x 1.8 mm or less in LTCC substrate, and its dielectric constant is 6.4. Most of the current base station supports dual polarization, so the required area for diplexer implementation is doubled and it is 4.0mm x 3.6mm. For example, in case of n258+n260 combination, 0.5λ at the center frequency is 4.67mm. when antenna element gap is 0.5λ or larger, it is possible to implement diplexer below each antenna elements.

It shows that if an insertion loss of about 1dB could be accepted, the architecture (b) can realize a base station which supports different frequency groups band combination.

Observation 4:	Possible to implement diplexer below multi-band mmWave array antenna elements.

3	Conclusions

This contribution provides our view on the multi-band BS architecture.

Observation 1:	When Phase Shifter in the Beam-former (BF) is shared with two or more bands, the beam direction of either one band cannot be properly controlled. And from the viewpoint of impact on communication distance, the inability to properly perform beamforming might be the most significant issue.

Observation 2:	The possible architectures that support Multi-band RIB are (a) and (b) for the same frequency group, and (b) for a different frequency group.
(a) PA, BF, ANT work as common active RF components
(b) ANT works as common active RF components, PA, BF are independent for each band

Observation 3:	Base station which has no common active RF components should not be defined as multi-band RIB.

Observation 4:	Possible to implement diplexer below multi-band mmWave array antenna elements.

4	References

RP-221001, “New SID: NR BS RF requirement evolution”, Huawei, HiSilicon.

R4-2212622, ” Multi-band BS in mm wave”, Ericsson

R4-2211658, “General consideration on mmWave multi-band BS”, CATT

R4-2211812, “Discussion on possible issues on performance of wideband RF and antenna architectures”, Nokia, Nokia Shanghai Bell

R4-2211775, “General consideration on mmWave multi-band BS”, Huawei, HiSilicon