3GPP TSG-RAN WG4 Meeting # 104-bis-e									    		       R4-2215777

Electronic Meeting, 10 October– 19 October, 2022

Agenda item:			6.22.2

Source:	Samsung

Title:	Simulation assumptions for above 10GHz NTN co-existence study

Document for:	Discussion

- Introduction

According to the Rel-18 NR NTN enhancement WID[], RAN4 has been mandated to study and identify NTN example band in above 10 GHz.

The following assumptions are taken a baseline for this work:

- GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered

- ESIM scenarios for NGSO in Ka band are not considered in this WI.

- Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified

- Regarding mobile VSAT, three types of terminals and scenarios exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study.

- FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2

- The ITU-R harmonized Ka band will serve as reference

- Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification.

For co-existence studies, following considerations should be taken into account:

- Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow.

- For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands.

With above said elements,  this document proposes initial simulation assumptions for the NTN coexistence study in above 10GHz bands.

It should be noted that several open issues for further discussion are highlighted in yellow mark.

- Discussion

Co-existence simulation scenarios

Since there is no rural scenario for FR2 TN, no relevant combinations need to be considered. TN Micro base station should be considered in dense urban scenarios, as it is one of the important base station types in FR2. The indoor scenario of the TN system is not proposed in this paper because indoor TN is much less interfered by NTN satellites than outdoor TN Micro base stations in dense urban. Once the dense urban TN system can coexist with the NTN system, there is no need to evaluate the indoor scenario. Otherwise, indoor scenario should be considered in further research.

Whether RAN4 should do the co-existence studies between NTN systems need to be discussed.

Proposal 1: The proposed scenarios for coexistence study are in the Table 2.1-1.

Table 2.1-1 Scenarios for NTN-NTN/TN co-existence

| FR2: 30GHz |  |  | Set 1 |  |  | Set 22 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GEO | LEO 600km | LEO 1200km5 | GEO | LEO 600km | LEO 1200km |
| NR | Rural |  | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Urban macro |  | X | X | X | X | X | X |
|  | Dense Urban |  | X | X | X | X | X | X |
| NTN1,4 | GEO3 | Set 1 | X | X | X | FFS | FFS | FFS |
|  | LEO 1200km |  | X | X | X | FFS | FFS | FFS |
|  | LEO 600km |  | X | X | X | FFS | FFS | FFS |
|  | GEO | Set 22 | FFS | FFS | FFS | FFS | FFS | FFS |
|  | LEO 1200km |  | FFS | FFS | FFS | FFS | FFS | FFS |
|  | LEO 600km |  | FFS | FFS | FFS | FFS | FFS | FFS |
| Note 1: Start with Earth Fixed beam first, Earth Moving Beams could be further discussed Note 2: Use Set 1 satellite antenna as the starting point for co-existence study. FFS Set2. Note 3: GEO and LEO only operate at adjacent channel. Note 4: Use GEO and LEO@600km when TN is victim. Note 5: Further check the possibility to remove LEO 1200km cases in future RAN4 meetings. |  |  |  |  |  |  |  |  |

Proposal 2： The aggressor and victim combinations are listed in Table 2.1-2.

Table 2.1-2 Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | FFS, the frequency of TN DL node has no overlapping with the NTN DL node. |
| 2 | TN with NTN | TN UL | NTN UL |  |
| 3 | TN with NTN | NTN DL | TN DL | FFS, the frequency of NTN DL node has no overlapping with the TN DL node. |
| 4 | TN with NTN | NTN UL | TN UL |  |
| 5 | TN with NTN | NTN UL | TN DL |  |
| 6 | TN with NTN | TN DL | NTN UL |  |
| 7 | NTN with NTN | NTN DL | NTN DL | FFS, this scenario should be discussed to decide whether relevant coexistence study is needed in 3GPP |
|  |  | NTN UL | NTN UL | FFS, this scenario should be discussed to decide whether relevant coexistence study is needed in 3GPP |

It should be noted that the exemplary band(s) of uplink and downlink of the NTN system in above 10GHz need to be further discussed and determined under the corresponding agenda item. However, in order to proceed the co-existence study between TN and NTN system, the following frequency points are proposed as the frequencies in the co-existence study.

Proposal 3：  For uplink of NTN system, we propose to use 30GHz, for downlink we propose to use 20GHz. And other assumptions are listed as table 2.1-3 based on the TR 38.803 and TR 38.821.

Table 2.1-3.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Dense urban | FFS | 200 MHz | TDD | 1 |
| TN Urban macro | FFS | 200 MHz | TDD | 1 |
| GEO | 20GHz(DL)/30GHz(UL),FFS | 400MHz | FDD | 1,2,31 |
| LEO | 20GHz(DL)/30GHz(UL), FFS | 400MHz | FDD | 1,2, 31 |

Network layout model

Proposal 4: Refers to TR38.863 section 6.2.1 to determine the FR2 TN network layout model. In additional, the deployment of Dense Urban micro base station should follow TR38.803.

Simulation parameters

NTN parameters

Satellite parameters

Two sets of satellite parameters are shown in Table 2.3-1, 2.3-2 and 2.3-3 according to TR 38.821.

There is no reference for the number of active UE (UL) of Ka-band NTN system, therefore, this assumption should be discussed.

Proposal 5: The proposed set-1 and set-2 satellite parameters for co-existence study are in the Table 2.3-1, 2.3-2 and 2.3-3.

Table 2.3-1 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | Around 20GHz | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| G/T | Around 30 GHz | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite Rx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

Table 2.3-2 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | Around 20GHz | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz |
| Satellite Tx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| 3dB beamwidth |  | 0.4412 deg | 4.4127 deg | 4.4127 deg |
| abs |  |  |  |  |
| Satellite beam diameter |  | 280 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| G/T | Around 30 GHz | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |

Table 2.3-3 Other parameters for NTN

| Parameters | NTN | Remark |
| --- | --- | --- |
| The number of active UE (UL) | FFS |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See[6] Session 6.2.6 |  |
| NTN satellite Noise figure in dB | See Table 2.3-4 |  |
| Handover margin | 3dB[note 1] |  |
| Note 1: propose reuse handover margin in TR 38.863 |  |  |

The noise figure values were calculated by the following equation:

GRx=G/T -TdB -NF

Proposal 6: Propose Table 2.3-4 as the noise figure parameter of NTN system.

Table 2.3-4 Set-1 NTN satellite Noise figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 28 | 13 | 13 |
| G_Rx (dBi) | 58.5 | 38.5 | 38.5 |
| NF (dB) | 5.9 | 0.9 | 0.9 |

UE parameters

UE parameters are shown in Table 2.3-5, according to TR 38.821. But some parameters should be further discussed.

Practically, the minimum requirement of satellite communication for the GEO UE is 20 degrees above the horizon, and 30 degrees above the horizon for the LEO UE. And NTN UEs main lobe should point to the NTN satellite, and the horizontal and vertical degrees change with the position of the NTN satellite. However, for co-existence studies, appropriate horizontal boresight and vertical tilt degrees assumptions of NTN UEs are needed to model the angular relationship between NTN terminals and TN BSs and terminals.

Proposal 7: Propose Table 2.3-5 as the UE parameter of NTN system.

Proposal 8: Horizontal boresight and Vertical tilt assumptions of NTN UEs should be discussed and decided by the meeting.

Table 2.3-5 UE characteristics for system level simulations

| Characteristics | VSAT(NOTE2) | Other (Note 1) |
| --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | TBD dBi per element |
| Antenna temperature | 150 K | TBD K |
| Noise figure | 1.2 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | TBD dBi per element |
| Antenna Vertical tilt | FSS | FSS |
| Antenna horizontal boresight | FSS | FSS |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |

TN parameters

The TN parameters assumptions are proposed according to the TR38.803. Macro base station and Dense Urban Micro base station are considered.

Proposal 9: Propose Table 2.3-6, 2.3-7 and 2.3-8 as the BS parameter of TN system.

Table 2.3-6 Simulation assumptions of TN respectively based on NR Macro Urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 200m (baseline) 300m (optional) |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 2.3-7 Simulation assumptions of TN respectively based on NR Dense Urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 28.9 m |  |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi |  |
| Shadowing correlation |  | Between cite: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 200m is assumed. |  |  |  |

Table 2.3-8: Other simulation parameters

| Parameters | Urban macro | Dense urban |
| --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (DL) | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (UL) | 200MHz | 200MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer |
| DL power control | NO | NO |
| UL power control | YES | YES |
| BS max TX power in dBm | 43dBm | 33dBm |
| UE max TX power in dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -40dBm | -40dBm |
| BS Noise figure in dB | Note 1 | Note 1 |
| UE Noise figure in dB | Note 1 | Note 1 |
| Handover margin | 3dB | 3dB |
| Note 1: For deriving ACIR/ACS values which are included in respond to WP5D, following NF are used in co-existence simulation study for both UE and BS. 30GHz: 9 and 11 dB, 45GHz: 11 and 13 dB, 70GHz: 13 and 15 dB. |  |  |

TN ACS and ACLR refer to TS38.101-2 and TS38.104.

Proposal 10: Propose Table 2.3-9 as the ACS and ACLR assumptions of TN system.

Table 2.3-9 ACLR/ACS for TN

|  |  | NR |
| --- | --- | --- |
| BS [5] | ACLR | 28 dB |
|  | ACS | 23.5 dB |
| UE [4] | ACLR | 17dB |
|  | ACS | 22dB |

Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling

Proposal 11: Refers to TR 38.863 to determine the Satellite and UE Antenna and beam forming pattern modelling.

TN BS and UE antenna and beam forming pattern modelling

Base station and UE antenna pattern modelling are proposed according to the TR 38.803. The BS has no sub-array configuration for FR2 BS, therefore, 10 degrees antenna down-tilt is proposed.

Proposal 12: Propose Table 2.4-1 and 2.4-2 as the antenna pattern assumptions of TN BS and UE.

Table 2.4-1: BS antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Antenna Downtilt | 10 degree |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

Table 2.4-2: UE antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| Antenna Downtilt | 10 degree |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

ACLR and ACS modelling

Proposal 13: Refers to TR38.803 section 5.2.6 to determine ACLR and ACS modelling.

Propagation model

Proposal 14: Refers to TR38.863 section 6.2.5 and TR38.811 section 6.6 to determine the propagation model.

Transmission power control model

Proposal 15: Refers to TR38.863 section 6.2.6 to determine the transmission power control model.

Received power model

Proposal 16: Refers to TR38.863 section 6.2.7 to determine the reveived power model.

Performance metric

Proposal 17: Refers to TR38.863 section 6.2.8 to determine the Performance metric.

Throughput ~ SNR mapping

Proposal 18: Refers to TR38.863 section 6.2.9 to determine throughput to SNR mapping.

- Conclusion

It is proposed to use the simulation assumptions in this paper for Ka-band NTN co-existence study.

Proposal 1: The proposed scenarios for coexistence study are in the Table 2.1-1.

Proposal 2： The aggressor and victim combinations are listed in Table 2.1-2.

Proposal 3：  For uplink of NTN system, we propose to use 30GHz, for downlink we propose to use 20GHz. And other assumptions are listed as table 2.1-3 based on the TR 38.803 and TR 38.821.

Proposal 4: Refers to TR38.863 section 6.2.1 to determine the FR2 TN network layout model. In additional, the deployment of Dense Urban micro base station should follow TR38.803.

Proposal 5: The proposed set-1 and set-2 satellite parameters for co-existence study are in the Table 2.3-1, 2.3-2 and 2.3-3.

Proposal 6: Propose Table 2.3-4 as the noise figure parameter of NTN system.

Proposal 7: Propose Table 2.3-5 as the UE parameter of NTN system.

Proposal 8: Horizontal boresight and Vertical tilt assumptions of NTN UEs should be discussed and decided by the meeting.

Proposal 9: Propose Table 2.3-6, 2.3-7 and 2.3-8 as the BS parameter of TN system.

Proposal 10: Propose Table 2.3-9 as the ACS and ACLR assumptions of TN system.

Proposal 11: Refers to TR 38.863 to determine the Satellite and UE Antenna and beam forming pattern modelling.

Proposal 12: Propose Table 2.4-1 and 2.4-2 as the antenna pattern assumptions of TN BS and UE.

Proposal 13: Refers to TR38.803 section 5.2.6 to determine ACLR and ACS modelling.

Proposal 14: Refers to TR38.863 section 6.2.5 and TR38.811 section 6.6 to determine the propagation model.

Proposal 15: Refers to TR38.863 section 6.2.6 to determine the transmission power control model.

Proposal 16: Refers to TR38.863 section 6.2.7 to determine the reveived power model.

Proposal 17: Refers to TR38.863 section 6.2.8 to determine the Performance metric.

Proposal 18: Refers to TR38.863 section 6.2.9 to determine throughput to SNR mapping.

- Reference

[1] TR.38.821 Solutions for NR to support Non-Terrestrial Networks (NTN)

[2] TR 38.811 Study on New Radio (NR) to support non-terrestrial networks

[3] TR 38.803 Study on new radio access technology: Radio Frequency (RF) and co-existence aspects

[4] TS 38.101-2 NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

[5] TS 38.104 NR; Base Station (BS) radio transmission and reception

[6] TR 38.863 Non-terrestrial networks (NTN) related RF and co-existence aspects

[7] 3GPP, RP-222654, Thales, “NR NTN (Non-Terrestrial Networks) enhancements (revision of RP-221819)