3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215783

# Electronic Meeting, Oct. 10 – 19, 2022

Source:	LG Electronics

Title:	Discussion on NR-U PC3 UE RF requirements

Agenda item:	5.28.3

Document for:	Discussion

Introduction

In RAN4#104-e meeting, some WFs [1][2] on NR-U PC3 requirements were approved. Based on the WFs, we provide MPR and A-MPR (South Korea) for NR-U PC3.

Discussion

MPR for NR-U PC3

The following are considered for NR-U PC3 MPR based on the WF [1].

- PC3 MPR for 1Tx - PC3 calibration point: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB0 waveform at 30dB ACLR - MPR is evaluated for the same SEM, EVM and IBE requirements than for PC5 - Focus MPR evaluation on single CC - Post PA losses 4dB - PC3 MPR for 2Tx - Calibration point for each PC5 PA: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB3 waveform at 27dB ACLR and 20MHz NR-U SEM - Focus MPR evaluation on single CC - Post PA losses 4dB - MPR Evaluation - MPR is evaluated for the same SEM, EVM and IBE requirements than for PC5 - MPR is evaluated for 16dB and 10dB antenna isolation - Waveform - CP-OFDM , DFT-s-OFDM - Modulation order - QPSK/16QAM/64QAM/256QAM - Channel Bandwidth - 20/40/60/80/100 MHz - Uplink configuration - Full allocation , Interlaced Allocation

And, the simulation scenarios in Table 1 are considered based on TR 38.849. Table 2 shows the related RB set-up.

Table 1.  Simulation scenario for MPR

| Test condition | DFT/CP | Modulation | Allocation |
| --- | --- | --- | --- |
| 1 | DFT-S-OFDM | QPSK | Full |
| 2 | CP-OFDM | QPSK | Full |
| 3 | DFT-S-OFDM | QPSK | Interlace_0 |
| 4 | CP-OFDM | QPSK | Interlace_0 |
| 5 | DFT-S-OFDM | 16QAM | Full |
| 6 | CP-OFDM | 16QAM | Full |
| 7 | DFT-S-OFDM | 16QAM | Interlace_0 |
| 8 | CP-OFDM | 16QAM | Interlace_0 |
| 9 | DFT-S-OFDM | 64QAM | Full |
| 10 | CP-OFDM | 64QAM | Full |
| 11 | DFT-S-OFDM | 64QAM | Interlace_0 |
| 12 | CP-OFDM | 64QAM | Interlace_0 |
| 13 | DFT-S-OFDM | 256QAM | Full |
| 14 | CP-OFDM | 256QAM | Full |
| 15 | DFT-S-OFDM | 256QAM | Interlace_0 |
| 16 | CP-OFDM | 256QAM | Interlace_0 |

Table 2.  RB Set-up

| DFT/CP | BW (MHz) | Allocation | RB Set-up |
| --- | --- | --- | --- |
| DFT-S-OFDM | 20 | Full | 100RB3 |
| CP-OFDM | 20 | Full | 106RB3 |
| DFT-S-OFDM | 20 | Interlace_0 | 1RB3 every10RBs(10x) |
| CP-OFDM | 20 | Interlace_0 | 1RB0 every10RBs(11x) |
| DFT-S-OFDM | 40 | Full | 216RB0 |
| CP-OFDM | 40 | Full | 216RB0 |
| DFT-S-OFDM | 40 | Interlace_0 | 1RB0 every10RBs(20x) |
| CP-OFDM | 40 | Interlace_0 | 1RB0 every10RBs(22x) |
| DFT-S-OFDM | 60 | Full | 162RB0 |
| CP-OFDM | 60 | Full | 162RB0 |
| DFT-S-OFDM | 60 | Interlace_0 | 1RB0 every5RBs(30x) |
| CP-OFDM | 60 | Interlace_0 | 1RB0 every5RBs(33x) |
| DFT-S-OFDM | 80 | Full | 216RB0 |
| CP-OFDM | 80 | Full | 217RB0 |
| DFT-S-OFDM | 80 | Interlace_0 | 1RB0 every5RBs(40x) |
| CP-OFDM | 80 | Interlace_0 | 1RB0 every5RBs(44x) |
| DFT-S-OFDM | 100 | Full | 273RB0 |
| CP-OFDM | 100 | Full | 273RB0 |
| DFT-S-OFDM | 100 | Interlace_0 | 1RB0 every5RBs(50x) |
| CP-OFDM | 100 | Interlace_0 | 1RB0 every5RBs(55x) |

With the simulation scenarios, Figure 1 ~ Figure 4 show the simulation results of Tx Power backoff for the following cases.

- Figure 1 : 1Tx 23dBm with ACLR = 27dB

- Figure 2 : 1Tx 23dBm with ACLR = 30dB

- Figure 3 : 2Tx 2x20dBm with ACLR of 30dB & Antenna Isolation = 10dB

- Figure 4 : 2Tx 2x20dBm with ACLR of 30dB & Antenna Isolation = 16dB

Here, channel bandwidth of 20MHz, 40MHz, 60MHz, 80MHz and 100MHz are considered for each scenarios.

Figure 1. Tx Power Backoff for 1Tx(23dBm) w/ ACLR of 27dB

Figure 2. Tx Power Backoff for 1Tx(23dBm) w/ ACLR of 30dB

Figure 3. Tx Power Backoff for 2Tx(2x20dBm) w/ ACLR of 30dB & Antenna Isolation of 10dB

Figure 4. Tx Power Backoff for 2Tx(2x20dBm) w/ ACLR of 30dB & Antenna Isolation of 16dB

From the Figures, it is observed.

- For 1Tx case, Tx power back off for ACLR of 30dB is higher than ACLR of 27dB for QPSK, 16QAM and 64QAM

- For 2Tx case, Tx power back off for antenna isolation of 10dB is almost same as antenna isolation of 16dB

Based on ACLR of 30dB, MPRs for 1Tx and 2Tx are proposed as Table 3 and Table 4 respectively. Here, MPR for Pi/2 BPSK is proposed with same MPR of QPSK.

Table 3. Proposed MPR for NR-U PC3 with 1Tx

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.0 |
|  | QPSK | ≤ 1.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 1.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 2.0 | ≤ 3.5 |
|  | 256 QAM | ≤ 3.0 | ≤ 3.5 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 5.5 | ≤ 5.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Table 4. Proposed MPR for NR-U PC3 with 2Tx

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 2.0 | ≤ 3.5 |
|  | QPSK | ≤ 2.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 4.5 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Table 6.2F.2-2 MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration |  |
| --- | --- | --- |
|  | A | B |
| 40 | 11 | 10, 01 |
| 60 | 111, 011, 110, 001, 010, 100 | None |
| 80 | 1111, 0111, 1110, 0110, 0001, 1000 | 1100, 0011, 0100, 0010 |
| 100 | 11111, 01111, 11110, 00111, 01110, 11100, 00011, 00110, 01100, 11000 | None |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |

Proposal 1: Define MPR as provided in Table 3 for NR-U PC3 with 1Tx (23dBm).

Proposal 2: Define MPR as provided in Table 4 for NR-U PC3 with 2Tx (2x20dBm) for both antenna isolation of 10dB and 16dB.

A-MPR for NR-U PC3 (LPI in South Korea)

The followings are considered for A-MPR of NR-U PC3 based LPI considering Korea Regulation based on the WF [1][2].

From WF[1],

- PC3 MPR/A-MPR for 1Tx - PC3 calibration point: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB0 waveform at 30dB ACLR - MPR is evaluated for the same SEM, EVM and IBE requirements than for PC5 - Focus MPR evaluation on single CC - Post PA losses 4dB - PC3 MPR/A-MPR for 2Tx - Calibration point for each PC5 PA: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB3 waveform at 27dB ACLR and 20MHz NR-U SEM - Focus MPR evaluation on single CC - Post PA losses 4dB - MPR Evaluation - MPR is evaluated for the same SEM, EVM and IBE requirements than for PC5 - MPR is evaluated for 16dB and 10dB antenna isolation - A-MPR - n96 NS_60 - PC3 and existing NS values - Add PC3 A-MPR to existing NS values for SP and LPI modes in all regions - PC3 A-MPR is only applicable to NS values for SP and LPI modes - Channel allocations and placement - Recommended WF - Re-use channel allocations (full and partial allocations) and channel placement from PC5 for PC3 simulations - Evaluate in-band PSD/EIRP limited channels should have dedicated A-MPR for PC3 - Waveform - CP-OFDM , DFT-s-OFDM - Modulation order - QPSK/16QAM/64QAM/256QAM - Channel Bandwidth - 20/40/60/80/100 MHz - Uplink configuration - Full allocation , Interlaced Allocation

From WF[2],

- New channels at band edge - WF - Consider the following options: - Option 1: Channel at band edge which have larger A-MPR than current edge channels shall be downlink only - Option 2: Enable first 20MHz for DL only - Option 3: Do not define the additional channel raster points - Additional channels for NS_53, 58, 59, 60 - WF - Further discuss - whether no additional channel for NS_58 shall be introduced - whether 20MHz channel at 5935MHz can be added for NS_59 - whether 40MHz channel at 5945MHz can be added for NS_59 - whether 60MHz channel at 5955MHz can be added for NS_59 - whether 80MHz channel at 5965MHz can be added for NS_59 - whether 100MHz channel at 5975MHz can be added for NS_59 - n96 upper channel study - Agreement - Study additional channels at the top of n96 - Study 40MHz channel at 7105MHz for NS_59 and reuse the A-MPR if possible - Study 80MHz channel at 7085MHz for NS_59 and reuse the A-MPR if possible - UNII-7 channel study for NS_54 - WF - Further discuss - whether introduction of 40MHz channel at 6545MHz can be studied - whether introduction of 60MHz channel at 6835MHz can be studied - whether introduction of 80MHz channels at 6565 and 6825MHz can be studied - whether introduction of 100MHz channels at 6575 and 6815MHz can be studied

For A-MPR, same simulation scenarios in Table 1 and same RB set-up in Table 2 are considered.

Band n96 NS_60 is related to LPI in South Korea Regulation [5] as seen in Table 5.

Table 5. Summary of NS values.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | TBD |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | N/A |
| Canada | NS_54 | NS_59 | TBD |
| Brazil | N/A | NS_53 | TBD |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | TBD |
| Colombia | N/A | NS_53 | N/A |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |

Table 6 shows Regulatory parameters in South Korea for license-exempt. So far, LPI and VLP are permissible in South Korea. The related UE requirements were specified for PC5 based LPI and PC5 based VLP in South Korea. Table 6.2F.3.10-1 in TS38.101-1 is A-MPR requirements for PC5 based LPI in South Korea. However, PC3 based LPI is not specified.

Table 6. Regulatory parameters in South Korea.

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |

Therefore, based on the WF [1], A-MPR for PC3 based LPI needs to be defined in South Korea.

Table 6.2F.3.10-1: A-MPR for NS_60 power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 | ≤ 3.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 5.0 | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |

In n96, available NR-ARFCNs for Channel Bandwidth of 20MHz, 40MHz, 60MHz, 80MHz and 100MHz and corresponding centre frequencies are summarized in Table 7 and Table 8 respectively.

Table 7. Allowed NREF (NR-ARFCN) for operation in Band n96

| Channel Bandwidth (total #) | Allowed NREF |
| --- | --- |
| 20 MHz (59) | 797000, 798332, 799668, 801000, 802332, 803668, 805000, 806332, 807668, 809000, 810332, 811668, 813000, 814332, 815668, 817000, 818332, 819668, 821000, 822332, 823668, 825000, 826332, 827668, 8290001, 8303321, 8316681, 8330001, 8343321, 8356681, 8370001, 8383321, 8396681, 8410001, 8423321, 8436681, 8450001, 8463321, 8476681, 8490001, 8503321, 8516681, 8530001, 8543321, 8556681, 8570001, 8583321, 8596681, 8610001, 8623321, 8636681, 8650001, 8663321, 8676681, 8690001, 8703321, 8716681, 8730001, 8743321 |
| 40 MHz (29) | 797668, 800332, 803000, 805668, 808332, 811000, 813668, 816332, 819000, 821668, 824332, 827000, 8296681, 8323321, 8350001, 8376681, 8403321, 8430001, 8456681, 8483321, 8510001, 8536681, 8563321, 8590001, 8616681, 8643321, 8670001, 8696681, 8723321 |
| 60 MHz (29) | 798332, 799668, 803668, 805000, 809000, 810332, 814332, 815668, 819668, 821000, 825000, 826332, 8303321, 8316681, 8356681, 8370001, 8410001, 8423321, 8463321, 8476681, 8516681, 8530001, 8570001, 8583321, 8623321, 8636681, 8676681, 8690001, 8730001 |
| 80 MHz (14) | 799000, 804332, 809668, 815000, 820332, 825668, 8310001, 8363321, 8416681, 8470001, 8523321, 8576681, 8630001, 8683321 |
| 100 MHz (17) | 799668, 803668, 810332, 814332, 821000, 825000, 8316681, 8356681, 8423321, 8463321, 8530001, 8570001, 8636681, 8676681, 8690001, 8703321, 8716681 |
| NOTE 1: This NR-ARFCN is not available for operation in band n96 in Europe |  |

Table 8. Rounded Centre frequency corresponding to the allowed NREF (NR-ARFCN) for operation in Band n96

| Channel Bandwidth (Total #) | Round(Fc) from Allowed NREF |
| --- | --- |
| 20 MHz (59) | 5955, 5975, 5995, 6015, 6035, 6055, 6075, 6095, 6115, 6135, 6155, 6175, 6195, 6215, 6235, 6255, 6275, 6295, 6315, 6335, 6355, 6375, 6395, 6415, 64351, 64541, 64751, 64951, 65151, 65351, 65551, 65751, 65951, 66151, 66351, 66551, 66751, 66951, 67151, 67351, 67551, 67751, 67951, 68151, 68351, 68551, 68751, 68951, 69151, 69351, 69551, 69751, 69951, 70151, 70351, 70551, 70751, 70951, 71151 |
| 40 MHz (29) | 5965, 6005, 6045, 6085, 6125, 6165, 6205, 6245, 6285, 6325, 6365, 6405, 64451, 64851, 65251, 65651, 66051, 66451, 66851, 67251, 67651, 68051, 68451, 68851, 69251, 69651, 70051, 70451,70851 |
| 60 MHz (29) | 5975, 5995, 6055, 6075, 6135, 6155, 6215, 6235, 6295, 6315, 6375, 6395, 64551, 64751, 65351, 65551, 66151, 66351, 66951, 67151, 67751, 67951, 68551, 68751, 69351, 69551, 70151, 70351, 70951 |
| 80 MHz (14) | 5985, 6065, 6145, 6225, 6305, 6385, 64651, 65451, 66251, 67051, 67851, 68651, 69451, 70251 |
| 100 MHz (17) | 5995, 6055, 6155, 6215, 6315, 6375, 64751, 65351, 66351, 66951, 67951, 68551,69551, 70151, 70351, 70551, 70751 |
| NOTE 1: This NR-ARFCN is not available for operation in band n96 in Europe |  |

Figure 5 shows the available centre frequencies and the number of channel bandwidth in n96. The highlighted with yellow colour is the edge channel for each channel bandwidth. For CBW of 20MHz, 60MHz and 100MHz, there is no GBW(Guard Bandwidth) for each edge channel. For CBW of 40MHz and 80MHz, there is GBW of 20MHz for each edge channel.

- 20MHz  : Edge channel = #59          (No GBW)

- 40MHz  : Edge channel = #1 or #29 (GBW = 20MHz)

- 60MHz  : Edge channel = #29          (No GBW)

- 80MHz  : Edge channel = #1            (GBW = 20MHz)

- 100MHz: Edge channel = #17          (No GBW)

2nd edge channel which is highlighted with orange colour is also considered for simulation.

Simulated edge channels are as follows.

- 20MHz  : Edge channel = #59, #1

- 40MHz  : Edge channel = #1,  #2

- 60MHz  : Edge channel = #29, #1

- 80MHz  : Edge channel = #1, #14

- 100MHz: Edge channel = #17, #1

Figure 5. Available centre frequencies in n96

With the simulation scenarios, Figure 6 ~ Figure 7 show the simulation results of Tx Power backoff and the proposed A-MPR for the following cases.

- Figure 6-1 : 1Tx (23dBm) at CBW = 20MHz

- Figure 6-2 : 1Tx (23dBm) at CBW = 40MHz

- Figure 6-3 : 1Tx (23dBm) at CBW = 60MHz

- Figure 6-4 : 1Tx (23dBm) at CBW = 80MHz

- Figure 6-5 : 1Tx (23dBm) at CBW = 100MHz

- Figure 7-1 : 2Tx (2x20dBm) at CBW = 20MHz

- Figure 7-2 : 2Tx (2x20dBm) at CBW = 40MHz

- Figure 7-3 : 2Tx (2x20dBm) at CBW = 60MHz

- Figure 7-4 : 2Tx (2x20dBm) at CBW = 80MHz

- Figure 7-5 : 2Tx (2x20dBm) at CBW = 100MHz

Figure 6-1. Tx Power Backoff for 1Tx(23dBm) @ CBW=20MHz

Figure 6-2. Tx Power Backoff for 1Tx(23dBm) @ CBW=40MHz

Figure 6-3. Tx Power Backoff for 1Tx(23dBm) @ CBW=60MHz

Figure 6-4. Tx Power Backoff for 1Tx(23dBm) @ CBW=80MHz

Figure 6-5. Tx Power Backoff for 1Tx(23dBm) @ CBW=100MHz

Figure 7-1. Tx Power Backoff for 2Tx(2x20dBm) @ CBW=20MHz

Figure 7-2. Tx Power Backoff for 2Tx(2x20dBm) @ CBW=40MHz

Figure 7-3. Tx Power Backoff for 2Tx(2x20dBm) @ CBW=60MHz

Figure 7-4. Tx Power Backoff for 2Tx(2x20dBm) @ CBW=80MHz

Figure 7-5. Tx Power Backoff for 2Tx(2x20dBm) @ CBW=100MHz

From the Figures, it is observed that.

- For 1Tx case,

  - The higher CBW is, the lower the Tx power back off is.

  - Tx power back off for the 2nd edge channel is almost same as that for the 1st edge channel except for CP-OFDM full allocation scenario with CBW of 60MHz.

    - In case of CP-OFDM full allocation scenario with CBW of 60MHz, Tx power back off for 1st edge channel is about 0.5dB~0.8dB higher than 2nd edge channel.

- For 2Tx case,

  - The higher CBW is, the lower the Tx power back off is.

  - Tx power back off for the 2nd edge channel is almost same as that for the 1st edge channel except for CP-OFDM full allocation scenario with CBW of 60MHz.

    - In case of CP-OFDM full allocation scenario with CBW of 60MHz, Tx power back off for 1st edge channel is about 1.2 higher than 2nd edge channel for QPSK, 16QAM and 64QAM.

  - Tx power back off for antenna isolation of 10dB is almost same as antenna isolation of 16dB.

Based on the simulation results, A-MPR for NR-U PC3 based LPI in South Korea are proposed as Table 9 and Table 10 for 1Tx and 2Tx respectively. Here, MPR for Pi/2 BPSK is proposed with same MPR of QPSK.

Table 9. Proposed A-MPR for NR-U PC3 with 1Tx (South Korea)

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | QPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 3.5 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 3.5 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 3.5 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 4.0 | ≤ 5.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

Table 10. Proposed A-MPR for NR-U PC3 with 2Tx (Korea)

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 4.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 5.5 | ≤ 6.0 | ≤ 5.5 | ≤ 5.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

And, NS_60 is proposed to be reused for A-MPR for NR-U PC3 based LPI in South Korea.

Proposal 3: Define A-MPR as provided in Table 9 for NR-U PC3 with 1Tx (23dBm) in South Korea.

Proposal 4: Define A-MPR as provided in Table 10 for NR-U PC3 with 2Tx (2x20dBm) for both antenna isolation of 10dB and 16dB in South Korea.

Proposal 5: Reuse NS_60 for A-MPR of NR-U PC3 based LPI in South Korea.

Conclusion

In this contribution, we provide the simulation results for NR-U PC3 based MPR and A-MPR in South Korea. Based on the result, we propose MPR and A-MPR as follow.

Proposal 1: Define MPR as provided in Table 3 for NR-U PC3 with 1Tx (23dBm).

Proposal 2: Define MPR as provided in Table 4 for NR-U PC3 with 2Tx (2x20dBm).

Proposal 3: Define A-MPR as provided in Table 9 for NR-U PC3 with 1Tx (23dBm) in South Korea.

Proposal 4: Define A-MPR as provided in Table 10 for NR-U PC3 with 2Tx (2x20dBm) in South Korea.

Proposal 5: Reuse NS_60 for A-MPR of NR-U PC3 based LPI in South Korea.

Table 3. Proposed MPR for NR-U PC3 with 1Tx

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.0 |
|  | QPSK | ≤ 1.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 1.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 2.0 | ≤ 3.5 |
|  | 256 QAM | ≤ 3.0 | ≤ 3.5 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 4.0 |
|  | 64 QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 5.5 | ≤ 5.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Table 4. Proposed MPR for NR-U PC3 with 2Tx

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 2.0 | ≤ 3.5 |
|  | QPSK | ≤ 2.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 4.5 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 5.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Table 9. Proposed A-MPR for NR-U PC3 with 1Tx (South Korea)

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | QPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.0 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 3.5 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 3.5 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 3.5 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 4.0 | ≤ 5.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

Table 10. Proposed A-MPR for NR-U PC3 with 2Tx (Korea)

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 4.0 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 5.5 | ≤ 6.0 | ≤ 5.5 | ≤ 5.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

- Reference

[1] R4-2214437, “WF on NR-U PC3 requirements”, Charter Communications Inc.

[2] R4-2214439, “WF on channel raster extension for NR-U 6GHz bands”, Apple Inc.

[3] RP-221813, “New WID: Introduction of evolved shared spectrum bands”, Apple Inc.

[4] R4-2103229, “WF on introduction of lower 6GHz NR unlicensed operation for Europe”, Nokia, Qualcomm, Charter Communications Inc.

[5] TR 38.849, Introduction of 6GHz NR unlicensed operation

[6] RP-222555, “Regulatory update for the 6GHz frequency range”, Apple Inc.

[7] TR 37.890, Feasibility Study on 6 GHz

[8] Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations"

- URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

- URL: https://www.law.go.kr/admRulLsInfoP.do?chrClsCd=010202&admRulSeq=2100000196973

- Annex : Regulatory parameters comparative for license-exempt (TR37.890)

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
|  | UAE | LPI (see 4.1.5) | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI (see 4.1.7) | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP (see 4.1.7) |  | 14 dBm |  |  |
|  | Saudi Arabia | LPI (see 4.1.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 10dBm/MHz |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  |  | VLP (4.2.5) |  | 17 dBm |  |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |
|  | Hong Kong | LPI (see 4.3.3) | 5925 – 6425MHz | 23dBm | 10dBm/MHz | In accordance with ETSI EN 303 687 |
|  |  | VLP (see 4.3.3) |  | 14dBm | 1dBm/MHz |  |
|  | Australia | LPI (see 4.3.4) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.4) |  | 14dBm | 1dBm/MHz |  |
|  | New Zealand | LPI (see 4.3.5) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.5) |  | 14dBm | 1dBm/MHz |  |
|  | Japan | LPI (see 4.3.6) | 5925 – 6425MHz | 23dBm |  |  |
|  |  | VLP (see 4.3.6) |  | 14dBm |  |  |
|  | Malaysia | LPI (see 4.3.7) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.3.7) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) |  |
| Note 1: For the exact conditions of operation, refer to the sub-clause which is mentioned in parenthesis (e.g. for the exact conditions for operating SP in US, refer to sub-clause 4.2.1.) |  |  |  |  |  |  |