3GPP TSG-RAN WG4 Meeting # 104bis-e						  R4-2215810

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	6.9.2.2

Source: 	OPPO

Title: 	Discussion on L1 enhancement for FR2 Scell activation delay reduction

Document for:	Approval

1	Introduction

A WF on Rel-18 eFeRRM[1] was approved in RAN4#104 meeting [2]. In this contribution, we discuss the requirements for FR2 SCell activation delay reduction of the WI NR_RRM_enh3.

- FR2 SCell activation delay reduction - Identify cases where FR2 SCell activation delay can be reduced (e.g., unknown target cell cases), and specify reduced delay requirements for such cases, including but not limited to [RAN4] - Study and, if feasible, enhance cell detection for unknown SCell and time/frequency tracking - Study and, if feasible, enhance L1-RSRP measurement delay reduction on target Scell - Note: Subject to RAN4’s agreement, the technical solutions can be extended to other general RRM requirements if applicable. - Specify if needed, reference signal enhancement and/or signaling enhancement for the UE to meet the enhanced delay requirements [RAN4, RAN2] - Note: No RAN1 work, i.e. introducing new RS, is expected. - Note: the technical solutions can be extended to FR1, when applicable

2	Discussion

In this WI, we aim at reducing the FR2 Scell activation delay for both known and unknown cases. RAN4 agreed to prioritize at least FR2 unknown Scell delay reduction in the 1st phase of the WI.

Figure 1. An example of FR2 unknown SCell activation

Enhancement for L1-RSRP

Issue 2-3-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception) - FFS on beam sweeping factor reduction for L1 measurement during unknown FR2 SCell activation. - Option 1a (vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 for FR2 - Option 1b (Ericsson): - RAN4 to discuss the solutions or pre-conditions to reduce the scaling factor N1 in FR2 SCell activation. - RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non-changing. Rx beam constant time to be agreed as [X=1280ms]. - If UE performed a full RX beam sweeping for a procedure, next procedures or steps fall within RX beam constant time do not need to perform RX beam sweeping. - UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before - Option 2 (LG, MTK, HW, ZTE): Consider reporting (rough) best Tx beam during cell detection to reduce L1-RSRP measurement delay Issue 2-3-2: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation related with WI of FR2 multi-Rx chain DL reception Agreement: - Conclusion of issue 2-2-2 is applied for this issue 2-3-2 Issue 2-3-5: Other enhancement for L1-RSRP measurement of FR2 unknown SCell activation FFS: - Proposal 1 (NTT DoCoMo): FR2 SCell activation delay requirement should be studied under the assumption that UE performs L1-RSRP measurements for a cell with different PCI from serving cell - Proposal 2 (Apple, OPPO, ZTE, Xiaomi, OPPO, Ericsson, Nokia, NTT DCM): RAN4 to study skipping L1-RSRP measurement - Proposal 2a (Apple, OPPO, LGE, NTT DCM, ZTE): use measurement result from L3 stage for L1-RSRP reporting, if L3 measurement and L1 measurement are using same RS or QCLed type D RSs - Proposal 2b (ZTE, NTT DCM, ZTE): For the case of IBM disabled, maybe the UE would receive DL transmissions from different serving cells with the same Rx beam assumption. In such case, not any L1-RSRP measurement and report is necessary for this to-be-activated SCells if one active serving cell existing. - Proposal 2c (Huawei, Apple, LGE, OPPO, NTT DCM, ZTE): UE can report the beam information based on cell search and skip the L1-RSRP procedure.

As discussed in last meeting, UE is assumed to be able to derive some rough beam information during L3 measurement, which can be reused for L1-RSRP measurement.

Once cell detection has been completed, UE may have some knowledges about Rx beams of to-be-activated SCell. If SSB is used to perform L1-RSRP measurement for UE, only candidate target beams other than all beams need to be measured, e.g., utilizing the best beam selected during cell detection. If CSI-RS is used to perform L1-RSRP measurement and it is QCL type D with SSB for cell detection, UE can also have opportunity to achieve L1 beam information from L3 measurement procedure.

In these cases, measurement results from L3 measurement can be used for L1-RSRP reporting. The Rx beam sweeping factor of L1-RSRP measurement may be reduced, or even L1-RSRP measurement can be skipped.

Proposal 1: Consider beam sweeping factor reduction for L1 measurement during unknown FR2 SCell activation.

The similar UE capability of beam sweeping factor can apply for L1-RSRP measurement, and a set of candidate values can be predefined, e.g., introduce a set of candidate values less than 8. We think the configuration can be shared and the signalling could apply for both L3 and L1 measurement.

Proposal 2: Define the same UE capability of Rx beam sweeping factor in FR2 for L1 and L3 measurement.

We are open to further discuss the feasibility and conditions of skipping L1-RSRP measurement, besides L3 measurement and L1 measurement are using same RS or QCL typeD RSs.

Proposal 3: Consider the feasibility of skipping L1-RSRP measurement, and study its conditions, e.g., L1/L3 measurement configuration at least including RSs type/RS BW/QCLed RS for L1 and L3 measurement.

TCI related enhancement for L1 part

Issue 2-3-6: Fine timing tracking for SSB corresponding to the TCI state during FR2 unknown SCell activation - FFS: whether and how to skip fine timing tracking for SSB corresponding to the TCI state, if SSB timing from L3 measurement stage or SSB timing from L1-RSRP measurement stage can be reused. Issue 2-3-7: TCI activation enhancement during FR2 unknown SCell activation FFS: - Proposal 1 (Intel, Apple, Xiaomi,): further discuss the UE beam assumption after L1-RSRP report to reduce MAC CE based TCI activation delay. - Proposal 1a(Huawei, Apple, Intel, Xiaomi, OPPO, MTK, Ericsson, NTT DCM): the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report. - Proposal 2 (ZTE): to consider whether the TCI state indication can be acquired by UE before SCell activation or not, which would impact the total latency. - Proposal 3 (ZTE): to consider whether and how to identify TCI state without L1-RSRP measurement and report, which is the core issue for latency reduction for unknown case.

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state. After TCI activation, in legacy activation procedure UE will perform the fine timing tracking and then UE is able to perform the CQI measurement.

We are open to further study whether and how SSB timing from L3 or L1 can be reused for fine timing tracking. The conditions may rely on RS configuration and other open issues related to the feasibility of L1 and L3 enhancement. Thus, this issue can be discussed after other open issues are concluded.

We also support to further discuss the UE beam assumption after L1-RSRP report to reduce MAC CE based TCI activation delay, e.g., TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report. In this case, the uncertainty of TCI configuration/activation can be removed.

Proposal 4: When TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report, the uncertainty of TCI configuration/activation can be saved.

RS related enhancement for L1 part

Aperiodic RS related enhancement for L1 part Issue 2-3-8: Aperiodic RS related enhancement during FR2 unknown SCell activation FFS: - Proposal 1 (Apple, Ericsson): RAN4 to discuss: use AP RS for L1-RSRP measurement if UE can indicate the completion of L3 stage or can indicate the readiness of L1 measurement - Proposal 2a (Apple, Xiaomi, MTK, Ericsson, ZTE): RAN4 to discuss: use A-TRS corresponding to the TCI state for fine timing tracking after TCI activation command - Proposal 2b(Xiaomi): If there is no any QCL information of reference signal for target SCell, A-TRS is configured for fine timing tracking, and the A-TRS is QCL-ed with the selected SSB index. - Proposal 3: Any other solutions to enable A-TRS usage to unknown SCell activation is not precluded.

According to the agreements in R17, A-TRS has been well verified and used for fast SCell activation, including both FR1 known and unknown case, and FR2 known case. It is feasible to reduce the time period of cell search, acquiring frequency/timing tracking, and even fine timing tracking.

Since aperiodic CSI-RS have been used for L1-RSRP measurement, we are not sure whether it is necessary to introduce the new AP RS.

Periodic signals such as SSB or CSI-RS are used for cell search and/or timing tracking, resulting in a longer latency to wait for the RS occasions. For FR2 unknown case, at least A-TRS corresponding to the TCI state can be used for fine timing tracking after TCI activation command. After UE report the L1-RSRP to network for TCI determination, network can also use A-TRS for UE to perform fine timing tracking for TCI after the TCI activation command to avoid waiting at most 1 periodic cycle for the next SSB. It can reduce the TFineTiming from ‘up to one time of SSB periodicity’ to a relatively small value based on the first coming A-TRS (TFirstATRS).

Observation 1: A-TRS can be configured to conduct fine timing tracking for TCI after the TCI activation command to reduce the waiting time for next SSB/CSI-RS.

Proposal 5: Use A-TRS corresponding to the TCI state for fine timing tracking after TCI activation command.

3	Conclusion

Proposal 1: Consider beam sweeping factor reduction for L1 measurement during unknown FR2 SCell activation.

Proposal 2: Define the same UE capability of Rx beam sweeping factor in FR2 for L1 and L3 measurement.

Proposal 3: Consider the feasibility of skipping L1-RSRP measurement, and study its conditions, e.g., L1/L3 measurement configuration at least including RSs type/RS BW/QCLed RS for L1 and L3 measurement.

Proposal 4: When TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report, the uncertainty of TCI configuration/activation can be saved.

Observation 1: A-TRS can be configured to conduct fine timing tracking for TCI after the TCI activation command to reduce the waiting time for next SSB/CSI-RS.

Proposal 5: Use A-TRS corresponding to the TCI state for fine timing tracking after TCI activation command.

4	Reference

[1] RP-221696, Revised WID on Rel-18 even Further RRM enhancement for NR and MRDC, Apple, OPPO

[2] R4-2214345, WF on R18 eFeRRM, Apple

[3] R4-2212063, Initial discussion on FR2 Scell activation delay reduction, OPPO