3GPP TSG-RAN WG4 Meeting # 104bis-e														R4-2215818

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			9.1

Source:	OPPO

Title:	Discussion on BWP operation without bandwidth restriction

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has approved the LS[1] and WF[2] on BWP operation without bandwidth restriction. And the similar discussion in last RAN-P meeting happened, and RAN tasked RAN4 to further evaluate the options listed in the LS.

Question 1: Whether it is a valid scenario in the standard to support the operation of BWP without SSB where the UE does not perform BM/RLM/BFD due to the lack of necessary reference signal (SSB and CSI-RS) in the active BWP. Answer: From the existing RAN4 specification point of view, it is not a valid scenario. Question 2: If the answer to question 1 is that this is not valid, how should the UE perform BM/RLM/BFD when the active BWP does not contain SSB. Answer: RAN4 has examined the Rel-15, Rel-16 and Rel-17 specs. The following possible solutions for the issue are identified. - Perform BM/RLM/BFD based on CSI-RS within active BWP - RAN4 has requirements to support BM/RLM/BFD based on CSI-RS within active BWP and no spec change is needed - Following potential independent implementations/features requires either existing RAN4 requirements to be updated or new requirements to be developed. - Perform BM/RLM/BFD based on SSB outside active BWP - UE’s capability to operate using larger BW covering SSB outside active BWP, or a UE that is equipped with a separate RF chain - BM/RLM/BFD on SSB outside BWP are performed with shared MG or NCSG for L3 measurement, or dedicated MG or NCSG for RLM/BFD/BM measurements. - NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD - Note: RAN4 does not reach consensus on whether to work on the above items in Rel-17 including to update the existing RAN4 requirements or to develop new requirements

In this contribution, we further provide our views on BWP operation without bandwidth restriction for legacy normal UE.

- Discussion

From RRM requirement perspective, RLM/BFD/CBD/BM measurements are all performed on the RS resources within active BWP for serving cells. If active BWP does not contain SSB or CSI-RS, no RRM requirements are specified for such cases. Since it is not supported from standardization perspective, it is totally up to UE implementation. From RAN4 perspective, it is not valid that the UE does not perform BM/RLM/BFD. Otherwise, the link between NW and UE are not guaranteed.

How should the UE perform BM/RLM/BFD when the active BWP does not contain SSB? Except performing BM/RLM/BFD based on CSI-RS within active BWP, RAN4 has also listed several options in last meeting.

One option could be that UE may use larger CBW than active BWP to contain target RS (SSB or CSI-RS) in the UE channel bandwidth. It is feasible for UE to meet RRM requirements in this scenario, but UE cannot achieve power saving gain. In addition, the impact on specification could depend on UE behaviour of RF retuning. Whether additional switching time or interruption due to UE CBW adaption is needed depends on different UE implementation. A lot of specification work is needed and this option would only be feasible in Rel-18.

Another option is that UE operates in active BWP without SSB but serving cell timing can be guaranteed by measurement with gap or NCD-SSB detection. RLM/BFD/CBD/BM measurements can be continued with such timing. To achieve this, shared/dedicated MG or NCSG are required for UE to perform SSB measurement outside BWP, or NCD-SSB can be used similar to Redcap UE. However, the existing RLM/BFD/CBD/BM measurements does not support measurement within gap. It could be not a restriction if additional scheme is allowed for UE, which depends on further discussion in R18, e.g., inter-cell L1 measurement with gap.

In general, we think it is too late to discuss any new solution in Rel-17. For R18, all three candidates identified in RAN4 LS should be considered. They have their own pros and cons. For example, the option without measurement gap may end up with much more frequent BWP switching and associated interruption. Or, it may be at the price of higher power consumption if UE always stays with wider BW.

Therefore, UE can be allowed to perform BM/RLM/BFD when the active BWP does not contain SSB, which is up to UE implementation at least in Rel-15/16/17. It is too early to preclude any option at this stage. We are open to discuss the solutions listed in RAN4’s LS in Rel-18. For example, it can be part of the Rel18 RAN4 WID NR_RRM_enh3.

Observation 1: It is too late to discuss any new solution in Rel-17 and each solution with pros and cons has not a small impact on specification.

Observation 2: UE can be allowed to perform BM/RLM/BFD when the active BWP does not contain SSB, which is up to UE implementation at least in Rel-15/16/17.

Proposal 1: Any solution is not precluded at this stage, and support to discuss in R18 FeRRM WI.

- Summary

In this contribution, we further provided our views on BWP operation without bandwidth restriction:

Observation 1: It is too late to discuss any new solution in Rel-17 and each solution with pros and cons has not a small impact on specification.

Observation 2: UE can be allowed to perform BM/RLM/BFD when the active BWP does not contain SSB, which is up to UE implementation at least in Rel-15/16/17.

Proposal 1: Any solution is not precluded at this stage, and support to discuss in R18 FeRRM WI.

- References

- R4-2214355, LS on Feature Group 6-1a “bwp-WithoutRestriction”, Qualcomm

- R4-2214351, WF on Figure Group 6-1a, Apple

- R4-2212548, Reply LS on BWP operation without bandwidth restriction, OPPO