3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215819

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.2.6.1

Source: 	OPPO

Title: 	Discussion on general RRM performance requirements for NR NTN

Document for:	Discussion

1	Introduction

Last RAN4 meeting had a general discussion on NTN test cases and reached some conclusion in [1]. This contribution will provide our consideration on the following maintenance issues.

- Issue 2-3: Test coverage regarding numerology and duplex mode. - Agreement: - Test configurations for TDD and numerology higher than 15kHz are not considered. - FFS on whether to define UE timing TC for 30KHz SCS. - Test configuration includes both 15KHz SCS and 30KHz SCS cases in the same TC, no separate TC is defined for 30KHz SCS case. - Issue 2-4: Test coverage regarding multiple SMTCs. - Agreement: - Test cases shall take multi-SMTC and multi-satellite tests into account. - Minimize the test case number. - RAN4 to define the following configurations for SMTC and satellite in clause A.3 RRM test configuration. - Config. 1: 2 SMTC per MO, each SMTC contains 1 SSB/Satellites - FFS further split into overlapping and non-overlapping cases - Config. 2: 1 SMTC per MO, each SMTC contains 2 SSBs/Satellites - FFS Config. 3: 2 SMTC per MO, each SMTC contains 2 SSB/Satellites - FFS define the configurations for measurement gap in clause A.3 RRM test configuration. - Config.1: 1 measurement gap configuration; (reuse the exiting measurement gap configuration) - Config.2: 2 measurement gap configurations (FNO case); - Config.3: 2 measurement gap configurations (PPO case); - FFS Config.4: 2 measurement gap configurations (FO case with MGRP = 160ms);

2	Discussion

Both 15kHz SCS and 30kHz SCS are supported for NTN, and their initial UL timing error requirements are different. We think it is necessary to define test case for 30kHz SCS as well. For timing advance adjustment accuracy, the delay and accuracy requirements for 15kHz SCS and 30kHz SCS are the same. We are fine to preclude 30kHz SCS in this case.

Proposal-1: Define UE transmit timing test case including both 15kHz SCS and 30KHz SCS.

For SMTC configuration, whether to introduce 2-SMTC configuration with 2 satellites in each SMTC needs further discussed. However, it is difficult to emulate 4 satellites. The intention in our understanding is to verify the newly introduced scaling factor Kmulti_SMTC. As a compromise, we can support 2-SMTC with 2 satellites in one SMTC and 1 satellite in another SMTC.

Proposal-2: If the case of multiple satellites in one SMTC is necessary, support 2-SMTC with 2 satellites in one SMTC and 1 satellite in another SMTC.

For MG configuration, the majority view is that FNO gaps should be defined. And the test case for inter-frequency measurement with two FNO gaps are approved. In the current test cases, SMTC 1 in even SFN with 0ms offset and SMTC 5 in odd SFN with 10ms offset defined in section A.3.11 are configured for the two neighbouring cells respectively. Then, the offset of the associated gaps should be 9ms and 19ms. If gap pattern ID 0 (MGRP=40ms, MGL=6ms) is used for both gaps. Then as shown in the figure-1 below, the distance between the two gaps is 4ms, which should be regarded as overlapping case. For FNO case, it is better to adopt either with another gap pattern with shorter MGL (e.g. ID 2 with MGL=3m) or the new SMTC configuration (e.g. SMTC pattern Y with 5ms offset in A.3.11.7). Since the mandatory gap patterns ID 0 and 1 are both with MGL=6ms, we prefer the new SMTC configuration. And the illustration of SMTC and FNO gap configurations are shown in figure-2.

Proposal-3: For FNO gaps, the associated SMTC should be pattern 1 and Y defined in clause A.3.11.

Figure-1: Illustration of SMTC 1 and SMTC 5 with MGRP=40ms gaps

Figure-2: Illustration of SMTC 1 and SMTC Y with MGRP=40ms gaps

The core requirements with FO measurement gaps are still under discussion. One option is to not define requirement since it is a corner case. Another option is to define requirements with the assumption of gap sharing rule. This is a different solution for gap collision handling, and we think it is necessary to define the corresponding test case if the latter solution is agreed.

Proposal-4: Define test case for FO gaps if the core requirements are defined.

3	Conclusion

This contribution discussed the maintenance issues for NTN test case, and gave the following proposals:

Proposal-1: Define UE transmit timing test case including both 15kHz SCS and 30KHz SCS.

Proposal-2: If the case of multiple satellites in one SMTC is necessary, support 2-SMTC with 2 satellites in one SMTC and 1 satellite in another SMTC.

Proposal-3: For FNO gaps, the associated SMTCs should be pattern 1 and Y defined in clause A.3.11.

Proposal-4: Define test case for FO gaps if the core requirements are defined.

4	Reference

- R4-2214473, WF on performance part for NR NTN, Xiaomi