# 3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215826

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	6.21.2

Source: 	OPPO

Title: 	Discussion on RRM requirements for Rel-17 MUSIM gaps

Document for:	Approval

1	Introduction

MUSIM gaps were discussed and introduced in Rel-17. However, the corresponding requirements were postponed due to the lack of TU in RAN4. During RAN-P #95e meeting, the WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR [1] was revised to include the RRM requirements for Rel-17 MUSIM gaps. And RAN4 #104e meeting had an initial discussion on this topic [2].

2	Discussion

2.1	On NW-A requirements

Issue 2-2-2: Scenario where network A requirement can be directly reused - Proposals - Option 1: when the MUSIM gap neither collides with any legacy gap nor collide with any SMTC/SSB or any resources for L1 measurement; or only MUSIM gaps are configured and the MUSIM gap does not collide with any SMTC/SSB or any resources for L1 measurement, network A measurement requirements can be reused. (vivo) - Option 2: RAN4 to specify that all the requirements outside MUSIM gaps for Network A are not impacted by the MUSIM operation. (Nokia) - Option 3: On top of option 1, the impact on UL related requirements/procedure can be added. (CMCC) - Option 4: Focus on scenario where NW A is impacted (Ericsson Apple oppo Huawei MTK) Issue 2-2-3: Principle on layer 3 measurement requirements after gap collision handling - Proposals - Option 1: The principle of defining scaling factor Kp and Kgap for multi-concurrent gaps are applied to the calculation of Kp and Kgap for layer 3 measurement (Apple xiaomi oppo MTK vivo) - Option 1a: re-use the ‘counting’ approach defined for Rel-17 concurrent MGs to define scaling factor for the impacts of MUSIM gaps (Apple xiaomi vivo) - Option 2: Define requirements after solving gap collision issue (CMCC Huawei vivo MTK Qualcomm Nokia) - Option 3: Too early to discuss this issue (Ericsson) Issue 2-2-4: Principle on L1 measurement requirements after gap collision handling - Proposals - Option 1: The principle of defining P value for L1 measurement and RLM/BFD measurement in Rel-17 cam be reused (Apple xiaomi oppo) - Option 1a: re-use the ‘counting’ approach defined for Rel-17 concurrent MGs to define scaling factor for the impacts of MUSIM gaps (Apple xiaomi oppo) - Option 2: Define requirements after solving gap collision issue (CMCC xiaomi vivo Huawei Qualcomm Nokia) - Option 3: Too early to discuss this issue (Ericsson)

All the candidate options for issue 2-2-2 are reasonable from our side. When the MUSIM gaps neither collide with L1/L3 measurement resources nor legacy MG for NW-A, obviously the measurements in NW-A will not be impacted and the existing requirements could be reused. However, we think the colliding scenario is inevitable considering that up to 3 periodic MUSIM gaps could be configured. The measurements in NW-A may be critical and the related measurement period could be clearly defined.

Proposal-1: Focus on scenario where NW-A is impacted and discuss the extended measurement period.

As for the principle to define extend measurement period in NW-A due to MUSIM gap collision, the conclusions in Rel-17 (i.e. the option 1 for issue 2-2-3 and 2-2-4) could be used, since we have reached a tentative agreement that priority rule for gap collision handling can be used as a baseline for collision between MUSIM gaps and legacy gaps. We are also open to other solutions if other gap collision handling solutions are approved.

Proposal-2: Reuse the principle of defining scaling factor Kp, Kgap and P for L3/L1 measurement measurements in Rel-17.

2.2	Gap collision handling

Issue 2-3-2-1: Clarification on the scope of Rel-17 legacy gap - Proposals: - Option 1: Discuss if concurrent MUSIM and other Rel17/18 measurement gap types is in the scope of this WID or NR_MG_enh2 (Nokia) - Option 2: In case 1, gaps to be considered include all gaps defined till Rel-17 including Pre-MG, NCSG and legacy gaps for measurement and other purposes (MTK Apple Charter CMCC Huawei vivo xiaomi Qualcomm oppo) - Option 2a (Ericsson): Use Option 2 with the following note: Note 1: The group needs to further consider how to handle Pre-MG/NCSG and MUSIM gaps. Note 2: The Pre-MG/NCSG and concurrent gaps are discussed in parallel in Rel-18 WI further MG enh.

First for the scope of Rel-17 legacy gap, option 2 can be supported. It is possible apply a pre-configured MG or NCSG for the measurements in NW-A. We prefer to discuss this issue in MUSIM WI, but are also fine to Rel-18 further MG enhancement WI.

Proposal-3: Support option 2: In case 1, gaps to be considered include all gaps defined till Rel-17 including Pre-MG, NCSG and legacy gaps for measurement and other purposes.

Issue 2-3-2-3: Priority of MUSIM against other legacy gaps - Proposals: - Option 1: MUSIM gaps should have high priority in the event of a collision (Charter Qualcomm) - Option 2: MUSIM gaps can be defined as the lowest priority, and periodic MUSIM gaps will be dropped once the gap dropping rule defined in Con-MGs is met () - Option 3: Up to NW configuration (Apple CMCC Huawei Xiaomi Charter oppo vivo) - Option 3a: Up to UE request (MTK) - Option 4: FFS (Ericsson Nokia) Issue 2-3-5-1: On aperiodic gap priority - Proposals: - Option 1: UE can request aperiodic MUSIM gap with a higher priority. (Ericsson Charter) - Option 2: Option 1 is up to UE implementation (vivo) - Option 3: FFS (Apple CMCC Huawei Qualcomm Xiaomi MTK)

Generally, we think the priority of MUSIM gaps should be up to NW configuration, which is the same as multiple concurrent gaps in Rel-17. Defining the highest or lowest priority for MUSIM gap is not suitable for all scenarios. UE may need to receiving paging/SIB information from NW-B within MUSIM gaps, requiring the highest priority, or vice versa. So we think NW configuring priority is more feasible. And it is better for UE to report some assistant information to NW-A to choose an appropriate priority. But whether and how to enable such the mechanism should be discussed in RAN2. For aperiodic gap, we don’t see much difference.

Proposal-4: Priority of MUSIM gaps, including both periodic and aperiodic gaps, should be up to NW configuration, and whether UE could request priority should be discussed in RAN2.

Issue 2-3-3-2: Priority of MUSIM against SMTC, and other L3/ L1 measurement resources - Proposals: - Option 1: MUSIM gaps should have high priority against SMTC and L1 measurement resources (Apple xiaomi oppo Qualcomm Huawei MTK vivo) - Option 2: NW-A’s RRM procedure, including DL SMTC should have higher priority than MUSIM gaps. The MUSIM periodic gaps should be dropped once the gap proximity rule is met. (Ericsson) - Option 3: As baseline solution, UE can only perform gap-less L3 measurement and L1 operation outside MUSIM gap. Other solutions are not precluded to handle collision between MUSIM gap and SMTC/RS for L1 operation. (Apple oppo xiaomi) - Option 4: FFS (Ericsson Nokia CMCC vivo)

In case of collision between MUSIM gap and SMTC/L1 resource, the gap should be prioritized as a baseline. One potential issue is that the critical measurement such as intra-frequency measurement for handover or TCI switch will be impacted by MUSIM gaps. We think this could be handled by NW-A implementation. When such the critical measurements are overlapped with the request MUSIM gaps, NW-A could reject the request.

Proposal-5: MUSIM gaps should have high priority against SMTC and L1 measurement resources.

Issue 2-3-5-2: On the time window W for aperiodic gap - Proposals: - Option 1: Discuss whether and how to determine the time window W when aperiodic MUSIM gap with higher priority is involved in collision (oppo) - Option 2: W could be the largest periodicity among all the periodic gaps + Time margin [M] for the one-shot aperiodic gap (MTK) - Option 3: FFS (oppo Ericsson Apple Nokia Charter CMCC Huawei Qualcomm xiaomi MTK vivo)

In order for data transmission/reception in NW-B, aperiodic MUSIM gap is introduced in Rel-17. So it is necessary to define requirements when aperiodic MUSIM gap collides with measurement in NW-A. One simple way is to reuse the existing time window W and do not consider aperiodic gap. When the measurement in NW-A is overlapped with aperiodic MUSIM gap with higher priority, simply clarify that the measurement period could be longer without explicitly define the extension. Another way is option 2 proposed in the last meeting. Introduce a time margin for window W to include all periodic and aperiodic gaps. We are open to both options.

Proposal-6: On the time window W for aperiodic gap:

- Option 1: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer

- Option 2: Take aperiodic gap into account when determining the time window W by adding a time margin.

2.3	On NW-B requirements

Issue 2-4-1: Whether to define network B requirements - Proposals: - Option 1: Define the requirements for Network B in RRC idle/inactive (Ericsson) - Option 2: No measurement requirements in network B will be defined by RAN4 (Apple Nokia Huawei Qualcomm MTK vivo) - Option 3: If there is a consensus to specify network B requirement, its priority should be lower compared with the work for network A requirements and could be carried out at the second phase in the WI time frame (Apple vivo)

For NW-B, the measurement period will also be impacted by MUSIM gaps. In simplest scenario where MUSIM gap is not overlapped with other gaps or not dropped, the measurement period can be updated by replacing DRX cycle by max(DRX cycle, MUSIM gap periodicity). However, when MUSIM gap is dropped due collision, the measurement period will be complicated. Meanwhile without new signalling introduced in RAN2, it may be hard for NW-B to get the full information of gap collision and the associated priority, especially for the collision between MUSIM gap and legacy gap in NW-A. In this scenario, no requirement is also acceptable for us.

Proposal-7: Support option 2: No measurement requirements in network B will be defined by RAN4.

2.4	Others

Issue 2-5-1: MUSIM overhead - Proposals: - Option 1: RAN4 to define MUSIM gap overhead for MUSIM gap(s) (xiaomi) - Option 2: wait for concurrent gap conclusion (Nokia) - Option 3: FFS (Apple oppo Nokia Charter Huawei Qualcomm Xiaomi MTK vivo) - Option 4: Reuse conclusion in concurrent gap if MUSIM gaps can be viewed as “one gap”(Ericsson) < Agreement in Aug 19 GTW session >:  Regarding the overhead cap on concurrent gaps in Rel-17, measurement requirement does not apply when more than one MGP is configured with MGRP=20ms in an FR.

In Rel-17, the overhead cap rule on multiple concurrent gaps as shown above was agreed in the last meeting. The main reason is to avoid too much throughput loss. From this perspective, the overhead cap should also be introduced for MUSIM gaps, and the principle in Rel-17 could be used as the baseline.

Different from concurrent gaps where at most 2 gaps could be configured in each FR, UE could be configured with up to 3 MUSIM gaps and one aperiodic gap. Excluding the case when more than one MGP with MGRP=20ms may also lead to a higher throughput loss in NW-A. For example, 2 MUSIM gaps with MGRP=40ms and 1 MUSIM gap with MGRP=20ms will cause the same throughput loss with 2 gaps with MGRP=20ms.

Another difference is the longer MGRP (320ms~5120ms) for MUSIM gaps, which is beneficial for the throughput in NW-A. In the scenario of 3 periodic MUSIM gaps, it is desirable to configured one/more MUSIM gaps with longer MGRP.

Proposal-8: Use the overhead cap principle on multiple concurrent gaps in Rel-17 as the baseline for MUSIM gaps, and discuss further enhancements considering:

- Up to 3 periodic MUSIM gaps and one aperiodic MUSIM gap

- Longer MGRP

3	Conclusion

This contribution gave our general views on RRM requirements for Rel-17 MUSIM gaps and the following proposals:

Proposal-1: Focus on scenario where NW-A is impacted and discuss the extended measurement period.

Proposal-2: Reuse the principle of defining scaling factor Kp, Kgap and P for L3/L1 measurement measurements in Rel-17.

Proposal-3: Support option 2: In case 1, gaps to be considered include all gaps defined till Rel-17 including Pre-MG, NCSG and legacy gaps for measurement and other purposes.

Proposal-4: Priority of MUSIM gaps, including both periodic and aperiodic gaps, should be up to NW configuration, and whether UE could request priority should be discussed in RAN2.

Proposal-5: MUSIM gaps should have high priority against SMTC and L1 measurement resources.

Proposal-6: On the time window W for aperiodic gap:

- Option 1: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer

- Option 2: Take aperiodic gap into account when determining the time window W by adding a time margin.

Proposal-7: Support option 2: No measurement requirements in network B will be defined by RAN4.

Proposal-8: Use the overhead cap principle on multiple concurrent gaps in Rel-17 as the baseline for MUSIM gaps, and discuss further enhancements considering:

- Up to 3 periodic MUSIM gaps and one aperiodic MUSIM gap

- Longer MGRP

4	Reference

- RP-220955, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo

- R4-2214349, WF on RRM requirements for Rel-17 MUSIM gaps, vivo